Emergency Operating Instructions of TOA Emergency Public Address System

TOA紧急广播系统应急操作说明

Emergency message panel FV-200EV-AS 紧急信息面板

Buttons and keys used for operation操作按钮和键

| - Talk switch 话筒开关 | - Clear Announcement Indicator解除广播显示 |
| --- | --- |
| - Fireman’s Microphone 消防员麦克风 | - Alert Announcement Button 警报广播按钮 |
| - Fireman’s Microphone Connector消防员麦克风接头 | - Evacuation Announcement Button 疏散广播按钮 |
| - Fire Indicator 火灾显示 | - False Announcement Button误报广播按钮 |
| - Alert Announcement Indicator警报广播显示 | - Clear Announcement Button解除广播按钮 |
| - Evacuation Announcement Indicator疏散广播显示 | - Stop Button 停止按钮 |
| - False Announcement Indicator误报广播显示 | - Power Indicator电源显示 |

PA Amplifier公共广播功率放大器

| - Power Switch电源开关 | - TEL Volume Control 电话接口音量控制 |
| --- | --- |
| - Power Indicator电源开关显示 | - AUX Volume Control 辅助接口音量控制 |
| - Signal Indicator 信号显示 | - Bass Control 低音控制 |
| - Peak Indicator 信号峰值显示 | - Treble Control 高音控制 |
| - Microphone Volume Control 麦克风音量控制 | - Master Volume Control 主音量控制 |

Announcement content 广播内容

- Alert announcement: [Female voice] (Siren tone * 3 times)

“Attention please! The fire alarm has been activated. We are now investigating the cause. Please wait for further information.”

警报广播：（女声）（警报音*3次）

“请注意！火警已被启动，原因正在调查，请等待进一步消息。”

- Evacuation announcement: [Male voice] (Siren tone * 3 times)

“There is a fire! Please evacuate in a calm and orderly manner. ”

疏散广播：（男声）（警报音*3次）

“着火了，请保持冷静，进行有序疏散。”

- False alarm announcement: [Female voice] (Siren tone * 3 times)

“Attention please! We have investigated the situation and there has been a false ，alarm. We apologise for any inconvenience caused. ”

误报广播：（女声）（警报音*3次）

“请注意！经仔细调查，火灾报警为误报，由此给您带来的不便我们深表歉意。”

- Clear announcement: [Female voice] (Siren tone * 3 times)

“Attention please! The emergency situation is now under control. We regret any inconvenience caused.”

解除广播：（女声）（警报音*3次）

“请注意！紧急情况已经得以控制，由此给您带来的不便我们深表歉意。”

Emergency Operating Procedures 紧急操作步骤

警报消息将广播到所有区域，警报广播显示灯将亮起，火灾显示灯将亮起。

自动启动（火灾报警系统）

手动启动（操作工）

检查现场情况

按下警报广播按钮

按下疏散广播按钮

按下误报广播按钮

如果没有火灾

发生火灾时

检查现场情况

误报消息将广播到所有区域，误报广播显示灯将亮起。

按下话筒开关向所有区域广播

完成紧急广播

当火已经被扑灭时

使用麦克风广播

疏散消息将广播到所有区域，疏散广播显示灯将亮起。

重置火警系统并按下停止按钮，开始普通广播。

重置火警系统并按下停止按钮

解除消息将广播到所有区域，解除广播显示灯将亮起。

按下解除广播按钮