Hengyi Industries Sdn Bhd

恒逸实业（文莱）有限公司

HYBN-T9-11-0042-2024-1

Refinery No.2 Dept. Lubrication

Management Rules

炼油二部润滑管理细则

Issued Date：May 2024      颁布日期：2024年5月

Preface前 言

This rule is drafted and formulated by No. 2 Refinery Department.

本管理细则由炼油二部起草和制定。

This rule is effective as of March 26, 2021.

本细则从2021年3月26日起实施。

此版为第二版，从2024年5月1日起实施，第一版废止。

# Content目录

1  Purpose 目的	1

2  Scope of Application 适用范围	1

3  Terms and Definitions 术语和定义	1

4  Management Responsibilities 管理职责	1

5  Manage content 管理内容	2

6　 Inspection and Supervision 检查与监督	5

7  Associated Procedures and Records 关联程序和记录	5

8　Supplementary Rules 附则	6

9  Appendices 附件	8

# 1  Purpose 目的

This System is hereby formulated to reduce the wear, extend the service life and guarantee safe and stable operation of equipment.

为减少设备磨损，延长设备使用寿命，保证设备安全稳定运行，特制订本制度。

# 2  Scope of Application 适用范围

This System is applicable to lubrication management of rotating equipment of the Company.

本制度适用于公司转动设备的润滑管理。

# 3  Terms and Definitions 术语和定义

Lubrication: refers to application of lubricants between the contact surfaces of each friction pair in relative motion to form a lubrication film between the friction surfaces, separate the dry friction surfaces and convert dry friction into friction between lubricant molecules, so as to reduce friction and wear.

润滑：指在发生相对运动的各对摩擦副的接触面之间加入润滑剂，使摩擦面之间形成润滑膜，将干摩擦面分隔开来，变干摩擦为润滑剂分子之间的摩擦，以达到降低摩擦和减小磨损为目的。

# 4  Management Responsibilities 管理职责

4.1　Management Position 管理岗位

4.1.1　The Equipment Manager is the specified administrative authority of lubrication management, which shall be responsible for formulating the lubrication management system, organizing to solve the technical problems in lubrication management,guiding and supervising equipment lubrication management of all relative works.

设备专业管理人员是润滑管理的归口管理岗位，负责制订本部门润滑管理制度；组织解决润滑管理中的技术问题。指导、管理本部门设备润滑工作。

4.1.2　Make the reports concerning equipment lubrication and the purchase plan of lubricating oil (grease),Assist to the Equipment Management Dept. for the application and promotion of advanced lubrication technologies.

编制设备润滑相关报表以及提报润滑油（脂）的采购计划，配合机动部施行润滑先进技术的应用和推广。

4.2　Coordinated management Position  协同管理岗位

4.2.1　HSE manager shall be responsible for disposing of waste lubricating oil、barrel etc in accordance with the Hazardous Waste Management Regulations.

HSE专业管理人员负责按照危废物管理规定处置废润滑油、废油桶等工业三废。

4.3　Executive Position 执行岗位

4.3.1　Operation Position shall be responsible for Lube oil’s storage、add、discharge、sampling、tools using and maintenance、oil station’s standardization work.

操作岗位负责润滑油保管、加注、排放、采样、润滑油具使用及维护、润滑油站及润滑部位的规格化工作。

# 5  Position Content 岗位内容

5.1　Management Position Content 管理岗位内容

5.1.1　Equipment manager shall report the actual consumption and year-end inventory of lubricating oil (grease) of this year in late December every year, and prepare the next annual plan of lubricating oil (grease).

设备工程师于每年12月下旬上报本年度润滑油（脂）实际消耗量及年终库存量，编制润滑油（脂）下一年度计划。

5.1.2　For working oil products, the lubricating oil (grease) requisition plan of the next month shall be reported in the EM system before the 25th day of each month; For non-working oil products, the requisition plan shall be reported 1 month in advance.Working oil products include L-DAB100 and L-DAB150 reciprocating compressor oil, L-TSA32, L-TSA46 and L-TSA68 anti-rust turbine oil, L-HM32, L-HM46 and L-HM68 anti-wear hydraulic oil, L-CKD150, L-CKD220 and L-CKD320 industrial closed gear oil, 2# and 3# lithium-based grease and 10# industrial white oil. Others are not commonly used.

对于常用油品，每月25日前上报下月润滑油（脂）领用计划；对于不常用油品，应提前1个月上报领用计划。（常用油品种类为：L-DAB100往复压缩机油、L-DAB150往复压缩机油、L-TSA32防锈汽轮机油、L-TSA46防锈汽轮机油、L-TSA68防锈汽轮机油、L-HM32抗磨液压油、L-HM46抗磨液压油、L-HM68抗磨液压油、L-CKD150工业闭式齿轮油、L-CKD220工业闭式齿轮油、L-CKD320工业闭式齿轮油等；2#、3#锂基脂、10#工业白油，其余为不常用油。）

5.1.3　Equipment manager shall request for oil as per the oil types specified in the "Five Fixations” for Pump Equipment Lubrication. When the oil (grease) type need to be changed, the influencing factors shall be identified and the "Application for Change of Lubricating Oil Type" shall be filled in and submitted to the Equipment Management Dept. for review.

设备管理人员应严格按《机泵设备润滑“五定表”》规定的用油品种领用；需要变更润滑油（脂）品种时，应识别影响因素并填写“润滑油种变更申请单”，报机械动力部审核。

5.1.4  The newly received lubricating oil must be provided with a copy of the analysis certificate, or re-sampling is necessary for analysis. Copies of the qualification certificate of each batch of oil (indicating the oil delivery time) shall be given to the receiving department together with the oil. The operation department shall have the right to request the Materials Supply Dept. to spot check the quality of the new oil and also have the right to reject the unqualified oil.

在领用时，新领的润滑油必须有分析合格证书复印件，如缺少必须重新采样分析。润滑油的合格证复印件（注明送油时间）在发放时一起领用，合格证保存期为一年。运行部有权要求物资装备部对新油质量进行抽检，对抽检不合格的油有权拒收。

5.1.5  The lubricating oil (grease) shall be properly stored and kept and not be piled up in the open air. Also, reserve for more than 30 days shall be kept and shall be kept on the site for no more than one year.

润滑油（脂）应妥善储存保管，不得露天堆放，并保持30天以上的润滑油（脂）储备量，现场储存时间不得超过一年。

5.1.6　The lubricating oil depot of the operation department shall be rain-proof, sun-proof, dust-proof, dry, clean and well ventilated, and provided with adequate fire-fighting facilities; Lubricating oil (grease) shall be provided with a qualified label, indicating the brand name of oil (grease), and different brands of lubricating oil (grease) shall be stored separately. The storage life of the oil (grease) is one year as indicated on the certificate.

润滑油站应防雨、防晒、防尘、干燥清洁、通风良好；润滑油（脂）要有合格标签，注明油牌号，不同牌号的润滑油要分别储存，不得混装或混合堆放。合格证保存期为一年。

5.1.7  Equipment manager should confirm the results of lubricating oil analysis monthly.The routine analysis items of lubricating oil include viscosity, acid value, flash point, water content and impurity. The machine (pump) unit with an independent lubricating oil system shall be comprehensively analyzed after overhaul or one month before shutdown for maintenance, including viscosity, flash point, acid value, water content, elemental spectral analysis, anti-emulsification and anti-oxidation. If the oil is analyzed to be unqualified, the operation department shall report to the Equipment Management Dept., make a replacement plan, and analyze the cause.

设备管理人员应每月对润滑油分析结果进行确认。润滑油常规分析项目为：粘度、酸值、闪点、水份、杂质五项；在大修后或装置停工检修前一个月，需进行全分析，包含粘度、闪点、酸值、水份、元素光谱分析、抗乳化、抗氧化性等。分析不合格时，运行部应报告机械动力部，做好置换或更换计划，并进行原因分析。

5.1.8 After the maintenance of the machine (pump) unit with an independent lubricating oil system, the lubricating oil system shall circulate for more than 4 hours and cannot be put into operation until it is analyzed to be qualified. The oil tank shall be dehydrated at least once a week. Check the hydration and inspection records weekly. If the water content in the oil tank is high in normal operation, the cause shall be found and the dehydration frequency shall be increased.

具有独立润滑油系统的机（泵）组检修后，润滑油系统应循环4小时以上，分析合格后方可试车投用。每周检查脱水记录；正常运行中发现油箱含水量多时，应查找原因并增加脱水频次。

5.1.9　The equipment with an independent oil station shall be provided with a dependent lubrication file, covering lubrication card, oil addition and replacement situation, treatment measures for unqualified oil as per analysis, change of oil type and renovation of lubrication system.

具有独立油站的设备应单独建立润滑档案，润滑档案的内容应包括：润滑卡片、加换油情况、油质分析异常时的处理措施、油种变更、润滑系统改造等。

5.1.10　Lubrication files shall not be established for equipment without an independent oil station, but the lubrication data shall be registered in the equipment file.

无独立油站的设备不单独建立润滑档案，但应在设备档案中登记润滑资料。

5.2　Operation Position Content 操作岗位内容

5.2.1  Lubrication Oil Storage 润滑油保管

5.2.1.1  On-site lubricating oil storage is the responsibility of the operating post. Including the maintenance of spare lubricating oil area, oil station and waste oil recovery area.

现场润滑油保管由操作岗位负责。包括备用润滑油区、油站、废油回收区的维护。

5.2.1.2  The lubricating oil barrel in the spare lubricating oil area is placed in the designated area. The lubricating oil in the spare area shall be moved into oil station in time when the barrel in the oil station is empty. The surface of the barrel and the area should be clean.

备用润滑油区的润滑油桶放置在指定区域。备用区的润滑油在油站内大桶空后及时替换。桶面及区域内整洁无油迹、无杂物。

5.2.1.3  Maintenance work in the oil station is carried out in accordance with the management rules of the oil station.

润滑油站内维护工作按照油站管理细则执行。

5.2.1.4  The lubricating oil barrels in the waste oil recovery area are placed in the designated area, and the waste oil barrels are stored by tightening the lid. The surface of the barrel and the area are clean.

废油回收区的润滑油桶放置在指定区域，废油桶拧紧桶盖存放。桶面及区域内整洁无油迹、无杂物。

5.2.2　Usage of Lubrication Oil 润滑油使用

5.2.2.1  The oil level standard is mainly based on the oil window and the red line of the oil cup. The oil window is between 1/2-2/3, and the oil cup is not less than 1/3.

润滑油使用部位的油位以油标及油杯的红线为主，油标取1/2-2/3之间，油杯不低于1/3。

5.2.2.2  Equipment lubrication, upon completion, shall be timely recorded in the EM system.

设备润滑作业完成后，应及时在设备EM系统中记录。

5.2.2.3 The person in charge of the equipment is responsible for the regular oil change operation of the lubricant.

设备包干负责人对润滑油定期换油作业负责。

5.2.2.4 Operator treats lubrication work shall be based on the contents of the five lubrication fixations (namely, fixed person, fixed point, fixed quality, fixed amount, and fixed time):

操作工对润滑部位的操作以润滑五定（即定人、定点、定质、定量、定时）的内容为准：

(1)	Fixed person: The lubrication point of each set of equipment shall be in the charge of a fixed person responsible for oil adding and replacement.

定人：每台设备的润滑点都有固定的加、换油负责人。

(2)	Fixed point: Oil shall be injected from the lubrication point as specified by the "Five Fixations” for Pump Equipment Lubrication.

定点：按《机泵设备润滑“五定表”》规定的润滑点注油。

(3)	Fixed quality: Oil shall be of the type and brand as specified by the "Five Fixations” for Pump Equipment Lubrication.

定质：按《机泵设备润滑“五定表”》规定的润滑油（脂）品种、牌号注油。

(4)	Fixed amount: Oil shall be injected as per the amount as specified by the "Five Fixations” for Pump Equipment Lubrication.

定量：按《机泵设备润滑“五定表”》规定的注油量注油。

(5)	Fixed time: Oil shall be injected and replaced at the time as specified by the "Five Fixations” for Pump Equipment Lubrication.

定时：按《机泵设备润滑“五定表”》规定的时间加换油。

Note: for the equipment with fully closed bearing and requiring no external grease adding, the “five fixations” for lubrication can be reduced to “three fixations”, i.e.: fixed person, fixed point and fixed quality.

注：针对采用全封闭轴承，无需外部加脂的设备，润滑“五定”内容仅包括定人、定点、定质。

5.2.2.5  Lubricating oil must go through "three-stage filtration" before use. See figure 1 for the schematic diagram of "three-stage filtration" for equipment lubrication.Filter precision: the first-stage strainer is 60-mesh, the second is 80-mesh, and the third is 100-mesh.

润滑油在使用前必须经过“三级过滤”，设备润滑“三级过滤”示意图见图1。滤网过滤精度：一级为60目，二级为80目，三级为100目。

Figure 1 Schematic diagram of "three-stage filtration" for equipment lubrication

图1 设备润滑“三级过滤”示意图

5.2.3　Oil station management rules油站管理细则

5.2.3.1  Confirm that the lubricating equipment and tools are in good condition. The lubricating equipment includes: hand pump,fixed oil drum (medium barrel), oil pot(small barrel), filter funnel, and oil pan. Tools and equipment include: waste oil receiving barrel, lubricating oil case, barrel opener, hydraulic cart.

确认润滑油具及工器具完好，润滑油具包括：手摇泵、固定油桶（中桶）、提油桶（小桶）、过滤漏斗、接油盘。工器具包括：废油接桶、润滑油柜、开桶扳手、液压推车。

5.2.3.2  The ground of the lubricating oil station is clean and tidy, free of debris, oil stains, dust, and imprints; the windows are clean and intact and often closed; the ceiling is not leaking, the walls are smooth and clean, and the lighting facilities are intact.

润滑油站地面整洁，无杂物、无油迹、无积尘、无印痕；窗户干净完好并常关；顶棚无漏、墙壁平整光洁，照明设施完好。

5.2.3.3  Lubricating oil tools and equipment are placed in a fixed position according to the red frame and relevant indication signs. Irrelevant items shall not be stored in the oil station.

润滑油具及工器具根据红框及相关指示标识定置定位摆放。油站内不得存放无关物品。

5.2.3.4  The five fixations for lubrication oil table on the wall and various indicating stickers are clear and intact. The complete sets of oil tools for each lubricating oil are placed according to the position of the stickers. Place the oil barrel in the red frame on the ground with the barrel mouth facing outward (back to the wall) and install the hand pump. Connect the hose to the hand pump and fix it so that the pipe mouth is upward. The fixed oil drum is placed in the red frame on the lubricating oil case with the oil drain valve facing outward. The oil pot, filter funnel and oil receiving pan are placed in the red frame in the lubricating oil case under the corresponding fixed oil drum, with the sticker side facing outward.

上墙润滑油五定表及各类指示贴纸清晰完好，各润滑油的成套配备油具根据贴纸位置对号摆放。领油大桶放置在地面红框内，桶口朝外（背向墙）并安装手摇泵，手摇泵接软管并固定使管口朝上。固定油桶放置在润滑油柜上的红框内，放油阀朝外。提油桶、过滤漏斗及接油盘放置在对应固定油桶下方的润滑油柜内红框中，带贴纸一侧朝外。

5.2.3.5  The three-stage lubricating oil filter device is specially designed for use according to the marked oil type.

润滑油三级过滤器具根据标记的油种牌号，专具专用。

5.2.3.6  Clean the oil tools in time after use to ensure that the inside and outside of the tools are clean. The filter is clean and free of debris, soft or hard particles. There are no water droplets or particles in the lubricating oil drum. There is no oil drop or particles on the inner and outer surfaces of the oil pan and lubricating oil case, and the door handle of the lubricating oil case is closed. After the hand pump is used, the oil is drained out of the mouth and fixed the mouth of the pipe upward, and there is no oil on the surface of the oil tank.

油具使用后及时清理，确保器具内外洁净。滤网清洁无杂物，无软或硬质颗粒。润滑油桶内无水珠、无颗粒。接油盘及润滑油柜内外表面无油污、无颗粒，润滑油柜门把手关闭。手摇泵使用后口朝下滴尽存油并重新固定使管口朝上，领油大桶表面无油污。

5.2.3.7  Each record book such as the shift book of the lubricating oil station is intact and recorded truthfully.

各记录本如润滑油站交接班本完好并如实记录。

5.2.3.8  Make sure the doors and windows are closed when leaving the petrol station.

离开油站时确认门窗关闭。

5.2.4  Lubrication oil sampling

润滑油采样分析

5.2.4.1  Complete the sampling task on time according to the lubricant analysis list and analysis frequency in the inspection plan and regular work schedule.

根据检验计划及定期工作安排中的的润滑油分析清单及分析频次，按时完成取样任务。

5.2.4.2  The Lab Dept. shall be responsible for sending the sampling bottles to the operation department for sampling and then the Lab Dept. shall take the bottles with samples back for analysis. In addition, the Lab Dept. must sign the hand-over/take-over document with the operation department when sending or taking lubricating oil sampling bottles, and informal sampling bottles are forbidden.

质量检验部负责将采样瓶送至运行部并取回分析，运行部负责采样；运行部将样品与质量检验部交接时，需签字交接，禁止使用非正规采样瓶。

5.2.4.3  Dehydration and inspection are necessary upon each oiling, and records shall be made also. If the water content in the oil tank is high in normal operation,should inform equipment manager immediately,the cause shall be found and the dehydration frequency shall be increased.

油箱每周应不少于一次脱水检查，每次加油后应进行脱水检查，并做好记录；正常运行中发现油箱含水量多时，应汇报设备管理人员，查找原因并增加脱水频次。

# 6  Inspection and Supervision 检查与监督

The Equipment Manager Dept. shall conduct supervision, inspection and assessment in accordance with the provisions of this System.

设备管理人员按照本制度的规定要求进行监督、检查、考核

# 7  Associated Procedures and Records 关联程序和记录

7.1  Associated procedures 关联程序

7.1.1  Lubricating Oil Requisition Procedures (HYBN-T2-07-0007-2018-1)

润滑油领用程序  HYBN-T2-07-0007-2018-1

7.1.2  Lubricating Oil Analysis Management Procedures (HYBN-T2-07-0008-2018-1)

润滑油分析管理程序  HYBN-T2-07-0008-2018-1

7.1.3  Waste Assessment and Disposal Management Procedures of Lubricating Oil (HYBN-T2-07-0009-2018-1)

润滑油判废及处置管理程序  HYBN-T2-07-0009-2018-1

7.1.4  “Five Fixations” Management Procedures for Equipment Lubrication (HYBN-T2-07-0010-2018-1)

设备润滑五定管理程序  HYBN-T2-07-0010-2018-1

7.1.5  Three-stage Filtration Management Procedures for Equipment Lubrication (HYBN-T2-07-0011-2018-1)

设备润滑三级过滤管理程序  HYBN-T2-07-0011-2018-1

7.1.6  Equipment Lubrication Inspection Procedures (HYBN-T2-07-0012-2018-1)

设备润滑检查程序  HYBN-T2-07-0012-2018-1

7.2  Associated records 关联记录

7.2.1  Application for Change of Lubricating Oil Type (HYBN-T6-07-0059-001-2018)

润滑油种变更申请单  HYBN-T6-07-0059-001-2018

7.2.2  "Five Fixations” for Pump Equipment Lubrication (HYBN-T6-07-0060-001-2018)

机泵设备润滑“五定”表  HYBN-T6-07-0060-001-2018

7.2.3  Annual Plan of Lubricating Oil (Grease) (HYBN-T6-07-0061-001-2018)

润滑油（脂）年度计划表 HYBN-T6-07-0061-001-2018

7.2.4  Requisition Plan of Lubricating Oil (Grease) (HYBN-T6-07-0062-001-2018)

润滑油（脂）领用计划表  HYBN-T6-07-0062-001-2018

7.2.5  Application for Waste Assessment of Lubricating Oil (Grease) (HYBN-T6-07-0063-001-2018)

润滑油（脂）判废申请表  HYBN-T6-07-0063-001-2018

# 8  Supplementary Rules 附则

8.1  The rules are managed by the No.2 Refinery Dept. For the unfinished matters, please refer to the company's professional management systems.

本细则由炼油二部归口管理，未尽事宜参照公司各专业管理制度等执行。

8.2   The preparation and approval of the version of these rules are shown in Table 1.

本细则版本编制和审批情况见表1。

Table 1 Revision, preparation and approval of document

| 2 | 01/05/2024 | KONG QING ZHAN 孔庆站 | ADILI BULIBULI 阿地里·布力布力 | SUN JIAN HUAI孙建怀 |
| --- | --- | --- | --- | --- |
| Revision 版本 | Issued date 颁布日期 | Prepared by 编制人 | Reviewed by 审核人 | Approved by 批准人 |

表1 文件版本编制和审批情况

# 9  Appendices 附件

Appendix 1:Lubricating oil (grease) replacement standards

附件1：更换润滑油（脂）标准

1.1  The lubricating oil (grease) in use, if visually checked to meet one of the following conditions, shall be replaced: the lubricating oil becomes black obviously after continuous use for more than five years; the lubricating oil (grease) is seriously emulsified; the lubricating grease becomes dry and hard or contains visible solids.

对在用润滑油（脂）经目测检查，凡符合下列条件之一者，应置换或更换：润滑油连续使用时间超过五年且外观颜色明显变黑；润滑油（脂）乳化严重；润滑脂变干变硬或有明显可见的固体颗粒。

1.2　 The lubricating oil in use requiring routine analysis only, if meeting one of the following conditions, shall be partially replaced with new oil: the viscosity change exceeds 10% of the viscosity grade of the lubricating oil; the acid value exceeds 10% of the standard; the variation range of flash point exceeds 10% of the standard; the content of Equipment Management impurities is higher than 0.1% or many granular impurities are found in the oil although the content of Equipment Management impurities is no more than 0.1%; the water content is higher than 0.1%.

对只做常规分析的在用润滑油，凡符合下列条件之一者，应部分置换或更换新油：粘度变化超过润滑油粘度等级的10%；酸值超过标准的10%；闪点变化范围超过标准的10%；机械杂质高于0.1%或机械杂质含量虽未超过0.1%，但油中有较多的颗粒状杂质；水含量高于0.1%。

1.3   The lubricating oil in use requiring unconventional analysis, the Equipment Management Dept. and the operation department shall jointly determined whether replacement is necessary based on relevant standards.

对于需再做非常规分析的在用润滑油，由机械动力部与运行部参照相关标准共同判定。

Appendix 2:Waste assessment and disposal

附件2：判废及处置

2.1　Oil that meets one of the following conditions shall be deemed as waste: (1) it is unqualified as per analysis and unable to reach the standard through measures such as heating, filtration and dehydration; (2) its color obviously changes after long-term use; (3) it is from pump maintenance, regular oil replacement or other circumstances and unserviceable.

符合下列条件之一者为废油：分析不合格，无法通过加热、过滤、脱水等措施达到标准的；长期使用颜色明显变色的；机泵检修、定期换油或其他情况更换下来无法利用的。

2.2　 If the quantity for one replacement exceeds 1000kg, the "Application for Waste Assessment of Lubricating Oil" shall be filled in, in writing, and submitted to the Equipment Management Dept. for approval after being reviewed and approved by the director of the operation department.

润滑油更换数量一次超过1000kg的，应书面填写“润滑油判废申请单”，经运行部主管领导审核同意后报机械动力部审批。

2.3　 During the overhaul, the operation department shall actively re-use the oil withdrawn from each machine (pump) and pay attention to the recovery and treatment of waste oil. It is strictly prohibited to pour or discharge the waste oil into the storm drain system.

大检修期间，各机（泵）退出的油品，运行部应积极做好再利用工作，同时应重视废润滑油回收与处理工作，严禁将废油品倒入或排入雨水沟系统内。