Hengyi Industries Sdn Bhd

恒逸实业（文莱）有限公司

HYBN-T9-11-0044-2024-1

Refinery No.2 Dept. Heating Furnace Management Rules

炼油二部加热炉管理细则

Issued Date：May 2024      颁布日期：2024年5月

Preface前 言

This rule is drafted and formulated by No. 2 Refinery Department.

本管理细则由炼油二部起草和制定。

This rule is effective as of March 26, 2021.

本细则从2021年3月26日起实施。

此版为第二版，从2024年5月1日起实施，第一版废止。

# Content目录

1 Purpose 目的	1

2 Scope of Application 适用范围	1

3 Management Responsibilities 管理职责	1

4 Management Content 管理内容	2

5　Associated Procedures and Records 关联程序和记录	8

6　Supplementary Rules附则	8

7  Appendices 附件	9

# 1 Purpose 目的

This System is formulated to reinforce the management of tube heating furnaces and realize

safe, economical and long-term operation.

为加强管式加热炉的管理，实现安全、经济、长周期运行，特制订本制度。

# 2 Scope of Application 适用范围

This Rules is applicable to the Refinery No.2Heating Furnace Management

本制度适用于炼油二部管式加热炉管理。

# 3 Management Responsibilities 管理职责

3.1 Specified administrative authority 归口管理部门

3.1.1 The Equipment Manager is the specified administrative authority of the

heating furnace, which shall be responsible for preparing the heating furnace management

system and supervising and inspecting this system.

设备专业管理是加热炉的归口管理岗位，制（修）订部门加热炉管理制度，并监督检查。

3.1.2 Be responsible for reviewing and approving the heating furnace overhaul, update,

inspection and maintenance plan and organizing the implementation and acceptance.

负责制定加热炉大修、更新、检验与检修计划，并组织实施。

3.1.3 Be responsible for checking the operation and maintenance of heating furnaces by the

operation department, as well as the regular monitoring of the heating furnace efficiency and

the dew point temperature test of flue gas.

负责检查本部加热炉操作、维护以及加热炉效率定期监测、烟气露点温度测试工作。

3.1.4 Be responsible for the technical exchange of heating furnaces and auxiliary equipment

and the signing of technical appendices, and organize the promotion and application of new

technologies, new processes, new materials and new equipment.

参加加热炉及附属设备的技术交流和技术附件签订工作，配合做好新技术、新工艺、新材料、新设备的推广应用。

3.2  Coordinated management Position 协同岗位

3.2.1 The HSE Dept. shall be responsible for regular environmental monitoring of flue gas

emission of heating furnaces.

HSE专业管理人员负责定期对加热炉的烟气排放进行环保监测。

3.3 Executive Position 执行岗位

3.3.1 the Refinery No.2 equipment management personnel, professional process management personnel, professional HSE management personnel and shift group of the second refining department shall be the executive positions

炼油二部设备专业管理人员，工艺专业管理人员，HSE专业管理人员及班组为执行岗位。

3.3.1.1 Be responsible for the operation, daily maintenance and inspection of heating

furnaces involved in the department; Establish basic technical archives; Be responsible for

implementing the maintenance and repair works of heating furnaces and organizing the

acceptance.

负责本部门加热炉的运行操作、日常维护和巡检；建立基础技术档案；负责本部门加热炉维护、维修项目的实施并组织验收。

3.3.1.2 Be responsible for preparing and reporting the overhaul, update, inspection and

maintenance plans of heating furnaces involved in the department; Participate in the technical

exchange of heating furnaces and auxiliary equipment and the signing of technical appendices.

负责本部门加热炉大修、更新、检验、检修计划的编制和上报；参与加热炉及附属设备的技术交流和技术附件签订工作。

3.3.1.3 Be responsible for the formulation and implementation of the emergency plan for

major defects of heating furnaces involved in the department; Guarantee regular efficiency

monitoring and flue gas analysis for heating furnaces at least once a month; Be responsible for

the analysis of fuel gas and water quality as entrusted.

负责本部门加热炉重大缺陷应急预案的制定和落实工作；做好每月不少于一次的加热炉定期炉效监测及烟气分析工作；负责燃料气、水质等分析的委托工作。

# 4 Management Content 管理内容

4.1 Basic management work 基础管理工作

4.1.1 The operation department shall establish the basic files of heating furnaces, one file for

one furnace, and make a complete operation record.

本部门应建立加热炉基础档案资料，做到一炉一档，并有完整的运行记录。

4.1.1.1 The basic files shall include:

基础档案资料应包括：

(1) Heating furnace equipment records;

加热炉设备台帐；

(2) Structural drawing of complete equipment;

全套设备结构图纸；

(3) Operating procedures of heating furnace;

加热炉操作法；

(4) Accident plan, fault and accident records and cause analysis report of heating furnace;

加热炉事故预案、故障、事故记录及原因分析报告；

(5) Summary of Periodic Efficiency Monitoring and Analysis for Heating Furnace;

加热炉定期炉效监测分析汇总表；

(6) Maintenance, repair and technical renovation records and completion data;

检修、抢修、技术改造记录及竣工资料；

(7) Inspection report of furnace tube and fittings

炉管及附件检测报告。

4.1.1.2 The operating records shall include:

运行记录应包括：

(1) Process operation record;

工艺操作运行记录；

(2) Maintenance and repair records;

检、维修记录；

(3) Sulfur content analysis report of fuel

燃料含硫量分析报告。

4.1.2 The Equipment Manager shall organize trainings and exams on the required

knowledge of heating furnace operators who shall not take up their posts before passing the

trainings and exams.

设备专业管理人员应组织开展加热炉操作人员应知、应会知识培训和考试，合格后方可上岗。

4.1.3 Fuel gas for the heating furnace shall be measured separately; At the same time,

thermocouples, negative pressure meters and online oxygen content analyzers shall be

installed to ensure normal use.

加热炉燃料气应单独计量；同时安装热电偶、负压表和在线氧含量分析仪，并保证正常使用。

4.1.4 Each heating furnace shall be equipped with flue gas sampling points, for which the

requirements are shown in Appendix 1.

每台加热炉应设置烟气取样点，烟气取样点的要求见附件1。

4.1.5 The Equipment Manager quality analysis of fuel gas and make records. Control the

total sulfur content in fuel gas which shall be ≯ 100ppm, the total sulfur content in fuel oil which

shall be ≯ 1%, the content of solid particles which shall be ≯ 2000ppm and the Engler viscosity

of fuel oil which shall be ≯ 4.5E0 ; The pressure of fuel gas and steam system shall be

stable; The atomized steam shall be superheated with a pressure higher than that of the fuel oil

0.05MPa. The Equipment Manager, the Scheduling & Dispatch Dept. and the

operation department shall be responsible for putting forward the requirements for fuel gas

quality analysis, and the Lab. Dept. shall issue the analysis frequency and implement the

analysis

设备专业管理人员定期组织对燃料气的品质分析并建立台帐。控制燃料气中总硫含量≯100ppm，固体颗粒含量≯2000ppm，燃料气、蒸汽系统压力稳定；雾化蒸汽应为过热蒸汽，压力高于燃料气压力0.05MPa。设备专业管理人员负责提出燃料气品质分析需求并根据分析结果进行调整操作。

4.2 Operation and maintenance 运行维护

4.2.1 The heating furnace shall be operated in strict accordance with the operating

procedures and control indexes. It is strictly prohibited to operate under excessive

temperatures, pressures or loads. Also, try to avoid operating under excessively low loads (less

than 60% of the design load).

加热炉操作运行要严格按炼油二部操作规程和控制指标进行，班组进行操作时严禁超温、超压、超负荷，尽量避免过低负荷运行（指低于设计负荷的60%）。

4.2.2 The average thermal efficiency (hereinafter referred to as thermal efficiency) of the

heating furnace during the normal operation period shall reach the following indexes:

加热炉在正常运行周期内的平均热效率（以下简称热效率）应达到以下指标：

(1) The thermal efficiency of heating furnace with thermal load of 10MW or above shall reach

90% at least;

热负荷在 10MW 及以上的加热炉的热效率应达到 90%以上；

(2) The thermal efficiency of heating furnace with thermal load of less than 10MW shall reach

the design value.

热负荷在 10MW 以下的加热炉的热效率应达到设计值。

4.2.3 Energy control indexes for heating furnace operation:

加热炉操作运行节能控制指标：

(1) The exhaust gas temperature generally shall not be greater than 170 °C. calibration and

flue gas dew point test shall be carried out if the sulfur content in fuel significantly deviates from

the design value, and then the reasonable flue gas temperature of the heating furnace shall be

confirmed (generally 20 ~ 30 °C higher than the dew point temperature).

排烟温度一般应不大于 170℃，如燃料含硫量偏离设计值较大，则应进行标定和烟气露点测试，

然后确定加热炉合理的烟气排放温度（一般应高于露点温度 20～30℃）；

(2) The CO content in flue gas shall be ≤100ppm in general;

烟气中 CO 含量一般≤100ppm；

（3） The oxygen content in the flue gas at the top of the flow chamber shall be controlled at 2%

~ 4% for the heating furnace burning gas

对流室顶部烟气中的氧含量，燃气加热炉控制在2%～4%。

(4) The external wall temperature of heating furnace shall comply with relevant design

regulations, generally ≤80°C for the body and ≤90°C for the bottom.

加热炉外壁温度应符合相关设计规定，一般炉体要求≤80℃，炉底≤90℃。

4.2.4 The operation department shall tighten up the inspection and management of the

heating furnace operation and strictly implement the patrol inspection system.

部门要加强加热炉运行情况的检查和管理，并严格执行设备巡回检查制度。

4.2.4.1 The Equipment Manager of the production equipment shall:

设备专业管理人员应做好下列工作：

(1) Inspect the operation of heating furnace at least once a day;

每日不少于一次的加热炉运行情况巡检；

(2) Inspect the operation and problems of heating furnace once a week and make records;

每周进行一次加热炉运行情况和问题检查，并做好记录备案；

(3) Prepare the operation analysis report of the heating furnace of the device every month,

and complete the filling and reporting of "Summary of Efficiency Monitoring and Analysis for

Heating Furnace” in the EM system before the 25th day of each month, which will be postponed

in case of holidays.

每月编写本装置加热炉运行情况分析报告，并于 25 日前完成 EM 系统中“加热炉热效率监测汇总表”的填报，节假日顺延。

4.2.4.2 Operators of production equipment shall strengthen the regulation of the throttle,

valve, air door and flue damper to ensure that the furnace chamber is bright and clear, to avoid

the burner flame from being too long or too great and avoid smoking. Also, the flame shall not

burn the furnace tube. Try to keep the nozzles generating consistent flames to maintain efficient

operation.

生产装置操作人员要加强三门一板（油门、汽门、风门，烟道档板）的调节，保证炉膛明亮不浑

浊，避免燃烧器火焰过长、过大、冒烟，严禁舔管。要尽量保持多火嘴齐火焰，维持高效运行。

(1) The outdoor operator shall check the following items during patrol inspection:

外操人员巡检时要检查：

1) Check whether the burner and fuel gas system are coked and blocked, whether the

combustion is normal, and whether the pilot burner is normal; If the oil or gas gun is found

damaged or abnormally burning, it shall be replaced, adjusted or cleaned in time; For the

standby burner, the air door and valve shall be closed.

燃烧器、燃料气系统是否结焦堵塞、燃烧是否正常，长明灯是否正常点燃；对油枪或瓦斯

枪发现损坏或不正常燃烧、熄灭应及时更换或调整、清理等；对备用的燃烧器应关闭风门、汽门。

2)Check whether the sight glass, fire hole, ignition hole, explosion-proof door, manhole door

and elbow box are tight to prevent air leakage. Also, check whether the steel frame and steel

plate of furnace body are intact and tight and over-temperature.

看火窗、看火孔、点火孔、防爆门、人孔门、弯头箱是否严密，防止漏风。检查炉体钢架和炉体

钢板是否完好严密，是否超温。

3）Each shift shall check whether the radiation furnace tube is subjected to local over burning,

cracking, bulking, bending and other abnormal phenomena, whether the inner lining of the

furnace is free of falling off, whether the components in the furnace are free from abnormalities

每班检查辐射炉管有无局部过烧、开裂、鼓包、弯曲等异常现象，检查炉内壁衬里有无脱落，炉

内构件有无异常，仪表监测系统是否正常。

4）Each shift shall check whether the burner’s air regulation system, air door and damper are

flexible and easy to use, whether the induced draft fan and blower of waste heat recovery

system are in normal operation, and whether the steam fire-extinguishing system is in good

condition.

每班检查燃烧器风量调节系统、风门档板是否灵活好用，余热回收系统的引风机、鼓风机是否正

常运行，蒸汽灭火系统是否完好。

(2) Indoor operators shall strictly follow the operating procedures for proper heating furnace

process operation control and always pay attention to the heating furnace’s feeding and

discharging system, including flow control, branch flow control, pressure control and flow,

pressure, temperature, oxygen content indications which shall be checked for abnormality or

deflection. In addition, the operators shall track whether the furnace control indicators are

normal in a real-time manner, and find out the cause for any abnormality and timely adjust and

eliminate the abnormality.

内操人员检查要求：应严格按操作法做好加热炉工艺操作控制，随时关注加热炉进出料系统，包

括流控、分支流控、压控及流量、压力、温度、氧含量的指示是否正常、有无偏流等。实时跟踪

加热炉各项控制指标是否正常，情况异常必须查明原因并及时调整处理。

4.2.5  Furnace shutdown must be strictly in accordance with the relevant provisions of the

technical specifications and post operation methods. A strict and detailed shutdown plan shall

be prepared before shutdown, specially indicating the requirements of preventing sulfides from

spontaneous combustion in the convection chamber and austenitic stainless steel furnace tube

from being stressed, corrosion and cracking due to polythionic acid. For this, dedicated

personnel shall be arranged to manage these matters in actual implementation. For the

austenitic stainless steel furnace tube of the convection chamber, spontaneous combustion can be prevented through chemical cleaning of the chamber. For the austenitic stainless steel

furnace tube of the radiation chamber, alkali cleaning of furnace tube is available.

加热炉开停工必须严格按照技术规程、岗位操作法的有关规定执行，开停工前必须制定详细严谨

的开停工方案，停工方案中特别要注明防止硫化物在对流室内自燃，以及防止连多硫酸造成奥氏

体不锈钢炉管应力腐蚀开裂要求，在实际执行中对硫化物自燃和防止连多硫酸造成奥氏体不锈钢

炉管应力腐蚀开裂要落实专人管理。对于对流室奥氏体不锈钢炉管可以结合对流室化学清洗进行

预防，辐射室奥氏体不锈钢炉管可安排炉管的碱洗。

4.2.6 The operation department shall make emergency plans and submit to the head of the

operation division of the Company for approval in case the heating furnace needs to be

monitored for operation due to major defects. The operators of production equipment must

seriously implement the special maintenance program during daily inspection.

加热炉因存在重大缺陷需监控运行时，运行部应制定应急预案，并报公司分管领导批准执行。生

产装置操作人员在进行日常检查时，必须认真落实特护方案。

4.2.7  The oxygen content in the daily operation of the heating furnace is that in zirconia. The

operation department shall contact the Instrument Control Dept. for checking and report the

processing information to the Equipment Management Dept. in a big difference lies between

the analysis value given by the Lab. Dept., the monitoring value of furnace efficiency and the

online zirconia display value.

加热炉日常运行氧含量以氧化锆为准，当质量检验部化验分析值、炉效监测值与在线氧化锆显示

值差别较大时，运行部要及时联系仪表控制部进行校核并向机械动力部反馈处理信息。

4.3  Check the maintenance检维修

4.3.1 The Equipment Manager shall carefully prepare the furnace maintenance plan, fill in

the overhaul, update and inspection items and report to the Equipment Management Dept. for

approval before implementation according to the daily maintenance and shutdown inspection &

testing results of the heating furnace as well as relevant provisions of the "Rules for

Maintenance and Overhaul of Tubular Heating Furnace" (SHS01006-2004).

设备管理人员根据对加热炉日常维护和停工检查、检测的结果，对照《管式加热炉维护检修规程》

SHS01006-2004 的有关规定，认真编制加热炉检修计划，填报大修、更新和检验项目，报机械

动力部批准后实施。

4.3.2   The Equipment Manager shall strengthen the daily maintenance of the heating furnace, especially the maintenance of the induced (forced) draught fan, flue damper, soot blower and

other accessories and their actuators. In case of any problem, timely repair and troubleshooting

are necessary to guarantee normal operation of the heating furnace.

设备管理人员加强加热炉的日常维护，特别是对引（鼓）风机、烟道挡板、吹灰器等附件及其执行机构的保养。发现问题要及时修理，排除故障，不得影响加热炉的正常运行。

4.3.3  For shutdown for maintenance, the radiation furnace tube must be inspected and tested

for such items as appearance inspection and measurement, thickness and hardness measurement, surface metallographic examination, ultrasonic testing, X-ray inspection of

welding joints or other nondestructive testing and analysis of scale on the surface of furnace

tube. The testing items shall be arranged and implemented by qualified testing units according

to the actual situation, and the problems found in the inspection and testing shall be dealt with in time or preventive measures shall be taken.

停炉检修时，必须进行辐射炉管的检查检测，检测项目主要包括：外观检查及测量；测厚及硬度

测量；表面金相检测；超声波检测、焊缝 X 射线拍片检测或其它无损检测；炉管表面垢物分析等。检测项目应根据实际情况安排并落实有资质的检测单位进行，检查检测中发现的问题应及时处理或制定防范措施。

4.3.5  The inspected items shall be accepted according to the relevant rules and regulations.

检修完的项目应按有关规程、规范要求进行检查验收。

# 5　Associated Procedures and Records 关联程序和记录

5.1  Associated procedures 关联程序

5.1.1 Heating Furnace Management Procedure (HYBN-T2-07-0024-2018-1)

加热炉管理程序 HYBN-T2-07-0024-2018-1

5.2  Associated records 关联记录

5.2.1 Summary of Efficiency Monitoring and Analysis for Heating Furnace

(HYBN-T6-07-0073-001-2018)

加热炉热效率监测汇总表 HYBN-T6-07-0073-001-2018

5.2.2  Summary of Periodic Efficiency Monitoring and Analysis for Heating Furnace

(HYBN-T6-07-0074-001-2018)

加热炉定期炉效监测分析汇总表 HYBN-T6-07-0074-001-2018

# 6　Supplementary Rules附则

6.1 The rules are managed by the No.2 Refinery Dept. For the unfinished matters, please refer to the company's professional management systems.

本细则由炼油二部归口管理，未尽事宜参照公司各专业管理制度等执行

6.2   The preparation and approval of the version of these rules are shown in Table 1.

本细则版本编制和审批情况见表1。

Table 1 Revision, preparation and approval of document

表1 文件版本编制和审批情况

| 2 | 01/05/2024 | KONG QING ZHAN 孔庆站 | ADILI BULIBULI 阿地里·布力布力 | SUN JIAN HUAI孙建怀 |
| --- | --- | --- | --- | --- |
| Revision 版本 | Issued date 颁布日期 | Prepared by 编制人 | Reviewed by 审核人 | Approved by 批准人 |

# 7  Appendices 附件

7.1  Requirements for flue gas sampling point 烟气取样点要求

Appendix 1 附件 1

Requirements for Flue Gas Sampling Point

烟气取样点要求

1 Setting of flue gas sampling point

烟气取样点位置设置

(1) Radiation chamber: for any heating furnace with two or more radiation chambers, each

radiation chamber shall be equipped with flue gas sampling points at proper locations to check

whether the chambers are provided with the same air supply. No flue gas sampling points shall

be provided for the heating furnace with one radiation chamber.

辐射室：对有两个或两个以上辐射室的多室加热炉，每个辐射室的相应位置应安装烟气取样点，

以便检查各室供风是否相同，单室可不装；

(2) Point before flue gas enters the convection chamber: Use this point to check the actual

situation of air supply required by combustion and control the air flow of the burner (it can be

shared if sampling points are available to the radiation chamber);

进对流室前：利用此点检验供给燃烧所需空气量的实际情况，控制燃烧器用风量（辐射室已设置

取样点的，可以共用）；

(3) Point after flue gas leaves the convection chamber: Use the excess air coefficient at this

point to calculate the thermal efficiency of the heating furnace and check the air leakage of the

convection chamber;

出对流室后：利用此点的过剩空气系数计算加热炉的热效率，并检验对流室的漏风情况；

(4) In case the waste heat recovery system is used, the flue gas sampling points shall be set

before and after the air preheater or the waste heat boiler, and the flue gas temperature after

the air preheater shall be used to calculate the thermal efficiency of the furnace;

采用余热回收系统时，应在空气预热器或余热锅炉前后均装设烟气取样点，利用空气预热器后的

烟气温度计算加热炉热效率；

(5) The dew-point temperature sampling point shall be set at the outlet of the last-level heated

surface. The sampling point shall be set at the outlet of the convection chamber for the heating

furnace without waste heat recovery system and at the outlet of the air preheater or waste heat

boiler for the furnace with a waste heat recovery system.

露点温度取样点设置在最后一级受热面出口，对于无余热回收系统的加热炉应设在对流室出口，

采用余热回收系统的加热炉设在空气预热器或余热锅炉出口。

２ Size of sampling hole

测孔大小

(1) The flue gas sampling hole shall have a diameter of no less than 20mm;

烟气取样点孔径不小于 20mm；

(2) The dew-point temperature sampling hole shall have an inner diameter of no less than

40mm.

露点温度取样点孔内径不小于 40mm。

3 Precautions for installation of flue gas sampling point

烟气取样点安装注意事项

(1) No air leakage shall occur near the sampling point;

取样点附近不能有漏风；

(2) The sampling point should be set at the narrowest part of the flue duct if possible, where

the velocity of flue gas is high enough to achieve thorough mixing of the combustion products;

取样点应尽可能选在烟道的最窄处，在该处有较高的烟气流速，促使燃烧产物充分混合；

(3) The sampling point shall extend to the middle of the flue gas flow section. In case of any

difficulty, the sampling point shall strength into the flow section for at least 500mm. Multi-point

sampling can reduce the measurement error;

取样点最好伸至烟气流通截面的中部，有困难时，伸入长度至少在 500 毫米以上，多点取样可以

减少测量误差；

(4) The sampling point for combustion control shall not be placed behind the convection

chamber; The sampling point for measuring the thermal efficiency shall be set in the non-eddy

area of the stack, that is, not in the turning area.

控制燃烧的取样点不应放在对流室以后；测定热效率的取样点应放在烟囱内非涡流区，即不应设

在转向区域；

(5) The wide-section flues shall be provided with facilities with multiple sampling points, or

sampling with a single tube shall be adopted.

对于宽截面烟道，应设置多点取样设施，或采用单管伸缩式取样法。