Hengyi Industries Sdn Bhd

恒逸实业（文莱）有限公司

HYBN-T9-11-0046-2024-1

Refinery No.2 Dept. Special Care Management Rules for Large Compressors

炼油二部大机组特护管理细则

Issued Date：May 2024      颁布日期：2024年5月

Preface前 言

This rule is drafted and formulated by No. 2 Refinery Department.

本管理细则由炼油二部起草和制定。

This rule is effective as of March 26, 2021.

本细则从2021年3月26日起实施。

此版为第二版，从2024年5月1日起实施，第一版废止。

# Content目录

1  Purpose 目的	1

2  Scope of Application 适用范围	1

3  Terms and Definitions 术语和定义	1

4  Management Responsibilities 管理职责	1

5  Manage content 管理内容	2

6　 Inspection and Supervision 检查与监督	5

7  Associated Procedures and Records 关联程序和记录	5

8　Supplementary Rules 附则	6

9  Appendices 附件	8

|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Refinery No.2Special Care Management Rules for Large Compressors炼油二部大机组特护管理细则 |  |  |  |  |
|  | Doc No. | HYBN-T9-11-0046-2024-1 | Ver No. | 2 | Page 1 of 11 |

# 1  Purpose 目的

This System is formulated to ensure that the operation of large compressors is under comprehensive and systematic monitoring and guarantee the safe operation of devices in a long period.

为确保大机组运行状况处于全面、系统的监控状态，保证装置长周期安全运行，特制订本制度。

# 2  Scope of Application 适用范围

This System is applicable to the special care management of large compressors of the in the Refinery No.2.

本制度适用于炼油二部各装置大机组的特护管理。

# 3  Terms and Definitions 术语和定义

Large compressors: centrifugal compressors and reciprocating compressors with a shaft power of not less than 500KW.

大机组：指轴功率大于等于500KW的离心式压缩机和往复式压缩机。

# 4  Management Responsibilities 管理职责

4.1 Responsibilities of equipment management personnel 设备管理人员的职责

4.1.1　Formulate the working standard of the department's top-grade maintenance equipment circuit inspection, define the content and requirements of the special care inspection, and report to the Mechanical power Department for the record.

制定本部门特级维护设备巡回检查的工作标准，明确特护检查的内容和要求，报机械动力部备案。

4.1.2  Responsible for daily inspection of special care equipment; Participate in the weekly inspection and monthly evaluation of special care equipment organized by the Mechanical power Department; Complete all tasks assigned by the appraisal meeting on time.

负责特护设备日检工作；参加机械动力部组织的特护设备周检及月讲评；按时完成讲评会布置的各项工作。

4.1.3　 Deal with general problems found in daily and weekly inspection, report to mechanical power department for major problems and take corresponding measures.

处理日检、周检中查出的一般问题，对于重大问题上报机械动力部并采取相应措施。

4.1.4　 Participate in the test and confirmation of the interlock protection system after the overhaul of the special care equipment; Responsible for the management of special care equipment brand.

参与特护设备大修后联锁保护系统的试验、确认；负责特护设备牌的管理。

4.2 Duties of the class 班组的职责

Responsible for the special care work (daily inspection) of each large unit and key equipment during the shift, careful operation and maintenance, and ensure the safe and stable operation of the special care equipment.

负责当班期间的各大型机组和关键设备的特护工作（日检），精心操作维护，搞好特护设备的安全平稳运行。

# 5  Manage content 管理内容

5.1　Special equipment 特护设备

See Attachment 1 for the list

清单见附件1

5.2　Intensive care personnel 特护人员

The shift intensive-care personnel shall be those on the list of intensive-care personnel on duty. The special care personnel of the department management shall be the equipment technicians. The list of daily and weekly special care personnel shall be submitted to the Mechanical power Department for approval, and personnel change shall be approved by the mechanical power Department.

班组的特护人员由当班特护人员名单内人员担任。部门管理层的特护人员由设备技术人员担任。日检、周检特护人员名单续报机械动力部审批，人员变更须经机械动力部批准。

5.3　 Daily inspection 日检

5.3.1　 The special care personnel for daily inspection shall check the special care equipment twice a day according to the working standard with the special care card put on from 8:00 to 10:00 and from 14:00 to 16:00. Moreover, the personnel shall check the operating parameters and conditions of the special care equipment, and fill in the Daily Inspection and Countersignature Records of the Special Care Equipment.

日检特护人员每天按工作标准对特护设备巡回检查两次并挂特护牌，时间为8:00～10:00、14:00～16:00，对特护设备的运行参数及状况进行检查，并填写《特护设备日检会签记录表》。

5.3.2　Plant supervisor technician shall inspect the special care equipment once a day with the special care card put on, fill in the column of "Comments of Technical Personnel" in the Daily Inspection and Countersignature Records of the Special Care Equipment, and check the implementation of the daily inspection of special care personnel.

装置主管技术人员应对特护设备每天巡检一次，并挂特护牌，填写《特护设备日检会签记录表》中“装置技术人员意见”栏，并检查作业人员特护日检执行情况。

5.3.3　The daily inspection for special care shall be uninterrupted throughout the year, and the daily inspection on holidays shall be carried out by the personnel on duty.

全年特护日检不间断，节假日的日检由值班人员负责。

5.3.4　One piece of Daily Inspection and Countersignature Records of the Special Care Equipment shall be prepared every month and placed in the specified location and managed by the operation department, and kept for 1 year

《特护设备日检会签记录表》每月一本，放置在指定位置，由各区域负责管理，保存期为1年。

5.3.5  The problem found by the special care personnel in the daily inspection shall be promptly addressed or reported to relevant personnel to implement the special care management procedures for large compressors.

特护人员在日检中发现的问题应及时处理或通知有关人员，执行大机组特护管理程序。

5.4　 Weekly inspection 周检

5.4.1  The special care personnel for weekly inspection shall pay attention to the running state of compressors, and check the running status and parameters of the special care equipment and the implementation of special care daily inspection after gathering on every Wednesday morning or at the time as specified by the director of the Equipment Management Dept. at the place specified by the Equipment Management Dept. The special care card shall be put on as follows: the card for the “first week” shall be put on for the first weekly inspection every month and the “second week” for the second weekly inspection, and so on. In addition, the Weekly Inspection and Countersignature Records of the Special Care Equipment shall be filled in and kept by the Equipment Management Dept. for future reference for 1 year.

周检特护人员应关注机组运行状态，并于每周三上午或按机械动力部主管人员通知时间在指定地点集合后，对特护设备的运行状况、运行参数、特护日检情况等进行检查，并挂特护牌，每月第一次周检则挂“一周”，第二次周检则挂“二周”，依此类推。填写《特护设备周检会签记录本》，由机械动力部保存备查，保存期为1年。

5.5　 Monthly evaluation月讲评

5.5.1　The department shall collect the equipment operation data of the current month and the task completion information of last month before the monthly evaluation, and assign personnel to participate in the evaluation on time, and fill in the Operation Situation of Special Care Equipment before the 5th day of each month

部门在月讲评前应做好本月设备运行情况和上月任务完成情况的收集，并指派人员按时参加讲评，在每月5日前填报《特护设备运行情况表》。

5.5.2　The evaluation shall focus on the common problems found and good experience obtained in the operation and management of special care equipment, on the key technical problems requiring multi-discipline cooperation, and on the main operation, online monitoring and maintenance status, major defects, rectification, major technical renovation and application of new technologies of the special care equipment.

讲评内容：特护设备运行和管理中出现的共性问题和好的经验；需要多专业协同解决的关键技术问题；特护设备的主要运行、在线监测、检修状况、重要缺陷、整改情况、重大技术改造及新技术应用情况。

5.6  Emergency management 应急管理

5.6.1　 The Equipment Management Dept. shall organize the operation department and the Maintenance Dept. to guide the startup or major operation of the special care equipment after overhaul. In case the special care equipment is abnormal or must run with defects, the operation department shall inform the Equipment Management Dept. which shall lead to organize consultation and propose corresponding measures.

特护设备在大修后的开车或重大操作，设备管理人员通知机械动力部组织相关维保部门到场进行指导；特护设备出现异常或需带病运行时，设备管理人员应通知机械动力部，由机械动力部牵头组织进行会诊，提出相应措施。

5.7  Others 其它

5.7.1　Special care shall be suspended during the maintenance of the special care equipment.

特护设备检修期间，暂停特护工作。

5.7.2　 Bladder type hydropneumatic accumulators for energy storage play an important role in energy storage and pressure stabilization in the oil system of special care equipment and are of great significance in reducing pressure fluctuations in the oil system caused by switching operations of oil pumps and oil filters and ensuring the smooth operation of the compressor. See Appendix 2 for the application requirements of the accumulators.

能量存储用囊式蓄能器在特护设备油系统中起到存储能量、稳定压力的重要作用，对于减少油泵、滤油器切换等操作引起的油系统压力波动，保障机组的平稳运行有重要的意义，其使用要求见附件2。

# 6　 Inspection and Supervision 检查与监督

6.1　 The special care personnel who carry out the inspection conscientiously, take the initiative to eliminate equipment defects, or find major hidden dangers of equipment during the inspection, avoid equipment damage and prevent equipment accidents, shall be given appropriate rewards according to the regulations of the Ministry of Economic Responsibility assessment.

对认真进行巡检，主动消除设备缺陷，或巡检中发现设备重大隐患，避免设备损坏以及防止设备发生事故的特护人员，根据部经济责任考核条例给予适当奖励。

6.2　 The special care personnel who fail to perform special care duties, delay the treatment of key equipment or affect the normal operation of equipment and cause equipment damage shall be assessed according to relevant regulations.

对不履行特护职责，延误关键设备的处理或影响设备正常运行而造成设备损坏的特护人员，根据有关规定进行考核。

6.3　 The special care personnel did not carry out the inspection according to the specified time, and were assessed according to the performance assessment rules of the second part of oil refining.

特护人员没按规定时间进行检查，按炼油二部绩效考核细则进行考核。

# 7  Associated Procedures and Records 关联程序和记录

7.1  Associated procedures 关联程序

7.1.1  Special Care Management Procedures for Large Compressors

(HYBN-T2-07-0005-2018-1)

大机组特护管理程序  HYBN-T2-07-0005-2018-1

7.1.2　Emergency Management Procedures for Large Compressors

(HYBN-T2-07-0006-2018-1)

大机组应急管理程序  HYBN-T2-07-0006-2018-1

7.2  Associated records 关联记录

7.2.1　1030-K101A Daily Inspection and Countersignature Records

(HYBN-T6-07-0011-001-2018)

1030-K101A日检会签记录表  HYBN-T6-07-0011-001-2018

7.2.2　1030-K101B Daily Inspection and Countersignature Records

(HYBN-T6-07-0012-001-2018)

1030-K101B日检会签记录表  HYBN-T6-07-0012-001-2018

7.2.3　1040-K101 Daily Inspection and Countersignature Records

(HYBN-T6-07-0013-001-2018)

1040-K101日检会签记录表模板  HYBN-T6-07-0013-001-2018

7.2.4　1040-K102A Daily Inspection and Countersignature Records

(HYBN-T6-07-0014-001-2018)

1040-K102A日检会签记录表模板  HYBN-T6-07-0014-001-2018

7.2.5　1040-K102B Daily Inspection and Countersignature Records

(HYBN-T6-07-0015-001-2018)

1040-K102B日检会签记录表  HYBN-T6-07-0015-001-2018

7.2.6　1040-K102C Daily Inspection and Countersignature Records

(HYBN-T6-07-0016-001-2018)

1040-K102C日检会签记录表  HYBN-T6-07-0016-001-2018

7.2.7　Operation Situation of Special Care Equipment (HYBN-T6-07-0053-001-2018)

特护设备运行情况表  HYBN-T6-07-0053-001-2018

7.2.8  List of Special Care Personnel for Special Care Equipment

(HYBN-T6-07-0054-001-2018)

特护设备特护人员申报表  HYBN-T6-07-0054-001-2018

7.2.9  List of Special Care Personnel for Special Care Equipment

(HYBN-T6-07-0055-001-2018)

特护设备特护人员一览表  HYBN-T6-07-0055-001-2018

7.2.10  Monthly Evaluation Records of Special Maintenance of Special Care Equipment (HYBN-T6-07-0056-001-2018)

特护设备特级维护月讲评记录  HYBN-T6-07-0056-001-2018

7.2.11  List of Special Care Equipment (HYBN-T6-07-0057-001-2018)

特护设备特护设备清单  HYBN-T6-07-0057-001-2018

7.2.12  Inspection Records of Air Pressure of Bladder of Accumulator

(HYBN-T6-07-0058-001-2018)

蓄能器皮囊气压检查记录表  HYBN-T6-07-0058-001-2018

# 8　Supplementary Rules 附则

8.1  This System is under the jurisdiction of Refinery No.2.

本制度由炼油二部归口管理。

8.2  This System is drafted by Refinery No.2

本制度起草部门：炼油二部。

8.3  Refinery No.2 is responsible for the interpretation of this System

本制度解释权归炼油二部拥有。

8.4  Revision, preparation and approval of this System are shown in Table 1:

本制度版本编制和审批情况见表1：

Table 1 Revision, preparation and approval of document

表1 文件版本编制和审批情况

| 2 | 01/05/2024 | GAO JUN JIE 高俊杰 | ADILI BULIBULI 阿地里·布力布力 | SUN JIAN HUAI孙建怀 |
| --- | --- | --- | --- | --- |
| Version 版本 | Issued Date 颁布日期 | Compiler 编制人 | Reviewer 审核人 | Approval 批准人 |

# 9  Appendices 附件

Appendix 1	List of Special Care Equipment of Refinery No.2

附件1  炼油二部特护设备清单

Table 2 List of Special Care Equipment of Refinery No.2

表2  炼油二部特护设备清单

| 所属单位 | 装置名称 | 设备名称 | 设备位号 |
| --- | --- | --- | --- |
| Refining Dept. #2 炼油二部 | 2.2 MMTPA Diesel Hydrotreating Unit 220 万/年柴油加氢精制 | Makeup hydrogen compressor 新氢压缩机 | 1030-K101A/B |
| Refining Dept. #2 炼油二部 | 2.2 MMTPA Diesel Hydrocracking Unit 220 万/年加氢裂化 | Recycle hydrogen compressor 循环氢压缩机 | 1040-K101 |
| Refining Dept. #2 炼油二部 | 2.2 MMTPA Diesel Hydrocracking Unit 220 万/年加氢裂化 | Makeup hydrogen compressor 新氢压缩机 | 1040-K102A/B/C |

Appendix 2	Application Requirements of Bladder Type Hydropneumatic Accumulators for Energy Storage

附件 2	能量存储用囊式蓄能器使用要求

1 Installation 安装

1.1 The air charging valve of the accumulator shall be installed vertically and upwardly, with sufficient space reserved for maintenance, inspection and repair.

蓄能器应充气阀朝上垂直安装，并留有足够的维护、检查和检修空间。

1.2 The accumulator shall be fixed reliably and shall not be fixed by direct welding.

蓄能器应固定可靠，不得直接焊接固定。

1.3 A shut-off valve (valve 1) shall be installed between the accumulator and the pipeline to be used in case of air filling, oil filling or long-time shutdown; An oil drain valve (valve 2) shall be set behind the shut-off valve. If possible, a pressure gauge shall be installed in front of the oil drain valve for online inspection of air bladder’s pressure (as shown in the figure on the right). An additional accumulator of the same specification can be connected in parallel if possible for the convenience of switching during inspection. Moreover, a small-diameter oil inlet bypass can be added to prevent oil shock.

蓄能器和管路之间应安装切断阀（阀 1），供充气、充油或长时间停机时使用；切断阀后应设置放油阀（阀 2

Oil return Oil supply

Valve 1

Valve 2

Valve 3

），可能的情况下在放油阀前安装压力表，供在线检查气囊压力时使用（见右图）。条件允许的情况下可额外并联一只同规格的蓄能器，以备检查时切换。为防止进油冲击，可增设小口径进油旁路。

1.4 The inflating needle valve cover of the accumulator shall be provided with an "O" ring to prevent air leakage from the bladder.

蓄能器充气针阀阀盖应设置“O”型圈，以防止皮囊向外漏气。

2 Nitrogen filling 氮气的充装

2.1 Before nitrogen filling, the shut-off valve between the slow accumulator and the pipeline shall be slowly closed, and the oil valve shall be opened to slowly drain the oil in the accumulator.

充气前应关闭缓蓄能器和管路之间的切断阀，打开放油阀，缓慢排净蓄能器中的油。

2.2 Nitrogen shall be filled with slowly to avoid damage of the bladder

充气应缓慢进行，以防冲破皮囊。

2.3 Oxygen, compressed air or other flammable gases shall not be filled with.

不得充装氧气、压缩空气或其他易燃气体。

2.4 Nitrogen filling shall be performed with special tools attached to the unit which shall be properly kept by the operation department.

氮气的充装使用随机所带的专用工具，该工具由运行部妥善保管。

2.5 Charging pressure standard: the charging pressure of the bladder of lubricating oil accumulator shall be 80% of the lower limit of the lubricating oil pressure and that of the control oil accumulator shall be 80% of the lower limit of the control oil pressure. The charging pressure of the system accumulator bladder shall be 80% of the lower limit.

充气压力标准：润滑油蓄能器皮囊充气压力为润滑油压低报警值的 80%；控制油蓄能器皮囊充气压力为控制油压力低报警值的 80%；系统蓄能器皮囊充气压力为低报警值的 80%。

2.6 Check the air tightness of the inflating needle valve of the accumulator after charging, screw on the valve cover, and check the air tightness of the valve cover.

充气完成后应检查蓄能器充气针阀气密性，然后拧上阀盖，再检查阀盖的气密性。

3 Examination and maintenance检查和维护

3.1 The bladder pressure shall be periodically checked, with records made by following the format as described later.

皮囊气压应定期检查，并做好纪录，记录格式后附。

3.2 Inspection period: the accumulator in normal operation shall be inspected every six months; The bladder pressure shall be checked once the accumulator is stopped for maintenance.

检查周期：正常运行的蓄能器每六个月检查一次；每次机组停机检修时，应检查皮囊气压。

4 Inspection method of bladder pressure皮囊气压的检查方法

4.1 Close the shut-off valve between the accumulator and the pipeline, slowly drain the oil in the accumulator, and directly check the charging pressure with the charging tool.

关闭蓄能器和管路之间的切断阀，缓慢排净蓄能器中的油，利用充气工具直接检查充气压力。

4.2 For the inspection of accumulator with a pressure gage in front of the oil drain valve, the shut-off valve between the accumulator and the pipeline shall be slowly closed and the oil drain valve shall be slowly opened to check the indication change of the pointer. The pointer slowly drops at the beginning and then rapidly drops when a certain pressure is reached. The pressure where sudden change occurs to the moving speed of the indicator is the charging pressure.

在放油阀前有压力表的蓄能器，检查时关闭缓蓄能器和管路之间的切断阀，缓慢打开放油阀，注意压力表指针的变化，开始时指针慢慢下降，达到某压力时，急速下降到零，指针移动的速度发生突变的压力值即为充气压力。

4.3 The operator shall check the inflating needle valve of the accumulator and other sealingpoints for air tightness once a month and make records.

操作人员每月一次检查蓄能器充气针阀和其他密封点的气密性，并做好记录。

4.5 When the air pressure of the bladder is less than 70% of the standard charging pressure, nitrogen shall be added and the final charging pressure shall be recorded.

气囊气压小于充气压力标准的 70%时，应补充氮气并纪录最终充气压力。

4.6 The shut-off valve between the accumulator and the pipeline shall be closed in case of long-time suspension of the accumulator, thus to keep the oil pressure in the accumulator above the charging pressure.

蓄能器长期停用时，应关闭蓄能器和管路之间的切断阀，保持蓄能器里的油压在充气压力以上。

4.7 For any accumulator listed as a pressure vessel, the pressure bearing shell shall be inspected periodically in accordance with the Specifications for Vessel. For an accumulator not listed as a pressure vessel, the pressure bearing shell shall be inspected periodically by referring to the specifications for pressure vessels.

蓄能器已列入压力容器的，承压壳体按《容规》定期检验，不属于压力容器的，参照压力容器定期检验。

4.8 Before the accumulator is disassembled for maintenance, the shut-off valve between the accumulator and the pipeline shall be closed, the pressure oil shall be removed, and the nitrogen in the bladder shall be removed with the charging tool.

蓄能器检修拆卸前应关闭蓄能器和管路之间的切断阀，卸去压力油，使用充气工具放掉皮囊中的氮气。

4.9 Oil filling shall be conducted slowly (preferably through a small-diameter bypass) when the accumulator is put into use after inspection and maintenance, to avoid system pressure fluctuation.

蓄能器检查和检修结束投用时，应缓慢充油（最好用小口径旁路），避免引起系统压力波动。

4.10 The accumulator bladder shall be replaced in case of overhaul of the accumulator.

蓄能器皮囊更换周期与机组大检修同步。