|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Equipment and Facilities List 设备设施清单模板 |  |  |
|  | Template No. | HYBN-T7-08-2008-2024-2 | Page 1 of 2 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6- -2008-0016-2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  | 公用工程部 |  |  | Name of unit (area) 单元（区域）名称 |  |  | circulating water 1 第一循环水场 |
| Reviewed by 审核人 |  |  | Shishuigen施水根 |  |  | Date 日期 |  |  | 2025年2月 10 日 |
| S/N 序号 | No. of SCL form SCL表编号 |  |  | Name of equipment/facility 设备设施名称 |  |  | Inspection item 检查项目 |  |  |
| 1 | 001 |  |  | 5101-P101A/B/C/D、P102A/B Circulation pump operation machine pump inspection 5101-P101A/B/C/D、P102A/B 循环水泵运行机泵检查 |  |  | 1.whether the motor bearing grease is overflowing, refilling records.电机轴承润滑脂是否溢出，加注记录； 2.whether the motor temperature and current meet the requirements.电机温度，电流是否符合要求； 3.Whether the motor vibration values meet the requirements电机振动值是否符合要求； 4.Is the motor noise increasing电机杂声是否增大； 5.whether the pump bearing grease is overflowing, refilling records机泵轴承润滑脂是否溢出，加注记录； 6.Whether the temperature of the pump bearings meets the requirements.机泵轴承温度是否符合要求； 7.Whether the vibration values of the machine pump are in accordance with the requirements.机泵振动值是否符合要求； 8.Whether the outlet pressure of the pump is normal.机泵出口压力是否正常； 9.There are no leaks in the machine seal.机封有无泄漏。 |  |  |
| 2 | 002 |  |  | 5101-CWT101A~F Cooling tower inspection - CWT101A~F 冷却塔检查 |  |  | 1.The wind turbine is secure and free from damage.风筒是否牢固、有无破损情况； 2.Checking for uniformity of water distribution.检查布水是否均匀； 3.Check the water collector, nozzle and filler for breakage.检查收水器、喷头、填料有无破损情况。 |  |  |
| 3 | 003 |  |  | 5101-FAN101A~F 冷却塔风机系统 |  |  | 1.Whether the motor bearing grease is overflowing, refilling records电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current are normal.电机温度、电流是否正常； 3.Whether the motor vibration values meet the requirements.电机振动值是否符合要求； 4.；Whether there is an increase in motor noise.电机杂声是否增大； 5.Whether the oil level of the gearbox is normal.减速箱油位是否正常； 6.Whether the vibration of the reducer meets the requirements.减速机振动是否符合要求。 |  |  |
| 4 | 004 |  |  | SR201A~H bypass filter SR201A~H 旁滤器 |  |  | 1.Confirm the normal status of each pneumatic valve确认各气动阀门的状态是否正常； 2.Whether the differential pressure between the inlet and outlet of the by-pass filter is normal.旁滤器进出口压差是否正常； 3.Whether the display and operation of the on-site PLC control panel are normal.现场PLC控制面板显示、操作是否正常； 4.Confirm that there are no "runs and leaks".确认有无“跑冒滴漏”。 |  |  |
| 5 | 005 |  |  | Dosing room 加药间 |  |  | 1.；Doors and windows are not damaged and do not close.门窗有无损坏、关不上情况 2.Whether the exhaust fan is working properly.排风扇是否能正常工作； 3.Eye wash water quality, water pressure is normal.洗眼器水质、水压否正常； 4.The conformity of the piles of pharmaceuticals with the requirements.药剂堆放是否符合要求； 5.The level of the chemical storage tank and the normal operation of the chemical pump.药剂储罐液位、药剂泵运行是否正常； 6.Safety signs are complete.安全标识是否齐全。 |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。