|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 001 |  |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  |  | Diesel fuel 加注柴油 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  |  | Diesel fuel 加注柴油 |  | Vehicle driving safety 车辆行驶安全 |  | Personal injury/equipment damage 人员伤害/设备损坏 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | Oil leakage 油品泄漏 |  | Environmental pollution 环境污染 |  | Occasionally 偶尔 | 3 | 3 | 9 | III | Strengthen the inspection of pipelines and joints before unloading卸油前管线、接头处检查 |  |  | 2 | 2 | 4 | I |
|  |  |  |  |  | Refuel the tank with diesel fuel. 向油箱内加注柴油 |  | Pollution of the environment caused by leakage during the annotation process 加注过程中泄漏造成环境污染 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 002 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Crank 盘车 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 2 |  | Crank 盘车 |  | Internal and external operations are not contacted, the internal operations manually start the equipment 内外操未联系，内操手动启动设备 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | The scene did not hit “manual” or “stop” 现场未打到“手动”或“停止” |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | The device is running 设备处于运行状态 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | Failure to crank 2-3 laps as required 未按规定盘车2-3圈 |  | Equipment damage 设备损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 003 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Cut pump 切泵 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 3 |  | Cut pump 切泵 |  | The standby pump status is not checked and confirmed 未检查确认备用泵状态 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | Hit the rotating parts when starting the pump 启泵时碰到转到部件 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | When the pump is cut, the pump is holding back pressure 切泵时机泵憋压 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | The flow of the pump is interrupted or fluctuates greatly when the original operation is stopped 停原运行泵流量中断或波动较大 |  | Affect the stable operation of subsequent devices 影响后续装置稳定运行 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 004 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | sampling 取样 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 004 |  | sampling 取样 |  | Sampling bottle is not cleaned取样瓶未清洗 |  | Analysis error 分析误差 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | The sampler is not cleaned in place 取样器清洗不到位 |  | data error 数据错误 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Wrong sample taken during sampling 取样时样品取错 |  | data error 数据错误 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Wrong label 标签贴错 |  | data error 数据错误 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 005 |  | Department 部门: | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Changing the lubrication oil 更换润滑油 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 005 |  | Change the lubricating oil 更换润滑油 |  | Replace the lubricating oil without stopping the pump 更换润滑油未停泵 | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Waste lubricating oil is dumped at will 废润滑油随意倾倒 | environment pollution 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Wrong lubricating oil or improper oil level 润滑油加错或油位不合适 | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | On-site lubricating oil is not cleaned 现场润滑油未清理 | environment pollution 污染环境 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 006 |  | Department 部门: | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Machine pump inspection and maintenance 机泵检维修 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 006 |  | Machine pump inspection and maintenance 机泵检维修 |  | Interlocking and self operating after pump shutdown 泵停后联锁自启 | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Wrong power failure equipment tag number 停电设备位号错误 | Process accident 工艺事故 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 设备带压或存在有介质 The equipment is under pressure or there is a medium | 人身伤害 Personal injury |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 系统未完全隔离 The system is not completely isolated | 引起安全事故 Cause a safety incident |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 设备带压或存在有介质 The equipment is under pressure or there is a medium | 人身伤害 Personal injury |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Tool slipped 工具滑落 | Object strike 物体打击 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 007 |  | Department 部门: | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Valve well switching of valve 阀门井开关阀门 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 007 |  | Valve well switching of valve 阀门井开关阀门 |  | The manhole cover is open without ventilation 井盖打开未通风 | Poisoning, suffocation 中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | No gas detection analysis was performed 未进行气体检测分析 | Poisoning, suffocation 中毒、窒息 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Guardian is not present 监护人不在现场 | Poisoning, suffocation 中毒、窒息 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | The valve opens too fast 阀门打开速度太快 | Larger leakage due to unqualified inspection and repair parts 检修部位因检修不合格造成更大泄漏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 008 |  | Department 部门: | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | High-altitude inspection 高处巡检 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 008 |  | High-altitude inspection 高处巡检 |  | Fall from height 高处坠落 | Personal injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Lightning strike 雷击 | Personal injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Go up and down the stairs without supporting the handrails 上下楼梯不扶扶手 | Personal injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Fall from height 高处坠落 | Personal injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Safety hazards were not reported in time 安全隐患未及时汇报 | Safety incident property loss 安全事故 财产损失 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5101- 009 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Water supply and firefighting booster pump 给水及消防加压泵 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Device inspection 装置巡检 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 009 |  | Device inspection 装置巡检 |  | Incomplete inspection tools 巡检工器具配戴不齐全 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
|  |  |  |  | Go up and down the stairs without using the handrails 上下楼梯不扶扶手 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Fall from height 高处坠落 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Safety hazards were not reported in time 安全隐患未及时汇报 |  | Safety incident property loss 安全事故 财产损失 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |