|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T6- -2008- -2024 |  | Page 2 of 2 |  |
| Department 部门 |  |  | Utilities department 公用工程部 |  | Name of unit (area) 单元（区域）名称 |  | Rainwater monitoring and accident pool 雨水监控及事故池 |
| Reviewed by 审核人 |  |  | Shishuigen 施水根 |  | Date 日期 |  | Feb 10,2025 2025年2月10日 |
| S/N 序号 | No. of SCL form SCL表编号 |  | Name of equipment/facility 设备设施名称 |  | Inspection item 检查项目 |  |  |
| 1 | 001 |  | Inspection for operation of emergency water transfer pump P01 事故水转输泵运行机泵检查 |  | 1.Whether the motor bearing grease overflows and fill the record电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements电机温度，电流是否符合要求； 3.Whether the motor vibration value meets the requirements电机振动值是否符合要求； 4.Whether the motor noise increases电机杂声是否增大。 5.Pump bearing grease and filling records机泵轴承润滑脂，加注记录； 6.Whether the pump bearing temperature meets the requirements机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements机泵振动值是否符合要求； 8.Whether the pump outlet pressure is normal机泵出口压力是否正常； 9.Check the mechanical seal for leakage机封有无泄漏。 |  |  |
| 2 | 002 |  | 1# rainwater monitoring pool rainwater pump operation inspection P02、P03A~D 1#雨水监控池雨水泵运行机泵检查 |  | 1.Whether the motor bearing grease overflows and the filling record电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements电机温度，电流是否符合要求； 3.Whether the motor vibration value meets the requirements电机振动值是否符合要求； 4.Whether the motor noise increases电机杂声是否增大。 5.Pump bearing grease and filling records机泵轴承润滑脂，加注记录； 6.Whether the pump bearing temperature meets the requirements机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements机泵振动值是否符合要求； 8.Whether the pump outlet pressure is normal机泵出口压力是否正常； 9.Check the mechanical seal for leakage机封有无泄漏。 |  |  |
| 3 | 003 |  | 2# rainwater monitoring pool rainwater pump operation inspection P04A/B、P05A~C 2#雨水监控池雨水泵运行 |  | 1.Whether the motor bearing grease overflows and the filling record电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements电机温度，电流是否符合要求； 3.Whether the motor vibration value meets the requirements电机振动值是否符合要求； 4.Whether the motor noise increases电机杂声是否增大。 5.Pump bearing grease and filling records机泵轴承润滑脂，加注记录； 6.Whether the pump bearing temperature meets the requirements机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements机泵振动值是否符合要求； 8.Whether the pump outlet pressure is normal机泵出口压力是否正常； 9.Check the mechanical seal for leakage机封有无泄漏。 |  |  |
| 4 | 004 |  | Inspection of emergency water lift pump and dirty oil lift pump P06A/B/C、P07A/B 事故水提升泵、污油提升泵运行机泵检查 |  | 1.Check whether the motor bearing grease overflows and fill the record电机轴承润滑脂是否溢出，加注记录 2.Whether the motor temperature and current meet the requirements电机温度，电流是否符合要求； 3.Whether the motor vibration value meets the requirements电机振动值是否符合要求； 4.Whether the motor noise increases电机杂声是否增大。 5.Pump bearing grease and filling records机泵轴承润滑脂，加注记录； 6.Whether the pump bearing temperature meets the requirements机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements机泵振动值是否符合要求； 8.Whether the pump outlet pressure is normal机泵出口压力是否正常； 9.Check the mechanical seal for leakage机封有无泄漏。 |  |  |
| 5 | 005 |  | Operation inspection of artificial lake delivery pump P08A/B/C 人工湖输送泵运行机泵检查 |  | 1.Whether the motor bearing grease overflows and the filling record电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements电机温度，电流是否符合要求； 3.Whether the motor vibration value meets the requirements电机振动值是否符合要求； 4.Whether the motor noise increases电机杂声是否增大。 5.Pump bearing lubricating oil and filling records机泵轴承润滑油，加注记录； 6.Whether the pump bearing temperature meets the requirements机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements机泵振动值是否符合要求； 8.Whether the pump outlet pressure is normal机泵出口压力是否正常； 9.Check the mechanical seal for leakage机封有无泄漏。 |  |  |
| 6 | 006 |  | Rainwater monitoring pool, rainwater accident pool, suction pool and waste oil pool 雨水监控池、雨水事故池、吸水池、污油池 |  | 1.Whether there is leakage around the tank池体四周有无泄漏； 2.Whether the guardrail is firm护栏是否牢固； 3.Whether the lighting lamp is normal照明灯是否正常； 4.Whether there is large floating garbage in the pool池内是否大的漂浮垃圾。 |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。