|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-001 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P01 accident water transfer pump 装置/设备/设施：P01事故水转输泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 001 | P01 Accident water transfer pumpRun motor check P01事故水转输泵运行电机检查 |  | Whether the grease of the motor bearing overflows, fill the record 电机轴承润滑脂是否溢出，加注记录 | Motor bearing burnt 电机轴承烧 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the motor temperature and current meet the requirements 电机温度，电流是否符合要求 | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the motor meets the requirements 电机振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the noise of the motor has increased 电机杂声是否增大 | Motor shaft 电机抱轴 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-002 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P01 accident water transfer pump 装置/设备/设施：P01事故水转输泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 002 | P01 Accident water transfer pumpRunning machine pump inspection P01事故水转输泵运行机泵检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-003 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:02, P03A~DRainwater pump 装置/设备/设施：P02、P03A~D雨水泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 003 | P02、P03A~D Rainwater pump running motor inspection P02、P03A~D雨水泵运行电机检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-004 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:02, P03A~DRainwater pump 装置/设备/设施：P02、P03A~D雨水泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 004 | P02、P03A~D Rainwater pump running machine pump inspection P02、P03A~D雨水泵运行机泵检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-005 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P04A/B, P05A~C Rainwater pump 装置/设备/设施：P04A/B、P05A~C 雨水泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 005 | P04A/B、P05A~C Rainwater pump running motor inspection P04A/B、P05A~C 雨水泵运行电机检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-006 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P04A/B, P05A~C Rainwater pump 装置/设备/设施：P04A/B、P05A~C雨水泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 006 | P04A/B、P05A~C Rainwater pump running machine pump inspection P04A/B、P05A~C 雨水泵运行机泵检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-007 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P06A/B, P07A/B Accident water lift pump, dirty oil lift pump 装置/设备/设施：P06A/B、P07A/B 事故水提升泵、污油提升泵 |  |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 007 | P06A/B、P07A/B Accident water lift pump, dirty oil lift pumpRun motor check P06A/B、P07A/B 事故水提升泵、污油提升泵运行电机检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-008 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P06A/B, P07A/B Accident water lift pump, dirty oil lift pump 装置/设备/设施：P06A/B、P07A/B 事故水提升泵、污油提升泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 008 | P06A/B、P07A/B Accident water lift pump, dirty oil lift pumpRunning machine pump inspection P06A/B、P07A/B事故水提升泵、污油提升泵运行机泵检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-009 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P08A/B/C Artificial lake transfer pump 装置/设备/设施：P08A/B/C 人工湖输送泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 009 | P08A/B/C Inspection of the operation motor of the artificial lake transfer pump P08A/B/C人工湖输送泵运行电机检查 |  | Machine pump bearing grease filling record 机泵轴承润滑脂加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-010 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:P08A/B/C Artificial lake transfer pump 装置/设备/设施：P08A/B/C 人工湖输送泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 010 | P08A/B/CMan-made lake transportation pump running machine pump inspection P08A/B/C人工湖输送泵运行机泵检查 |  | Machine pump bearing lubricant filling record 机泵轴承润滑油加注记录 | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature of the pump bearing meets the requirements 机泵轴承温度是否符合要求 | Bearing damage 轴承损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the pump meets the requirements 机泵振动值是否符合要求 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure of the pump is normal 机泵出口压力是否正常 | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the machine seal is leaking 机封有无泄漏 | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5131-011 |  | Unit:Utilities department 单位：公用工程部 | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis Date：Feb 10,2025 分析日期 : 2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:rainwater monitoring and accident pool 单元（区域）/工艺过程名称：雨水监控及事故池 |  |  | Unit/equipment/facility:Rainwater monitoring pool,rainwaterAccident pool, suction pool,Slop 装置/设备/设施：5131-BA-02/05雨水监控池、5131-BA-07雨水事故池、5131-BA-03/06吸水池、5131-BA-04污油池 |  |  |  |  |  |  |  | Reviewed by: WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 011 | Check whether there is any leakage around the pool body 巡检观察池体四周有无泄漏 |  | No leakage from surrounding walls 四周墙体无泄漏 | Leaking wall 墙体漏水 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  | Whether the guardrail is firm 护栏是否牢固 |  | Strong guardrail 护栏牢固 | The railing is damaged and the operator is dangerous 栏杆损坏操作人员发生危险 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  | Whether the lighting can illuminate normally 照明灯是否能正常照明 |  | The light does not turn on 照明灯不亮 | Personal injury 人身伤害 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  | Whether there are large floating objects in the pool 池内有无大漂浮物 |  | No debris in the pool 池内无杂物 | Damaged pump 机泵损坏 | Occasionally 偶尔 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |