|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-001 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  | Oil collection operation 收油作业 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 001 | Oil collection operation 收油作业 |  | Drown 淹溺 |  |  | Personal injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | The sump level is too high 污油池液位过高 |  |  | Polluted environment 污染环境 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-002 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  | Clear pond 清池 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 002 | Clear pond 清池 |  | Pump down 泵抽空 |  |  | Equipment damage 设备损坏 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Poisoning 中毒 |  |  | Personal injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Fall 跌落 |  |  | Personal injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Heatstroke 中暑 |  |  | Personal injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-003 | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | Sampling 取样 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 003 | Sampling 取样 |  | Sampling bottle is not cleaned 取样瓶未清洗 |  | Analysis error 分析误差 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Sampler cleaning is not in place 取样器清洗不到位 |  | Data error 数据误差 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Wrong sample taken during sampling 取样时样品取错 |  | Data error 数据错误 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | Wrong label 标签贴错 |  | Data error 数据错误 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-004 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | Changing lubricating oil 更换润滑油 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 004 | Changing lubricating oil 更换润滑油 |  | Replace the lubricating oil without stopping the pump 更换润滑油未停泵 |  |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Waste lubricating oil is dumped at will 废润滑油随意倾倒 |  |  | Polluted environment 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Wrong lubricating oil or improper oil level 润滑油加错或油位不合适 |  |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | On-site lubricating oil is not cleaned 现场润滑油未清理 |  |  | Polluted environment 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-005 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | Pump inspection and maintenance 机泵检维修 |  |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 005 | Pump inspection and maintenance 机泵检维修 |  |  | Interlocking and self operating after pump shutdown 泵停后联锁自启 |  |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Wrong power failure equipment tag number 停电设备位号错误 |  |  | Process accident 工艺事故 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 设备带压或存在有介质 The equipment is under pressure or there is a medium |  |  | 人身伤害 Personal injury |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 系统未完全隔离 The system is not completely isolated |  |  | 引起安全事故 Cause a safety incident |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | 设备带压或存在有介质 The equipment is under pressure or there is a medium |  |  | 人身伤害 Personal injury |  | 很少发生 Rarely happens | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Tool slipped 工具滑落 |  |  | Object strike 物体打击 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-006 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  | Device inspection 装置巡检 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 006 | Device inspection 装置巡检 |  | Incomplete inspection tools 巡检工器具配戴不齐全 |  |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Not following the designated route 不按指定路线巡检 |  |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Walk through the field of patrol inspection 巡检走过场 |  |  | Equipment damage or rainwater pollution not found 设备损坏或雨水污染未发现 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Safety hazards were not reported in time 安全隐患未及时汇报 |  |  | Safety incident property loss 安全事故 财产损失 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-007 | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | Opening and closing operation of wall panel valve 壁板阀启闭操作 |  |  | Members of analysis group 分析小组成员: |  |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 007 | Opening and closing operation of wall panel valve 壁板阀启闭操作 |  | The status of wall panel valve is not confirmed in place 壁板阀状态确认不到位 |  |  |  | Equipment damage 设备损坏 |  |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | Poor lubrication of screw rod 丝杆润滑不良 |  |  |  | Equipment damage 设备损坏 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Misoperation of wall valve 壁板阀误操作 |  |  |  | Equipment damage 设备损坏 |  |  | Occasionally 偶尔 | 3 | 3 | 9 | III | Operate carefully and slowly, strictly follow the operating procedures, and assess. 操作细心缓慢、严格操作法操作，考核。 |  |  | 2 | 2 | 4 | I |
|  |  |  | Abnormal limit 限位异常 |  |  |  | Equipment damage 设备损坏 |  |  | Occasionally 偶尔 | 3 | 3 | 9 | III | Before operating the valve, conduct a comprehensive inspection, operate with care and slowly, strictly follow the operation method. 阀门操作前进行全方面检查，操作细心缓慢、严格操作法操作 |  |  | 2 | 2 | 4 | I |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-008 | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | 6#, 7# wall plate valve grille garbage cleaning 6#、7#壁板阀格栅垃圾清理 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 008 | 6#, 7# wall plate valve grille garbage cleaning 6#、7#壁板阀格栅垃圾清理 |  | The operator is not clear about the work risk 作业人作业风险不清楚 |  | Personal injury 人员伤害 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | The guardian is not at the scene 监护人员不在现场 |  | Personal injury 人身伤害 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Personnel fall 人员坠落 |  | Personal injury 人身伤害 |  |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | Place the rubbish at will 垃圾随意放置 |  | Environmental pollution 环境污染 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5131-009 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Rainwater monitoring and accident pool 雨水监控及事故池 |  |  |  |  |  |  | Analysis date：February 10,2025 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  | Rainwater pump insulation test 雨水泵绝缘测试 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 009 | Rainwater pump insulation test 雨水泵绝缘测试 |  | The supervisor is not present, resulting in misoperation 监护人员不在场，造成误操作 |  |  | Electric shock 触电 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Not tested in the correct way 未按正确方法测试 |  |  | The detection data is incorrect, forcibly starting the device, causing damage to the device.检测数据不正确，强制启动设备，造成设备损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  | Failure to confirm with the site during power transmission 送电时未和现场确认好 |  |  | Personal injury人身伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |