|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 13 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Confined space work 受限空间作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Operation plan and risk identification 作业计划与风险辨识 |  | Failure to formulate operation plan in advance 未提前制定作业计划 |  | Risks missed and emergency preparedness insufficient 风险漏判、应急准备不足 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | Fill out the confined space operation plan in advance and clarify the risk control measures 提前填报受限空间作业计划，明确风险控制措施 |  | 1 | 2 | 2 | Ⅰ |
|  |  |  |  | Emergency repairs failed to comply with approval procedures, resulting in inadequate preparation of emergency measures 紧急抢修未履行审批程序导致应急措施准备不充分 |  | Management out of control caused the accident 管理失控引发事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Implement the unplanned fire approval process 执行非计划动火审批流程 2. Formulate an emergency response plan and conduct pre-operation briefing 制定应急处置预案并进行作业前交底 |  | 1 | 4 | 4 | Ⅰ |
| 2 |  | System isolation and replacement 系统隔离及置换 |  | Incomplete isolation (such as failure to remove blind plates) leads to medium intrusion 隔离不彻底（如未盲板抽堵）导致介质窜入 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Use blind plates (not valves) to isolate or port, and the system uses steam or nitrogen to replace the stored toxic and combustible media 使用盲板（非阀门）隔离或端口，系统采用蒸汽或氮气进行置换存放的有毒可燃介质 2. System pressure relief, valves are locked with tags 系统泄压，阀门挂牌上锁 |  | 1 | 4 | 4 | Ⅰ |
| 3 |  | System ventilation and gas detection 系统通风及气体检测 |  | No detection or insufficient detection points 未检测或检测点不足 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Sampling results must be obtained before entering the confined space. The qualified standards are CO < 30ppm, combustible gas < 10% LEL, H2S < 7PPM, benzene < 3mg/m3, oxygen 19.5%-23.5%. Unqualified systems must not be entered without specifying special operation plans and implementing safety measures. 进入受限空间前必须有采样结果，合格标准为CO＜30ppm,可燃气＜10%LEL,H2S＜7PPM,苯＜3mg/m3，氧气19.5%-23.5%,不合格系统未指定特殊作业方案和落实安全措施不得进入 2. System testing should adopt multi-point testing and be representative. 系统检测应该采用多点检测，具有代表性 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | Natural ventilation is ineffective and harmful gases accumulate 自然通风无效有害气体积聚 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Use forced ventilation, and toxic and combustible gases inside the system should be monitored in real time. 采用强制通风，系统内部有毒可燃气体应实时监测 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | Special environments that cannot be ventilated due to spontaneous combustion have not formulated special operation plans 存在自燃等无法通风的特殊环境未制定特殊作业方案 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Operators of unqualified systems in special environments need to prepare special confined space operation plans and emergency plans, and clarify fire and poison prevention measures. 特殊环境不合格系统作业人员需要编制特殊受限空间作业方案及应急预案，明确防火防毒措施 |  | 1 | 4 | 4 | Ⅰ |
| 4 |  | Go through the confined space approval procedures before the operation 作业前办理受限空间审批手续 |  | Failure to obtain a confined space operation permit 未办理受限空间作业许可证 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | In accordance with the company's "Safety Management System", implement the approval system for confined space operations, and prohibit operations without permission 按照公司《安全管理制度》，执行受限空间作业审批制度，未经许可禁止作业 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | The operation area is not isolated or clearly marked 作业区域未隔离或标识不清 |  | Non-operating personnel enter the dangerous area 非作业人员进入危险区域 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Set up cordon isolation warnings and set up manhole locks at the entrance, and set up confined space operation warning signs 设置警戒线隔离警戒并在入口处设置人孔锁闭器隔离，设置受限空间作业警示标识 2. Arrange special personnel for supervision 安排专人监护 |  | 1 | 4 | 4 | Ⅰ |
| 5 |  | Personnel and equipment preparation 人员与装备准备 |  | Not equipped with an isolated air respirator 未配备隔离式空气呼吸器 |  | Suffocation and poisoning 窒息中毒 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Complete isolation and qualified analysis systems need to be equipped with emergency isolation air respirators 隔离彻底和分析合格系统需要配备应急隔离式空气呼吸器 2. Operators of unqualified systems need to prepare special confined space operation plans and emergency plans不合格系统作业人员需要编制特殊受限空间作业方案及应急预案 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | Using non-explosion-proof tools 使用非防爆工具 |  | Fire and explosion 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Unqualified systems need to use copper tools or nitrogen purge protection 不合格系统需要使用铜制工具或氮气吹扫保护 2. Use explosion-proof lamps with safe voltage使用安全电压的防爆灯具 |  | 1 | 4 | 4 | Ⅰ |
| 6 |  | Personnel entry 人员进入 |  | Without continuous monitoring or communication interruption 无连续监护或通讯中断 |  | Suffocation or personal injury 导致窒息、人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Provide safety education and training to operators so that they can master the correct way to wear safety belts and respirators. 对作业人员进行安全教育培训，使其熟练掌握安全带和呼吸器的正确佩戴方法。 2. Before operators enter confined spaces, supervisors shall check whether they are wearing safety protection equipment. 在作业人员进入受限空间前，由监护人员检查其安全防护设备佩戴情况。 3. Clarify the responsibilities of supervisors, prohibit supervisors from leaving their posts, and maintain effective communication between supervisors and operators. 明确监护人员职责，严禁监护人员脱岗，监护人员与作业人员保持有效的沟通联系。 |  | 1 | 4 | 4 | Ⅰ |
| 7 |  | Operation process monitoring 作业过程监控 |  | When working in a confined space, toxic and harmful gases accumulate due to poor ventilation, or are caused by fire operations, etc.; workers are not found and evacuated in time when they are unwell 在受限空间内作业时，因通风不良导致有毒有害气体积聚，或因动火作业等引发；作业人员身体不适未及时发现并撤离 |  | Suffocation, poisoning, fire, and explosion 窒息中毒 火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Maintain good ventilation in confined spaces continuously. Forced ventilation equipment can be used to ensure air circulation. 持续保持受限空间内的通风良好，可采用强制通风设备，确保空气流通。 2. If hot work is required, strictly implement the hot work approval system, implement fire prevention and explosion prevention measures, and equip fire extinguishing equipment. 如需动火作业，严格执行动火作业审批制度，落实防火防爆措施，配备灭火器材。 3. During the operation, regularly test the gas in the confined space, pay close attention to the physical condition of the operators, and immediately stop the operation and evacuate if they feel unwell. 作业过程中，定时对受限空间内的气体进行检测，密切关注作业人员身体状况，如有不适立即停止作业并撤离。 |  | 2 | 4 | 8 | Ⅱ |
| 8 |  | Emergency and rescue 应急与救援 |  | Blind rescue 盲目施救 |  | Casualties increased 伤亡扩大 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Establish a professional rescue team for the company 建立企业专职救援队 2. Employees are familiar with the alarm process and can call the police in time when people are poisoned 员工熟悉报警流程，人员中毒时能够及时报警 |  | 2 | 3 | 6 | Ⅱ |
|  |  |  |  | Escape route blocked 逃生通道被堵塞 |  | Delayed escape 延误逃生 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Tools/pipelines should not be piled up in construction manholes and other passages 施工人孔等通道不得堆放工具/管线 2. Supervise and inspect, and clean up the gas-extinguishing rescue blockage in time 监督检查，发现消气防救援杜塞及时清理 |  | 2 | 3 | 6 | Ⅱ |
| 9 |  | End of operation 作业结束 |  | Failure to conduct a comprehensive inspection of the confined space may result in tools, materials or personnel being left behind; Failure to close the entrance and exit of the confined space in time may result in unauthorized personnel entering by mistake 未对受限空间进行全面检查，可能遗留工具、材料或人员；未及时封闭受限空间出入口，导致无关人员误入 |  | Personal Injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Fill out the tool registration form for people entering and leaving the space. After the operation is completed, conduct a comprehensive inspection of the confined space to confirm that there are no tools, materials, or people left behind. 填写进出人员工具登记表，作业结束后，对受限空间进行全面检查，确认无遗留工具、材料和人员。 2. Close the entrances and exits of the confined space in a timely manner and set up obvious warning signs to prevent unauthorized personnel from entering. 及时封闭受限空间出入口，设置明显的警示标识，防止无关人员进入。 |  | 2 | 3 | 6 | Ⅱ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |