|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 14 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Working at heights 高处作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Operational planning and risk identification 作业计划与风险辨识 |  | Failure to formulate an operation plan in advance 未提前制定作业计划 |  | Risks missed and emergency preparedness inadequate 风险漏判、应急准备不足 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | Submit the height operation plan in advance and clarify the risk control measures 提前填报高处作业计划，明确风险控制措施 |  | 1 | 2 | 2 | Ⅰ |
|  |  |  |  | Failure to comply with the approval procedures for emergency repairs, resulting in inadequate preparation of emergency measures 紧急抢修未履行审批程序导致应急措施准备不充分 |  | Management out of control leads to accidents 管理失控引发事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Implement the approval process for unplanned high-altitude operations; 执行非计划高处审批流程； 2. Develop emergency response plans and conduct pre-operation briefings 制定应急处置预案并进行作业前交底 |  | 1 | 4 | 4 | Ⅰ |
| 2 |  | Approval of high-altitude work permit before operation 作业前高处作业票审批 |  | Failure to obtain a permit for high-altitude operations 未办理高处作业许可证 |  | Management out of control leads to accidents 管理失控导致事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | In accordance with the company's "Safety Management System", implement the high-altitude operation approval system, and prohibit operations without permission 按照公司《安全管理制度》，执行高处作业审批制度，未经许可禁止作业 |  | 1 | 4 | 4 | Ⅰ |
| 3 |  | Worker preparation 作业人员准备 |  | Failure to conduct health checks on operators, and allowing operators with high blood pressure, heart disease, and other conditions that are not suitable for high-altitude operations to work 未对作业人员进行健康检查，患有高血压、心脏病等不适合高处作业的人员进行作业 |  | Sudden illness during operation leading to fall 作业过程中突发疾病导致坠落 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Re-identify risks before work. 作业前重新识别风险。 2. Conduct a comprehensive health check on workers before work, and strictly prohibit workers with diseases that are not suitable for working at heights from participating in the work. 作业前对作业人员进行全面健康检查，严禁患有不适合高处作业疾病的人员参与作业。 3. Workers should truthfully report their health conditions. 作业人员如实申报自身健康状况。 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | Failure to wear protective equipment correctly 未正确穿戴防护用品 |  | Fall, being hit by an object 坠落、物体打击 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Five-point double-hook safety belt (must be hung high and used low) 五点式双钩安全带（必须高挂低用） 2. Anti-slip shoes, safety helmet + chin strap 防滑鞋、安全帽+下颌带 |  | 1 | 4 | 4 | Ⅰ |
| 4 |  | Equipment and tool inspection 设备与工具检查 |  | The climbing equipment used (such as scaffolding, hanging basket, etc.) has structural defects, loose parts, etc., or the safety protection equipment (such as safety belts, safety nets, etc.) is damaged or unqualified 使用的登高设备（如脚手架、吊篮等）存在结构缺陷、部件松动等问题，或者安全防护用品（如安全带、安全网等）损坏、不合格 |  | Cause workers to fall 导致作业人员坠落 |  | Occasionally偶尔发生 |  | 3 | 4 | 12 | Ⅲ | 1. Strictly inspect the climbing equipment and safety protection equipment before operation to ensure that they are intact and meet safety standards. 作业前对登高设备和安全防护用品进行严格检查，确保其完好无损且符合安全标准。 2. Regularly maintain the climbing equipment and conduct a trial run inspection before each use. 对登高设备进行定期维护保养，每次使用前进行试运行检查。 3. Purchase safety protection equipment from regular channels and conduct regular inspections. 从正规渠道采购安全防护用品，并定期进行检测。 |  | 1 | 4 | 4 | Ⅰ |
| 5 |  | Working platform and channel inspection 作业平台与通道检查 |  | Scaffolding is not set up in a standardized manner 脚手架搭设不规范 |  | Platform collapse, fall平台坍塌、坠落 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. The scaffolding is set up by a dedicated person and used in accordance with the acceptance and qualified registration 脚手架由专人搭设，按照验收合格挂牌使用 2. The steel grating platform must be welded and fixed钢格板平台必须焊接固定 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | Edge holes not covered 临边孔洞未覆盖 |  | People/tools fall 人员/工具坠落 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Lay steel plates or guardrails or anti-fall nets 铺设钢板或防护栏或防坠网 2. Set up isolation warning lines to remind 设置隔离警戒线提醒 |  | 1 | 4 | 4 | Ⅰ |
| 6 |  | Work environment assessment 作业环境评估 |  | No effective warning signs were set up at the work site, or the surrounding environment (such as bad weather, strong winds, nearby live objects, etc.) was not assessed and corresponding measures were not taken 作业现场未设置有效的警示标识，或未对周边环境（如恶劣天气、强风、临近带电体等）进行评估和采取相应措施，可能 |  | Workers falling due to misoperation or external factors 导致作业人员误操作或受到外部因素影响而坠落 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Set up obvious warning signs at the work site and prohibit unauthorized personnel from entering. 在作业现场设置明显的警示标识，禁止无关人员进入。 2. Conduct a comprehensive assessment of the work environment before work, and stop work in case of severe weather (level 6 or above strong winds, heavy rain, heavy fog, etc.). 作业前对作业环境进行全面评估，如遇恶劣天气（六级及以上大风、暴雨、大雾等）停止作业。 3. Take effective insulation isolation measures when working near live parts. 对临近带电体作业时，采取有效的绝缘隔离措施。 |  | 1 | 4 | 4 | Ⅰ |
| 7 |  | Working at heights 高处作业过程 |  | Workers fail to wear safety belts properly when working at heights, or walk or jump at heights without protection 作业人员在高处作业时未正确系挂安全带，或在无防护的高处行走、跳跃 |  | Leading to falling accidents 导致坠落事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Provide safety education and training to operators, emphasizing the correct use of safety belts and their importance. 对作业人员进行安全教育培训，强调正确使用安全带的方法和重要性。 2. Set up strong life ropes, safety handrails and other protective facilities in high-altitude working areas. 在高处作业区域设置牢固的生命绳、安全扶手等防护设施。 3. Arrange special personnel for supervision and promptly correct unsafe behaviors of operators. 安排专人监护，及时纠正作业人员的不安全行为。 |  | 1 | 4 | 4 | Ⅰ |
|  |  |  |  | When working at height, improper material transfer methods 在高处作业时，物料传递方法不当 |  | Materials fall and injure people below 导致物料掉落砸伤下方人员 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. During the operation process, the supervisor shall check that it is strictly forbidden to throw materials at high altitudes, and materials shall be transferred using tools such as ropes and hanging baskets. 作业过程监护人检查，严禁在高处抛掷物料，采用绳索、吊篮等工具进行物料传递。 2. Set up a warning area below the material transfer area and prohibit personnel from passing 在物料传递区域下方设置警戒区域，禁止人员通行。 |  | 1 | 3 | 3 | Ⅰ |
| 8 |  | Emergency and rescue 应急与救援 |  | No rescue channel set up 未设置救援通道 |  | Delayed rescue 延误救援 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. At least two independent escape routes are reserved in the operation area作业区域保留至少两条独立逃生路径 2. Check the fire-fighting passages in time in the daily operation area, and stop and clear the blocked passages in time if found日常作业区域对消气防通道及时检查，发现堵塞通道及时制止并疏通 |  | 1 | 3 | 3 | Ⅰ |
|  |  |  |  | Working at heights, high temperatures, and not rotating work 高处作业高温未轮换作业 |  | HHheat stroke 中暑 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Drink more water during the rotation of personnel 人员轮换作业休息期间多饮用水 2. Operators are equipped with heatstroke prevention and cooling medicines 作业人员配备防暑降温药品 |  | 1 | 3 | 3 | Ⅰ |
| 9 |  | End of work 作业结束 |  | Failure to clean up materials and tools at the work site in a timely manner may cause materials to fall and injure people, or improper methods of removing climbing equipment 未及时清理作业现场的物料和工具，可能导致物料掉落伤人，或者拆除登高设备时方法不当 |  | Personal Injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | After the operation is completed, clean up the materials and tools at the work site in a timely manner to ensure that the site is clean after work. 作业结束后，及时清理作业现场的物料和工具，做到工完场清。 |  | 1 | 3 | 3 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |