|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 15 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Groundbreaking 动土作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before operation 作业前准备 |  | Failure to inspect underground pipelines, cables and other facilities may cause damage during construction, resulting in 未对地下管线、电缆等设施进行检查，施工时可能造成损坏，引发 |  | Power outages, water outages, communication interruptions, etc.停电、停水、通信中断等事故 |  | Occasionally 偶尔发生 |  | 3 | 4 | 12 | Ⅲ | 1. Re-identify risks before operation. 作业前重新识别风险。 2. Contact relevant departments before operation to obtain the distribution map of underground pipelines, cables and other facilities. 作业前联系相关部门，获取地下管线、电缆等设施的分布图。 3. Organize construction personnel to conduct safety briefings and clarify the location and protection measures of underground facilities. 组织施工人员进行安全交底，明确地下设施位置及保护措施。 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Approval of groundbreaking permit before operation 作业前动土作业票审批 |  | Failure to obtain groundbreaking permit 未办理动土作业许可证 |  | Power outages, water outages, communication interruptions, etc. Digging up flammable pipelines and causing fires and explosions 停电、停水、通信中断等事故，挖断易燃管线导致火灾爆炸 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Implement the relevant regulations on groundbreaking operations issued by the Mobile Department 执行机动部发布的破土作业相关规定 2. Contact relevant departments before the operation, obtain underground pipeline and cable diagrams, sign the groundbreaking operation ticket, and give on-site instructions to construction personnel 作业前联系相关部门，获取地下管线、电缆图，并在动土作业票会签，对施工人员现场交底 3. For the excavation and leak detection of flammable and explosive pipelines, a special plan must be formulated to clarify the fire and explosion prevention measures during the excavation process针对易燃易爆管线开挖查漏作业要制定专项方案，明确开挖过程防火防爆措施 |  | 1 | 3 | 3 | Ⅰ |
| 3 |  | Excavation 土方开挖 |  | Excavation depth exceeds the specified depth, and slope or support is not provided as required 开挖深度超过规定，未按要求放坡或支护，可能导致 |  | Cause earthwork collapse and bury workers. 土方坍塌，掩埋作业人员 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. According to the soil conditions and excavation depth, the slope or support should be carried out in strict accordance with the specifications. 根据土质情况和开挖深度，严格按照规范要求进行放坡或支护。 2. Regularly inspect the slope and support structure, and deal with abnormalities in a timely manner. 定期对边坡和支护结构进行检查，发现异常及时处理。 3. Limit the loading at the edge of the pit to avoid excessive pressure on the slope. 限制坑边堆载，避免对边坡造成过大压力。 4. When the operation is suspended, the surrounding area of ​​the excavation pit needs to be isolated and guarded to prevent unauthorized personnel from entering by mistake. 作业暂停需要对开挖坑洞周边隔离警戒防止无关人员误入 |  | 1 | 3 | 3 | Ⅰ |
|  |  |  |  | Improper machine operation 机械操作不当 |  | Personnel are hit by the excavator or underground pipelines and cables are cut 人员被挖机碰撞或挖断地下管线电缆 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Set up a warning area and assign a full-time guardian to implement the relevant measures of the groundbreaking work ticket 设置警戒区域现场配备专职监护人落实动土作业票相关措施 2. Manual excavation should be used in areas near pipelines or cables 临近管线或电缆区域应采用人工开挖 |  | 1 | 3 | 3 | Ⅰ |
| 4 |  | Workers were working on the site. 人员动土作业 |  | Workers did not wear personal protective equipment such as helmets and safety shoes. 作业人员未佩戴安全帽、安全鞋等个人防护用品 |  | Injuries caused by being hit by objects, slipping and falling, etc.物体打击、滑倒坠落等受伤 |  | Rarely 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. Provide safety education and training to operators, emphasizing the importance of wearing personal protective equipment correctly. 对作业人员进行安全教育培训，强调正确佩戴个人防护用品的重要性。 2. Set up obvious safety warning signs at the construction site to remind operators to wear protective equipment. 在施工现场设置明显的安全警示标识，提醒作业人员佩戴防护用品。 3. Arrange special personnel to patrol and correct and punish those who fail to wear protective equipment as required. 安排专人巡查，对未按要求佩戴防护用品的人员进行纠正和处罚。 |  | 1 | 3 | 3 | Ⅰ |
| 5 |  | Soil pile transportation operation 土堆运输作业 |  | Moving vehicles were overloaded and speeding 土方运输车辆超载、超速行驶 |  | Accidents such as vehicle loss of control and rollover 车辆失控、翻车等事故 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Check the transport vehicles to ensure that the vehicles are in good performance. Overloading and speeding are strictly prohibited. 对运输车辆进行检查，确保车辆性能良好，严禁超载、超速行驶。 2. Set up speed limit and load limit signs at the construction site and arrange special personnel to direct traffic. 在施工现场设置限速、限载标识，安排专人指挥交通。 3. Regularly conduct safety training for transport vehicle drivers to improve safety awareness. 定期对运输车辆驾驶员进行安全培训，提高安全意识。 |  | 1 | 3 | 3 | Ⅰ |
| 6 |  | Operation completed 作业结束 |  | The excavated area was not backfilled and leveled in time 未对开挖区域进行及时回填和平整 |  | Accidents resulting in people falling or tripping导致人员坠落、绊倒等事故 |  | Rarely 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. After the operation is completed, the excavated area should be backfilled and leveled in time to restore the original state of the ground. 作业结束后，及时对开挖区域进行回填和平整，恢复地面原状。 2. Set up obvious warning signs and protective facilities in the unbackfilled area. 在未回填区域设置明显的警示标识和防护设施。 |  | 1 | 3 | 3 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |