|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 16 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Lifting operation 吊装作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before operation 作业前准备 |  | Failure to conduct a comprehensive inspection of the lifting equipment, such as wear of the hook and broken wire rope 未对吊装设备进行全面检查，如吊钩磨损、钢丝绳断丝 |  | Personal injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1.作业前重新识别风险 2.每次作业前，按照设备检查表对吊装设备进行详细检查，包括吊钩、钢丝绳、制动器等关键部件 3.对吊装设备进行维护保养 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Lifting operation ticket or lifting plan approval 吊装作业票或吊装方案审批 |  | Failure to apply for a lifting operation ticket or prepare a special lifting operation plan 未办理吊装作业票或编制特殊吊装作业专项方案 |  | Lifting out of control 吊装失控 |  | Rarely 很少发生 |  | 3 | 3 | 9 | Ⅲ | 1. Implement the relevant lifting operation system issued by the mobile department 执行机动部发布的有关吊装作业制度 2. Implement operation permit management, and strictly prohibit operations without tickets执行作业许可管理，严禁无票作业 |  | 2 | 2 | 4 | Ⅰ |
| 3 |  | Site survey and layout 场地勘察与布置 |  | The work site is uneven, there are obstacles, affecting the stability of the crane, or no warning area is set, and unauthorized personnel enter the work area 作业场地不平整、有障碍物，影响吊车稳定，或未设置警戒区域，无关人员进入作业区 |  | Personal injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Survey the site before operation, fill in potholes, and remove obstacles; set up obvious warning signs in the operation area, arrange special personnel to supervise, and prohibit unrelated personnel from entering 作业前对场地进行勘察，填平坑洼，清除障碍物；在作业区域设置明显的警戒标识，安排专人监护，禁止无关人员进入 |  | 2 | 2 | 4 | Ⅰ |
| 4 |  | Binding of hoisted objects 吊物捆绑 |  | Improper binding method, unstable center of gravity, looseness of the hoisted object, and fall during hoisting 捆绑方式不当，吊物重心不稳、松动，吊运过程中发生坠落 |  | Mechanical injury 机械伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Choose appropriate binding methods and slings according to the shape and weight of the hoisted object; perform trial lifting after binding to check the stability of the hoisted object 根据吊物形状、重量选择合适的捆绑方式和吊索具；捆绑完成后进行试吊，检查吊物稳定性 |  | 2 | 2 | 4 | Ⅰ |
| 5 |  | Lifting operation 起吊操作 |  | Illegal operation, such as tilted lifting, too fast lifting speed, causing the hoisted object to shake and collide 违规操作，如歪拉斜吊、起吊速度过快，导致吊物晃动、碰撞 |  | Personal injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Lifting drivers and commanders must be certified, strictly abide by operating procedures, lift steadily, avoid tilting, slanting and rapid lifting 吊装司机及指挥需持证上岗，严格遵守操作规程，平稳起吊，避免歪拉斜吊和快速起吊 |  | 2 | 2 | 4 | Ⅰ |
| 6 |  | Lifting process 吊运过程 |  | People staying or passing under the lifting route 吊运路线下方有人员停留或通过 |  | Falling objects injuring people 吊物坠落伤人 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Clearly define the lifting route, set up a warning area below the lifting route, prohibit people from staying and passing; arrange for a dedicated person to command and ensure the safety of the lifting process 明确吊运路线，在吊运路线下方设置警戒区域，禁止人员停留和通过；安排专人指挥，确保吊运过程安全 |  | 2 | 2 | 4 | Ⅰ |
| 7 |  | Unloading operation 卸放作业 |  | The unloading position is not accurate, the hoisting objects are not placed stably, and they may fall over and injure people. 卸放位置不准确，吊物放置不稳，倾倒伤人 |  | Personal injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Determine the exact unloading location before unloading, clean up the surrounding debris; slowly release the hook, and wait until the lifting object is placed steadily before untying it在卸放前确定准确的卸放位置，清理周围杂物；缓慢放钩，待吊物放置平稳后再解除捆绑 |  | 2 | 2 | 4 | Ⅰ |
| 8 |  | Closing after operation作业后收尾 |  | The hoisting equipment is not reset, cleaned, and maintained, and the equipment parts are damaged or lost; the work site is not cleaned up. 未对吊装设备进行复位、清洁和保养，设备部件损坏丢失；未清理作业现场 |  | Causes equipment failure 导致设备故障 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | After the operation is completed, reset the lifting equipment, clean and maintain it; clean up the operation site, and properly store the tools and remaining materials. 作业结束后，将吊装设备复位，进行清洁和保养；清理作业现场，将工具和剩余材料妥善存放 |  | 1 | 3 | 3 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |