|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 17 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Blind plate extraction and plugging operation 盲板抽堵作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before operation 作业前准备 |  | Failure to conduct risk assessment on the operation site and failure to understand the properties of the medium in the pipeline, pressure and other parameters 未对作业现场进行风险评估，未了解管道内介质性质、压力等参数 |  | Leading to leakage, explosion and other accidents during the operation 导致作业过程中发生泄漏、爆炸等事故 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Re-identify risks before operation. 作业前重新识别风险。 2. Collect comprehensive information about pipelines before operation, including parameters such as medium composition, pressure, and temperature. 作业前全面收集管道相关信息，包括介质成分、压力、温度等参数。 3. Conduct a detailed risk assessment of the operation site and formulate targeted operation plans and emergency plans. 对作业现场进行详细的风险评估，制定针对性的作业方案和应急预案。 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Blind plate selection and inspection 盲板选择与检查 |  | The selected blind plate material and size do not meet the requirements, or the blind plate has defects, which may rupture during the plugging process 选用的盲板材质、尺寸不符合要求，或盲板存在缺陷，可能在抽堵过程中发生破裂，导致介质泄漏 |  | Causing medium leakage, fire, explosion, and personal injury 引发火灾、爆炸、人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. According to the medium characteristics and pressure parameters in the pipeline, correctly select the blind plate of appropriate material and size. 根据管道内介质特性和压力等参数，正确选择合适材质和尺寸的盲板。 2. Perform an appearance inspection on the blind plate to ensure that there are no defects such as cracks and sand holes, and perform non-destructive testing when necessary. 对盲板进行外观检查，确保无裂纹、砂眼等缺陷，必要时进行无损检测。 |  | 2 | 2 | 4 | Ⅰ |
| 3 |  | Approval of blind plate plugging operation ticket 盲板抽堵作业票审批 |  | Failure to issue blind plate plugging operation ticket 未开具盲板抽堵作业票 |  | Management out of control leading to accidents 管理失控导致事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | Follow the relevant regulations on blind plate extraction and plugging issued by the company's planning and coordination department. Operations are prohibited without approval of the operation ticket 按照公司计调部发布的有关盲板抽堵作业相关规定执行，未经作业票审批许可禁止作业 |  | 2 | 2 | 4 | Ⅰ |
| 4 |  | On-site safety measures 现场安全措施设置 |  | Failure to set up obvious warning signs at the work site, unauthorized personnel may enter the work area; Failure to effectively isolate the work area 未在作业现场设置明显的警示标识，无关人员可能进入作业区域；未对作业区域进行有效隔离 |  | Personal injury 导致人身伤害 |  | Rarely 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. Set up obvious warning signs around the work site, such as "Blind plate extraction and plugging operation, no entry", etc.在作业现场周围设置明显的警示标识，如“盲板抽堵作业，禁止入内”等。 2. Use warning tapes, fences, etc. to effectively isolate the work area and restrict unauthorized personnel from entering. 使用警戒带、围挡等对作业区域进行有效隔离，限制无关人员进入。 |  | 2 | 2 | 4 | Ⅰ |
| 5 |  | Blockage extraction operation 抽堵作业 |  | When removing and plugging the blind plate, improper operation, such as excessive force and inappropriate tools, may cause pipeline damage, blind plate falling off, and cause medium leakage; Failure to remove and plug the blind plate in the prescribed order 抽堵盲板时，由于操作不当，如用力过猛、工具不合适等，可能导致管道受损、盲板脱落，引发介质泄漏；未按规定顺序抽堵盲板 |  | Cause fire, explosion, poisoning, personal injury 引发火灾、爆炸、中毒、人身伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Provide professional training to operators so that they can master the correct operation methods and skills of blind plate extraction and plugging. 对作业人员进行专业培训，使其掌握正确的抽堵盲板操作方法和技巧。 2. Select appropriate extraction and plugging tools and ensure that the tools are intact. 选择合适的抽堵工具，并确保工具完好无损 3. Strictly follow the pre-established extraction and plugging sequence to perform the operation, and pay close attention to the changes in system pressure during the operation. 严格按照预先制定的抽堵顺序进行作业，作业过程中密切关注系统压力变化。 |  | 2 | 2 | 4 | Ⅰ |
| 6 |  | Post-operation inspection 作业后检查 |  | After the operation, the blind plate installation is not checked, which may cause the blind plate to be installed loosely and leak in the subsequent production process; the work site is not cleaned up, and the tools and debris left behind 作业结束后，未对盲板安装情况进行检查，可能导致盲板安装不牢固，在后续生产过程中发生泄漏；未清理作业现场，遗留的工具、杂物 |  | Catch fire and cause personal injury. 着火、人身伤害 |  | Rarely 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. After the operation is completed, conduct a comprehensive inspection of the installation position and sealing of the blind plate to ensure that the blind plate is installed correctly and firmly.作业结束后，对盲板的安装位置、密封情况等进行全面检查，确保盲板安装正确、牢固 2. Clean up the operation site in time, clean up the tools and debris, and restore the site to its original state. 及时清理作业现场，将工具、杂物等清理干净，恢复现场原状。 |  | 2 | 2 | 4 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |