|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 18 |  | Department 部门: |  | Utilities Department 公用工程部 |  |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Blind plate extraction and plugging operation 盲板抽堵作业 |  |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before operation 作业前准备 |  | Failure to issue a road-breaking notice in advance, and passing vehicles and pedestrians were unaware; Failure to prepare warning signs and protective facilities 未提前发布断路通知，过往车辆和行人不知情；未准备好警示标识和防护设施 |  |  | Causing collision accidents 引发碰撞事故 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Issue a circuit breaker notice in advance, specify the time, location and detour route of the circuit breaker. 提前发布断路通知，明确断路时间、地点和绕行路线。 2. Prepare sufficient and standard warning signs (such as warning lights, warning signs, roadblocks, etc.) and protective facilities according to operational requirements. 根据作业需求，准备充足且符合标准的警示标识（如警示灯、警示标志、路障等）和防护设施。 |  | 1 | 3 | 3 | Ⅰ |
| 2 |  | Application and approval of road-breaking operations 断路作业申请与审批 |  | Failure to apply for road-breaking operations according to the prescribed procedures, and privately carrying out road-breaking 未按规定流程申请断路作业，私自进行断路 |  |  | Causing traffic chaos, affecting normal traffic order, and even causing traffic accidents 导致交通混乱，影响正常通行秩序，甚至引发交通事故 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | In accordance with the company's "Safety Management System", the approval system for high-altitude operations is implemented, and operations without permission are prohibited 按照公司《安全管理制度》，执行高处作业审批制度，未经许可禁止作业 |  | 1 | 3 | 3 | Ⅰ |
| 3 |  | Warning signs and protective facilities are set up on site 现场设置警示标识和防护设施 |  | Warning signs are improperly set up, insufficient in number or damaged, and cannot effectively warn passing vehicles and pedestrians; protective facilities are not firmly built and cannot play a protective role 警示标识设置位置不当、数量不足或损坏，不能有效警示过往车辆和行人；防护设施搭建不牢固，起不到防护作用 |  |  | Leading to traffic accidents 导致交通事故 |  | Rarely occurs 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. According to the road conditions and the scope of operation, warning signs should be reasonably set up in accordance with traffic regulations to ensure that they are eye-catching and complete. 依据道路状况和作业范围，按照交通法规要求合理设置警示标识，确保醒目、完整。 2. Protective facilities should be stably built, regularly inspected, and damaged parts should be repaired or replaced in a timely manner. 对防护设施进行稳固搭建，并定期检查，及时修复或更换损坏部件。 |  | 1 | 3 | 3 | Ⅰ |
| 4 |  | Traffic diversion 交通疏导 |  | Insufficient traffic diversion personnel or improper diversion methods lead to traffic congestion and long-term vehicle detention 交通疏导人员不足或疏导方法不当，导致交通拥堵，车辆长时间滞留 |  |  | Causing vehicle scratches, collisions and other accidents 引发车辆刮擦、碰撞等事故 |  | Rarely occurs 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Reasonably arrange a sufficient number of traffic diversion personnel according to the traffic flow. 根据交通流量合理安排足够数量的交通疏导人员。 2. Provide professional training for traffic diversion personnel so that they can master the correct diversion methods and traffic command gestures. 对交通疏导人员进行专业培训，使其掌握正确的疏导方法和交通指挥手势。 |  | 2 | 2 | 4 | Ⅰ |
| 5 |  | Road-breaking construction 断路施工 |  | No effective protective measures were taken during the construction process, construction equipment and materials were placed in a messy manner; construction workers violated regulations 施工过程中未采取有效防护措施，施工设备、材料摆放杂乱；施工人员违规操作 |  |  | Leading to collisions and damage to passing vehicles and pedestrians 导致过往车辆和行人碰撞受损 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Use fences to effectively isolate the construction area, reasonably place construction equipment and materials, and set up obvious construction area signs.用围挡等对施工区域进行有效隔离，合理摆放施工设备和材料，设置明显的施工区域标识。 2. Provide safety education and operating procedures training for construction personnel, and arrange special personnel to supervise the construction process. 对施工人员进行安全教育和操作规程培训，安排专人监督施工过程。 |  | 2 | 2 | 4 | Ⅰ |
| 6 |  | End of operation 作业结束 |  | Failure to clean up the site in time, leaving construction equipment and materials behind; failure to restore traffic signs and facilities in time未及时清理现场，遗留施工设备、材料等；未及时恢复交通标志和设施 |  |  | Causing vehicle collision or person tripping, traffic accident 导致车辆碰撞或人员绊倒，交通事故 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Immediately organize personnel to clean up the site after the operation to ensure that there are no obstacles on the road. 作业结束后立即组织人员清理现场，确保道路无障碍物 2. Restore traffic signs and facilities according to the original standards and conduct inspections and confirmations. 按照原有标准恢复交通标志和设施，并进行检查确认。 |  | 2 | 2 | 4 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |