|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 19 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Radiation work 射线作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before operation 作业前准备 |  | Failure to conduct a comprehensive inspection of the radiation equipment may result in equipment failure, leading to abnormal radiation emission 未对射线设备进行全面检查，设备可能存在故障，导致射线异常发射 |  | Radiation damage to personnel 人员造成辐射伤害 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Before operation, conduct a comprehensive inspection of the radiation equipment according to the equipment operating procedures, including radiation sources, control systems, safety protection devices, etc. 作业前，按照设备操作规程对射线设备进行全面检查，包括射线源、控制系统、安全防护装置等。 2. Regularly perform maintenance and performance testing of radiation equipment to ensure that the equipment is in good operating condition. 定期对射线设备进行维护保养和性能检测，确保设备处于良好运行状态。 3. Select radiation flaw detection contractors that meet the company's safety requirements and performance requirements 选用符合公司安全要求和业绩要求的射线探伤承包商 |  | 1 | 4 | 4 | Ⅰ |
| 2 |  | Approval and publication of radiographic work permits 射线作业票审批及公布 |  | Failure to issue radiographic flaw detection work permits 未开具射线探伤作业票 |  | Radiation damage caused to personnel 人员造成辐射伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. In accordance with the company's "Safety Management System", implement the radiation operation approval system, and all affected departments will sign and prohibit operations without permission 按照公司《安全管理制度》，执行射线作业审批制度，各受到影响的部门进行会签，未经许可禁止作业 2. Release radiation flaw detection operation information in advance according to company requirements 按照公司要求提前发布射线探伤作业信息 |  | 1 | 4 | 4 | Ⅰ |
| 3 |  | Work site layout 作业现场布置 |  | Failure to set up effective warning areas and warning signs at the work site may lead to unauthorized personnel entering the work area by mistake 未在作业现场设置有效的警示区域和警示标识，无关人员可能误入作业区域 |  | Causing radiation damage 造成辐射伤害 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Set up obvious warning signs around the work site, such as "Beware of ionizing radiation", and use warning tape or fences to demarcate the warning area. 在作业现场周围设置明显的警示标识，如“当心电离辐射”等，并使用警戒带或围栏划定警示区域。 2. Arrange special personnel to guard the entrance of the warning area and prohibit unauthorized personnel from entering. 安排专人在警示区域入口处值守，禁止无关人员进入。 |  | 1 | 4 | 4 | Ⅰ |
| 4 |  | Personnel protection人员防护 |  | The operator did not wear personal protective equipment correctly, such as personal dosimeters, protective lead clothing, etc. 作业人员未正确佩戴个人防护用品，如个人剂量计、防护铅衣等 |  | Excessive radiation exposure 导致身体受到过量辐射 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Train operators to master the correct wearing methods and usage precautions of personal protective equipment. 对作业人员进行培训，使其掌握个人防护用品的正确佩戴方法和使用注意事项。 2. Regularly check the performance and integrity of personal protective equipment, and replace damaged or expired protective equipment in a timely manner. 定期检查个人防护用品的性能和完整性，及时更换损坏或过期的防护用品。 |  | 1 | 4 | 4 | Ⅰ |
| 5 |  | Radiation work 射线作业 |  | During the radiation operation, the operator violated the regulations, such as arbitrarily changing the radiation parameters and turning on the radiation source in an unsafe state在射线作业过程中，操作人员违规操作，如随意更改射线参数、在非安全状态下开启射线源 |  | Cause radiation accidents 导致辐射事故 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Select a radiographic flaw detection contractor that meets the company's safety requirements and performance requirements 选用符合公司安全要求和业绩要求的射线探伤承包商 2. Set up safe isolation warnings and guardians in the flaw detection area to prevent unauthorized personnel from entering by mistake 探伤区域设置安全的隔离警戒并设置监护人，防止无关人员误入 |  | 1 | 4 | 4 | Ⅰ |
| 6 |  | Post-processing 作业后处理 |  | After the operation, the radiation equipment was not properly stored and turned off, and the radiation source may be accidentally exposed; the dose of the operators was not tested and recorded. 作业结束后，未对射线设备进行妥善保管和关闭，射线源可能意外暴露；未对作业人员进行剂量检测和记录 |  | Radiation caused by personnel 人员造成辐射 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. After the operation is completed, turn off the radiation equipment in strict accordance with the operating procedures, and properly store the radiation source in a dedicated storage container. 作业结束后，严格按照操作规程关闭射线设备，并将射线源妥善保管在专用的储存容器中。 2. Conduct personal dose tests on operators, record the test results in detail, and establish personal dose files. 对作业人员进行个人剂量检测，并详细记录检测结果，建立个人剂量档案。 |  | 1 | 4 | 4 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |