|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Equipment and Facilities List 设备设施清单模板 |  |  |
|  | Template No. | HYBN-T7-08-2008-2024-2 | Page 1 of 2 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6- -2008- -2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  | Utilities 公用工程部 |  |  | Name of unit (area) 单元（区域）名称 |  |  | Installation area refrigeration station装置区制冷站 |
| Reviewed by 审核人 |  |  | Shishuigen 施水根 |  |  | Date 日期 |  |  | 2025年2月10日 |
| S/N 序号 | No. of SCL form SCL表编号 |  |  | Name of equipment/facility 设备设施名称 |  |  | Inspection item 检查项目 |  |  |
|  | 001 |  |  | 5603-P001A/B/, P002A/B Chilled water pump running machine pump inspection 5603-P001A/B/、P002A/B 冷冻水泵运行机泵检查 |  |  | 1.Whether the motor bearing grease overflows, fill in the record;电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements;电机温度，电流是否符合要求； 3.Whether the vibration value of the motor meets the requirements;电机振动值是否符合要求； 4.Whether the noise of the motor has increased.电机杂声是否增大 5.Whether the lubricating grease of the machine pump bearing overflows, fill the record;机泵轴承润滑脂是否溢出，加注记录； 6.Whether the temperature of the pump bearing meets the requirements;机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements;机泵振动值是否符合要求； 8.Whether the outlet pressure of the pump is normal;机泵出口压力是否正常； 9.Whether the machine seal is leaking机封有无泄漏 |  |  |
|  | 002 |  |  | - WCH-001A/B, 002A/B - Lithium bromide refrigeration unit operation inspectionWCH-001A/B、002A/B 溴化锂制冷机组运行检查 |  |  | 1.Whether the cold water temperature of the lithium bromide unit is normal;溴化锂机组冷水温度是否正常； 2.Whether the temperature of the hot water of the lithium bromide unit is normal;溴化锂机组热水温度是否正常； 3.Whether the vacuum degree of the lithium bromide unit is normal;溴化锂机组真空度是否正常； 4.Whether the circulating water, chilled water and hot water flow of the lithium bromide unit is normal;溴化锂机组循环水、冷冻水、热水流量是否正常； 5.Whether the circulating water, chilled water and hot water pressure of the lithium bromide unit is normal;溴化锂机组循环水、冷冻水、热水压力是否正常； 6. Whether the opening of the capacity valve of the lithium bromide unit is normal.溴化锂机组容量阀开度是否正常。 |  |  |
|  | 003 |  |  | 5603-PA-001 Constant pressure water supplement device 5603-PA-001 定压补水装置 |  |  | 1.Whether the liquid level of the water tank of the constant pressure water replenishment device is normal;定压补水装置水箱液位是否正常； 2.Whether the pressure setting of the constant pressure water supplement device is normal;定压补水装置压力设置是否正常； 3.Whether the safety accessories of the constant pressure water supplement device meet the requirements;定压补水装置安全附件是否符合要求； 4.Whether the water quality of the constant pressure water replenishment device meets the requirements.定压补水装置补水水质是否符合要求。 |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。