|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 21 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Equipment inspection and maintenance work 设备检维修作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Liang Bin,Yu Yapeng,Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun, Pan Wentao, Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Energy isolation before operation 作业前能量隔离 |  | Failure to shut down, power off, and display warning signs may result in accidental equipment startup 未停机、断电并挂警示标识，可能导致设备意外启动 |  | Causing casualties 造成人员伤亡 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Re-identify risks before operation 作业前重新识别风险 2. Strictly implement shutdown, power off, and tag-out procedures, with dedicated personnel responsible for confirmation. 严格执行停机、断电、挂牌程序，由专人负责确认。 3. Develop and post standard operating procedures and strengthen employee training. 制定并张贴标准操作流程，加强员工培训。 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Process treatment before operation 作业前工艺处理 |  | Failure to completely replace residual materials 未彻底置换残余物料 |  | Fire poisoning 火灾中毒 |  | Rarely 很少发生 |  | 2 | 5 | 10 | Ⅲ | The blind plate is confirmed at level 3 and the replacement is qualified. The unit should arrange personnel to confirm on-site that the conditions for safe delivery and maintenance are met. 盲板三级确认，分析置换合格，所在单位要安排人员现场确认达到安全交付检修条件 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Inspection of maintenance equipment and facilities检修设备设施检查 |  | Failure to conduct a comprehensive inspection of the equipment for maintenance may result in missing potential equipment failures and accidents during maintenance. 未对检维修设备进行全面检查，可能遗漏设备潜在故障，维修时发生意外 |  | Personal Injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | Check the equipment according to the professional division of labor 按照专业分工对设备进行排查 |  | 2 | 2 | 4 | Ⅰ |
| 3 |  | Issue inspection and maintenance work tickets 开具检维修作业票 |  | Maintenance work ticket not approved 检修作业票未审批 |  | Violation of laws and regulations 违反法律法规 |  | Unlikely to happen不太可能发生 |  | 1 | 5 | 5 | Ⅱ | Carry out maintenance work in accordance with the regulations issued by the Ministry of Transportation. At the same time, strictly implement the operation permit management. It is strictly forbidden to operate without a ticket. The unit where you are located supervises and inspects to stop violations in a timely manner. 按照机动部发布的有关检修作业规定执行，同时严格执行作业许可管理，严禁无票作业，所在单位监督检查及时制止违章 |  | 1 | 2 | 2 | Ⅰ |
| 4 |  | Disassembly work 拆卸作业 |  | Improper use of disassembly tools or incorrect operation methods 拆卸工具使用不当或操作方法错误 |  | Personal Injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | Dedicated personnel direct dismantling operations according to professional division of labor 按照专业分工专人指挥拆卸作业 |  | 2 | 2 | 4 | Ⅰ |
| 5 |  | High-altitude work 高处作业 |  | Not wearing safety belts or failing to build stable scaffolds when performing inspection and maintenance work at heights 在高处进行检维修作业时，未系安全带或未搭建稳固的脚手架 |  | Causing people to fall 导致人员坠落 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. At the same time, a high-altitude work ticket is issued, qualified safety belts are equipped for workers, and they are supervised to wear them correctly. 同时开具高处作业票，为作业人员配备合格的安全带，并监督其正确佩戴。 2. Professionals build scaffolds and conduct acceptance inspections. 由专业人员搭建脚手架，并进行验收。 |  | 2 | 2 | 4 | Ⅰ |
| 6 |  | Lifting work吊装作业 |  | Falling of suspended objects or toppling of cranes 吊物坠落或吊车倾倒 |  | Injure people due to falling objects 坠物伤人 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | At the same time, a lifting operation ticket is issued, an isolation warning area and professional supervision are set up, the lifting driver and commander are qualified to operate, and the lifting tools are inspected in advance. 同时开具吊装作业票，设置隔离警戒区域和专业监护，吊装司机和指挥符合相关资质作业，吊装工具进行提前的安全检查 |  | 2 | 2 | 4 | Ⅰ |
| 7 |  | Hoisting work 动火作业 |  | Failure to detect combustible gases before starting a fire, or the lack of fire-fighting equipment at the site of the fire 动火前未进行可燃气体检测，或动火现场未配备灭火器材 |  | Cause fire, explosion 引发火灾、爆炸 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. At the same time, a hot work ticket is issued, and a combustible gas test is carried out within 30 minutes before the start of the fire. The fire can only be started after passing the test. 同时开具动火作业票，动火前30分钟内进行可燃气体检测，合格后方可动火。 2. A sufficient number of suitable fire extinguishing equipment are equipped at the hot work site. 在动火现场配备足够数量且适用的灭火器材。 |  | 2 | 2 | 4 | Ⅰ |
| 8 |  | Confined space work 受限空间作业 |  | Failure to detect and ventilate confined spaces 未对受限空间进行气体检测和通风 |  | Causing poisoning, suffocation导致人员中毒、窒息 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. At the same time, a confined space operation ticket is issued, and a comprehensive gas test is conducted on the confined space before the operation, including oxygen content, toxic and harmful gas concentration, etc. 同时开具受限空间作业票，作业前对受限空间进行全面气体检测，包括氧气含量、有毒有害气体浓度等。 2. Continuous ventilation is carried out to ensure air circulation in the confined space. 持续通风，确保受限空间内空气流通。 |  | 2 | 2 | 4 | Ⅰ |
| 9 |  | Electrical work 电气作业 |  | Electrical workers not holding certificates or not taking insulation protection measures 电气作业人员未持证上岗，或未采取绝缘防护措施 |  | Electric shock accidents 触电事故 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. At the same time, temporary electricity operation tickets are issued to ensure that electrical operators are certified and conduct regular qualification reviews. 同时卡具临时用电作业票，确保电气作业人员持证上岗，定期进行资格审查。 2. Equip operators with qualified insulation protection equipment and supervise their use. 为作业人员配备合格的绝缘防护用品，并监督使用。 |  | 2 | 2 | 4 | Ⅰ |
| 10 |  | Parts installation 零部件安装 |  | Parts not properly debugged during installation 安装过程中未对零部件进行正确调试 |  | Unstable equipment operation 设备运行不稳定 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Installation and commissioning are carried out by dedicated personnel according to professional division of labor. 安装调试按照专业分工专人进行。 2. After installation, comprehensive commissioning and trial operation are carried out, and the unit is delivered after acceptance. 安装完成后进行全面调试和试运行，验收合格后交付所在单位。 |  | 2 | 2 | 4 | Ⅰ |
| 11 |  | Post-work cleanup 作业后清理 |  | Failure to clean up the site after work, leaving tools and debris behind 作业结束后未清理现场，遗留工具、杂物等 |  | Personal Injury 人身伤害 |  | Rarely 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Establish a site cleaning system and clarify the cleaning standards and responsible persons. 制定现场清理制度，明确清理标准和责任人。 2. Conduct an on-site inspection after the operation is completed to confirm that there are no leftover items. 作业结束后进行现场检查，确认无遗留物品。 |  | 2 | 2 | 4 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |