|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 22 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Wastewater treatment plant污水处理场 ASU & ACU空分空压 |  |  |  |  |  | Analysis date： MM DD, YYYY 分析日期：2025年3月25日 |  |  |  |  |
| Task 工作任务: |  |  | Scaffolding erection and dismantling operations 脚手架搭设与拆除作业 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, LiangBin, Yu Yapeng, Li Guangyuan, Lv Yingqiang, Deng Wentao，Zhang Yunbo，Yin Zujun,Pan Wentao,Jiang Ruibin, Dou Jianqiang, Zhao Xueren, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、邓文涛、张云波、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Preparation before installation搭设前准备 |  | The site was not cleaned and leveled, and the ground was not solid 未对搭设场地进行清理和平整，地面不坚实 |  | The scaffolding is unstable and collapses, causing personal injury. 脚手架整体失稳坍塌，人身伤害 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Before setting up, thoroughly clean the site and remove debris and obstacles. 搭设前对场地进行全面清理，清除杂物和障碍物。 2. Compact and level the ground, and lay pads or set up bases if necessary. 对地面进行夯实和平整处理，必要时铺设垫板或设置底座。 |  | 2 | 2 | 4 | Ⅰ |
| 2 |  | Material inspection 材料检查 |  | The scaffolding materials used (such as steel pipes, fasteners, scaffolding boards, etc.) had quality defects 使用的脚手架材料（如钢管、扣件、脚手板等）存在质量缺陷 |  | Personal injury, collapse accident人身伤害、坍塌事故 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. Strictly inspect the incoming scaffolding materials to check for defects in appearance and test the tightening torque of fasteners.对进场的脚手架材料进行严格检查，查看外观有无缺陷，对扣件进行拧紧力矩测试。 2. Refuse to use unqualified materials and regularly maintain and inspect the materials.拒绝使用不合格材料，定期对材料进行维护保养和检测。 |  | 2 | 2 | 4 | Ⅰ |
| 3 |  | Personnel qualifications and training 人员资质与培训 |  | If the erection personnel do not obtain the corresponding qualification certificates, or have not received professional training, and the operation is not standardized, it may cause the scaffolding to fail to meet safety requirements and increase the risk of collapse. 搭设人员未取得相应资质证书，或未接受专业培训，操作不规范，可能导致脚手架搭设不符合安全要求，增加坍塌风险 |  | Personal injury 人身伤害 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | 1. The erection personnel have the scaffolding operation qualification. 搭设人员具有脚手架作业资质。 2. The construction unit organizes the erection personnel to receive safety training and skill assessment to improve their professional level. 施工单位组织搭设人员进行安全培训和技能考核，提升其专业水平。 |  | 2 | 2 | 4 | Ⅰ |
| 4 |  | Scaffolding dismantling and erection related work tickets or plan approval 脚手架拆搭相关作业票或方案审批 |  | Working without a work ticket 未开具作业票作业 |  | Violation of laws and regulations 违法法律法规 |  | Unlikely to happen 不太可能发生 |  | 1 | 5 | 5 | Ⅱ | 1. Implement the company's work permit management, and the unit where you are located will inspect the construction unit and stop any violations in time 执行公司作业许可管理，所在单位对施工单位进行检查，发现违章及时制止 2. Special scaffolding requires a special plan and verification and approval 特殊脚手架需要编制专项方案并进行验证审批 |  | 1 | 4 | 4 | Ⅰ |
| 5 |  | Scaffolding erection 脚手架搭设 |  | During the erection process, reinforcement measures such as wall ties and scissor braces were not set up according to the specifications, or the spacing between the vertical poles and the horizontal poles was too large 搭设过程中未按规范要求设置连墙件、剪刀撑等加固措施，或立杆间距、横杆步距过大 |  | Leading to collapse accidents and personal injury 导致坍塌事故，人身伤害 |  | Rarely occurs 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Select contractors with relevant qualifications for scaffolding dismantling and installation and whose safety performance meets the company's requirements. 选用具有脚手架拆搭相关资质和安全业绩符合公司要求的承包商。 2. Appoint a dedicated person to supervise during the erection process and correct violations in a timely manner. 在搭设过程中设置专人监督，及时纠正违规行为。 3. Personnel with scaffolding acceptance qualifications shall conduct acceptance and put the scaffolding into use 由具有脚手架验收资质的人员进行验收，挂牌使用 |  | 2 | 2 | 4 | Ⅰ |
| 6 |  | Working at heights 高处作业 |  | Construction workers fail to wear safety belts properly when working at heights, or walk on unprotected scaffolding 搭设人员在高处作业时未正确佩戴安全带，或在无防护的脚手板上行走 |  | Lead to falling accidents 导致坠落事故 |  | Rarely occurs 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Provide safety education to the erection personnel, emphasizing the correct wearing and use of safety belts. 对搭设人员进行安全教育，强调安全带的正确佩戴和使用方法。 2. Install strong safety ropes and guardrails on the scaffolding to ensure the safety of workers. 在脚手架上设置牢固的安全绳和防护栏杆，确保作业人员行走安全。 |  | 2 | 2 | 4 | Ⅰ |
| 7 |  | Preparation before demolition 拆除前准备 |  | Failure to conduct a comprehensive inspection of the scaffolding, develop a demolition plan, or provide safety briefings to demolition personnel may lead to accidents during the demolition process 未对脚手架进行全面检查，未制定拆除方案，或未对拆除人员进行安全交底，可能导致拆除过程中发生意外 |  | Leading to poisoning and suffocation of personnel 导致人员中毒、窒息 |  | Rarely occurs 很少发生 |  | 2 | 4 | 8 | Ⅱ | Select contractors with relevant qualifications for scaffolding erection and dismantling and whose safety performance meets the company's requirements. Conduct a comprehensive inspection of the scaffolding before dismantling to assess its stability and safety. 选用具有脚手架拆搭相关资质和安全业绩符合公司要求的承包商，拆除前对脚手架进行全面检查，评估其稳定性和安全性。 |  | 2 | 2 | 4 | Ⅰ |
| 8 |  | Demolition work 拆除作业 |  | Wrong dismantling sequence, such as dismantling the wall connection first and then the rods; throwing the rods randomly during the dismantling process 拆除顺序错误，如先拆除连墙件再拆除杆件；拆除过程中随意抛掷杆件 |  | Leading to partial instability and collapse of the scaffolding, personal injury 导致脚手架局部失稳坍塌，人身伤害 |  | Rarely occurs 很少发生 |  | 2 | 5 | 10 | Ⅲ | 1. Strictly follow the principle of erecting first and then dismantling, and dismantling the last erection first, and dismantle the scaffolding in order. 严格按照先搭后拆、后搭先拆的原则，按顺序拆除脚手架。 2. Throwing rods is strictly prohibited, and the demolition work should be carried out by passing or lifting. 严禁抛掷杆件，采用传递或吊运的方式进行拆除作业。 3. Set up a warning area in the demolition area, arrange special personnel for supervision, and prohibit unauthorized personnel from entering. 在拆除区域设置警戒区域，安排专人监护，禁止无关人员进入。 |  | 2 | 2 | 4 | Ⅰ |
| 9 |  | Cleaning up after demolition 拆除后清理 |  | Failure to clean up the scaffolding materials on site in time after demolition may cause people to trip or materials to be lost; failure to restore the site after demolition 拆除后未及时清理现场的脚手架材料，可能导致人员绊倒或材料丢失；未对拆除后的场地进行恢复 |  | Personal injury 人身伤害 |  | Rarely occurs 很少发生 |  | 2 | 3 | 6 | Ⅱ | 1. Clean up the site in time after demolition, classify and store the scaffolding materials, and keep them properly. 拆除后及时清理现场，将脚手架材料分类存放，妥善保管。 2. Level and restore the site after demolition to meet the requirements of subsequent construction. 对拆除后的场地进行平整和恢复，满足后续施工要求。 |  | 1 | 2 | 2 | Ⅰ |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |