|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6- -2008- -2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  | Utilities Departmen 公用工程部 |  |  | Name of unit (area) 单元（区域）名称 |  |  | 2nd Circulating Water 二循环水场 |
| Reviewed by 审核人 |  |  | Shi Shuigen 施水根 |  |  | Date 日期 |  |  | 02 10, 2025 2025年2月10日 |
| S/N 序号 | No. of SCL form SCL表编号 |  |  | Name of equipment/facility 设备设施名称 |  |  | Inspection item 检查项目 |  |  |
|  | 001 |  |  | P201A/B/C、P202 Circulating water pump running motor P201A/B/C、P202 循环水泵运行电机 |  |  | 1.Whether the motor bearing grease overflows, fill in the record; 1.电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements; 2.电机温度，电流是否符合要求；. 3.Whether the vibration value of the motor meets the requirements; 3.电机振动值是否符合要求； 4.Whether the noise of the motor has increased. 4.电机杂声是否增大。 |  |  |
|  | 002 |  |  | P201A/B/C、P202 Circulating water pump running machine pump P201A/B/C、P202 循环水泵运行机泵 |  |  | 1.Whether the lubricating grease of the machine pump bearing overflows, fill the record; 1.机泵轴承润滑脂是否溢出，加注记录； 2.Whether the temperature of the pump bearing meets the requirements; 2.机泵轴承温度是否符合要求； 3.Whether the vibration value of the pump meets the requirements; 3.机泵振动值是否符合要求； 4.Whether the outlet pressure of the pump is normal; 4.机泵出口压力是否正常； 5.Whether the machine seal is leaking. 5.机封有无泄漏。 |  |  |
|  | 003 |  |  | 5111-CWT201A/B Cooling tower 5111-CWT201A/B 冷却塔 |  |  | 1.Whether the air duct is firm and damaged; 1.风筒是否牢固、有无破损情况； 2.Check whether the water distribution is even; 2.检查布水是否均匀； 3.Check whether the water collector, nozzle and packing are damaged. 3.检查收水器、喷头、填料有无破损情况。 |  |  |
|  | 004 |  |  | 5111-FAN201A/B Fan 5111-FAN201A/B 风机 |  |  | 1.Whether the motor bearing grease overflows, fill in the record; 1.电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current are normal; 2.电机温度、电流是否正常； 3.Whether the vibration value of the motor meets the requirements; 3.电机振动值是否符合要求； 4.Whether the noise of the motor increases; 4.电机杂声是否增大； 5.Whether the oil level of the gearbox is normal; 5.减速箱油位是否正常； 6.Whether the vibration of the reducer meets the requirements. 6.减速机振动是否符合要求。 |  |  |
|  | 005 |  |  | SR201A/B/C Bypass filter SR201A/B/C 旁滤器 |  |  | 1.Confirm whether the status of each pneumatic valve is normal; 1.确认各气动阀门的状态是否正常； 2.Whether the pressure difference between the inlet and outlet of the side filter is normal; 2.旁滤器进出口压差是否正常； 3.Whether the on-site PLC control panel display and operation are normal; 3.现场PLC控制面板显示、操作是否正常； 4.Confirm whether there is "running dripping". 4.确认有无“跑冒滴漏”。 |  |  |
|  | 006 |  |  | Dosing Room 加药间 |  |  | - Whether the doors and windows are 1.门窗有无损坏、关不上情况； 2.Whether the exhaust fan can work normally; 2.排风扇是否能正常工作； 3.Whether the water quality and water pressure of the eyewash are normal; 3.洗眼器水质、水压否正常； 4.Whether the medicine stack meets the requirements; 4.药剂堆放是否符合要求； 5.Whether the liquid level of the medicament storage tank and the medicament pump are operating normally; 5.药剂储罐液位、药剂泵运行是否正常； 6.Whether the safety signs are complete. 6.安全标识是否齐全。 |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。