|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job List 作业活动清单 |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T6-**-2001-***-2024 |  |  | Page 2 of 2 |
| Department 部门 |  |  | Utilities Department 公用工程部 |  |  | Unit (Area) 单元（区域） | Refrigeration station in front of the plant 厂前区制冷站 |
| Identified by 辨识人 |  |  | Wenjiancheng、Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 温建成、张浩然、于亚鹏、邓文涛、张云波、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  | Date 日期 | 02 10,2025 2025年2月10日 |
| S/N 序号 | Main job 主要作业活动 |  |  |  | Main procedures of job 作业活动主要步骤 |  |  |
| 001 | Lithium Bromide Refrigerator Switch 溴化锂制冷机切换 |  |  |  | 1.Check before starting the standby lithium bromide refrigerator备用溴化锂制冷机启动前检查 |  |  |
|  |  |  |  |  | 2.Switch to standby, stop lithium bromide chiller chilled water切换备用、停用溴化锂制冷机冷冻水 |  |  |
|  |  |  |  |  | 3.Open the front valve of the hot water regulating valve of the standby lithium bromide refrigerator打开备用溴化锂制冷机热水调节阀前阀 |  |  |
|  |  |  |  |  | 4.Start standby Lithium Bromide Refrigerator启动备用溴化锂制冷机 |  |  |
|  |  |  |  |  | 5.Stop the planned lithium bromide refrigerator停止计划停止的溴化锂制冷机 |  |  |
|  |  |  |  |  | 6.Close the front valve of the hot water regulating valve of the lithium bromide refrigerator that is out of service关闭停运溴化锂制冷机热水调节阀前阀 |  |  |
| 002 | Pump rotate 机泵盘车 |  |  |  | 1.Contact between panel and external operator内外操联系 |  |  |
|  |  |  |  |  | 2.Manually switch切手动 |  |  |
|  |  |  |  |  | 3.Determine the operating status确定运行状态 |  |  |
|  |  |  |  |  | 4.Turn 2-3 times盘车2-3圈 |  |  |
| 003 | Switch cold water pump 切换冷水泵 |  |  |  | 1.Check and confirm检查确认 |  |  |
|  |  |  |  |  | 2.Start the standby pump启备用泵 |  |  |
|  |  |  |  |  | 3.Switch pump切泵 |  |  |
|  |  |  |  |  | 4.Stop the original operation pump停原运行泵 |  |  |
| 004 | Replace the lubricating oil of the vacuum pump and cold water pump 更换真空泵、冷水泵润滑油 |  |  |  | 1.Manually after stopping the pump停泵后打手动 |  |  |
|  |  |  |  |  | 2.Vent the original lubricating oil放空原润滑油 |  |  |
|  |  |  |  |  | 3.Add new lubricating oil加注新润滑油 |  |  |
|  |  |  |  |  | 4.Resume and clean up the site恢复清理现场 |  |  |

Descriptions: 1. List major jobs for each process unit (area) which is considered as the assessment unit.

说明：1以工艺单元（区域）为评价单元，列出单元主要作业活动。

2 Simple jobs need to be more than 4 steps, while complex jobs need to be more than 8 steps.

简单作业活动要求分为4步以上，复杂作业活动要求分为8步以上。

3 This form is used for JHA risk assessment and shall be consistent with the No. in JHA form.

此表用于JHA法风险评价，JHA表编号要对应起来。