|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6- -2008- -2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  |  | Utilities Department 公用工程部 |  |  | Name of unit (area) 单元（区域）名称 |  | Refrigeration station in front of the plant 厂前区制冷站 |
| Reviewed by 审核人 |  |  |  | Shi Shuigen 施水根 |  |  | Date 日期 |  | 02 10, 2025 2025年2月10日 |
| S/N 序号 | No. of SCL form SCL表编号 |  | Name of equipment/facility 设备设施名称 |  |  | Inspection item 检查项目 |  |  |  |
|  | 001 |  | 5602-P001A/B/C/D Chilled water pump running motor inspection 5602-P001A/B/C/D 冷冻水泵运行电机检查 |  |  | 1.Whether the motor bearing grease overflows, fill in the record.电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements.电机温度，电流是否符合要求； 3.Whether the vibration value of the motor meets the requirements.电机振动值是否符合要求； 4.Whether the noise of the motor has increased.电机杂声是否增大。 |  |  |  |
|  | 002 |  | 5602-P001A/B/C/D Chilled water pump running machine pump inspection 5602-P001A/B/C/D 冷冻水泵运行机泵检查 |  |  | 1.Whether the lubricating grease of the machine pump bearing overflows, fill the record.机泵轴承润滑脂是否溢出，加注记录； 2.Whether the temperature of the pump bearing meets the requirements.机泵轴承温度是否符合要求； 3.Whether the vibration value of the pump meets the requirements.机泵振动值是否符合要求； 4.Whether the outlet pressure of the pump is normal.机泵出口压力是否正常； 5.Whether the machine seal is leaking.机封有无泄漏。 |  |  |  |
|  | 003 |  | 5602-WCH-001A/B/C/D Lithium bromide refrigeration unit operation inspection 5602-WCH-001A/B/C/D溴化锂制冷机组运行检查 |  |  | 1.Whether the cold water temperature of the lithium bromide unit is normal.溴化锂机组冷水温度是否正常； 2.Whether the temperature of the hot water of the lithium bromide unit is normal.溴化锂机组热水温度是否正常； 3.Whether the vacuum degree of the lithium bromide unit is normal.溴化锂机组真空度是否正常； 4.Whether the circulating water, chilled water and hot water flow of the lithium bromide unit is normal.溴化锂机组循环水、冷冻水、热水流量是否正常； 5.Whether the circulating water, chilled water and hot water pressure of the lithium bromide unit is normal.溴化锂机组循环水、冷冻水、热水压力是否正常； 6.Whether the opening of the capacity valve of the lithium bromide unit is normal.溴化锂机组容量阀开度是否正常。 |  |  |  |
|  | 004 |  | 5602-PA-001 Constant pressure water supplement device 5602-PA-001 定压补水装置 |  |  | 1.Whether the liquid level of the water tank of the constant pressure water replenishment device is normal.定压补水装置水箱液位是否正常； 2.Whether the pressure setting of the constant pressure water supplement device is normal.定压补水装置压力设置是否正常； 3.Whether the safety accessories of the constant pressure water supplement device meet the requirements.定压补水装置安全附件是否符合要求； 4.Whether the water quality of the constant pressure water replenishment device meets the requirements.定压补水装置补水水质是否符合要求。 |  |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。