|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5602-001 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Refrigeration station in front of the plant 厂前区制冷站 |  |  |  |  |  | Analysis date： 02 10, 2024 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Lithium Bromide Refrigerator Switch 溴化锂制冷机切换 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wen Jiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Check before starting the standby lithium bromide refrigerator 备用溴化锂制冷机启动前检查 |  | Inspection is not in place 检查不到位 |  | Lithium bromide refrigerator does not start 溴化锂制冷机未启动 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Switch to standby, stop lithium bromide chiller chilled water 切换备用、停用溴化锂制冷机冷冻水 |  | Chilled water network fluctuations 冷冻水管网波动 |  | The cooling effect in the building is not good 建构筑物内制冷效果不佳 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Open the front valve of the hot water regulating valve of the standby lithium bromide refrigerator 打开备用溴化锂制冷机热水调节阀前阀 |  | Irregular use of "F" wrench “F”扳手使用不规范 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Start standby Lithium Bromide Refrigerator 启动备用溴化锂制冷机 |  | Turn on the lithium bromide refrigerator by mistake 误将溴化锂制冷机启错 |  | High concentration of lithium bromide solution 溴化锂溶液浓度高 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 5 |  | Stop the planned lithium bromide refrigerator 停止计划停止的溴化锂制冷机 |  | Stop the lithium bromide refrigerator by mistake 误将溴化锂制冷机停错 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 6 |  | Close the front valve of the hot water regulating valve of the lithium bromide refrigerator that is out of service 关闭停运溴化锂制冷机热水调节阀前阀 |  | Irregular use of "F" wrench “F”扳手使用不规范 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5602-002 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Refrigeration station in front of the plant 厂前区制冷站 |  |  |  |  |  | Analysis date： 02 10, 2024 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Pump rotate 机泵盘车 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wen Jiancheng 温建成 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |  |
| 1 |  | Contact between panel and external operator 内外操联系 |  | Panel and field operator are not contacted, the panel operator manually start the equipment 内外操未联系，内操手动启动设备 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | Manually switch 切手动 |  | The site did not put "Manual" or "Stop" 现场未打到“手动”或“停止” |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | Determine the operating status 确定运行状态 |  | The device is running 设备处于运行状态 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Turn 2-3 times 盘车2-3圈 |  | Failure to rotate 2-3 round as required 未按规定盘车2-3圈 |  | Equipment damage 设备损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5602-003 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Refrigeration station in front of the plant 厂前区制冷站 |  |  |  |  |  | Analysis date： 02 10, 2024 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Switch cold water pump 切换冷水泵 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wen Jiancheng 温建成 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |  |
| 1 |  | Check and confirm 检查确认 |  | Unchecked to confirm the status of the standby pump 未检查确认备用泵状态 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | Start the standby pump 启备用泵 |  | When the pump is started, the turning part is encountered 启泵时碰到转到部件 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 3 |  | Switch pump 切泵 |  | When the pump is started, the turning part is encountered 切泵时机泵憋压 |  | Pump damage 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Stop the original operation pump 停原运行泵 |  | The flow of the pump is interrupted or fluctuates greatly when the original operation is stopped 停原运行泵流量中断或波动较大 |  | Affect the stable operation of subsequent devices 影响后续装置稳定运行 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5602-004 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Refrigeration station in front of the plant 厂前区制冷站 |  |  |  |  |  | Analysis date： 02 10, 2024 分析日期：2025年2月10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Replace the lubricating oil of the vacuum pump and cold water pump 更换真空泵、冷水泵润滑油 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wen Jiancheng 温建成 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Manually after stopping the pump 停泵后打手动 |  | Replace the lubricating oil without stopping the pump 更换润滑油未停泵 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 2 |  | Vent the original lubricating oil 放空原润滑油 |  | Waste lubricating oil is dumped at will 废润滑油随意倾倒 |  | polluted environment 污染环境 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 3 |  | Add new lubricating oil 加注新润滑油 |  | Wrong lubricating oil or improper oil level 润滑油加错或油位不合适 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Resume and clean up the site 恢复清理现场 |  | On-site lubricating oil is not cleaned 现场润滑油未清理 |  | polluted environment 污染环境 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |