|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：001 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility:Installation/equipment/facilities: sewage lift pump 装置/设备/设施：污水提升泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before pump operation ### 机泵投用前条件确认 |  | The installation is completed and the commissioning is qualified ### 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout ### 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally ### 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage ### 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Inspection of pump operation status 机泵运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements ### 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements 出口压力符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal ### 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Pump shutdown 机泵停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Pump power failure 机泵断电 |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. - 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：002 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility:Installation/equipment/facilities: sludge lift pump 装置/设备/设施：污泥提升泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before pump operation 机泵投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal ### 机封损坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout ### 轴承磨损、烧坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally ### 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking ### 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Inspection of pump operation status 机泵运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements ### 出口压力符合要求 |  | Equipment damage ### 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements ### 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Pump shutdown 机泵停运 |  | Mechanical damage 机械伤害 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Pump power failure 机泵断电 |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly ### 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：003 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility:sludge and scum lifting pump 装置/设备/设施：油泥浮渣提升泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before pump operation 机泵投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Inspection of pump operation status 机泵运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements 出口压力符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Pump shutdown 机泵停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | 机泵断电 Pump power failure |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：004 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: waste oil lift pump 装置/设备/设施：污油提升泵 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before pump operation 机泵投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Inspection of pump operation status 机泵运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements 出口压力符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Pump shutdown 机泵停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | 机泵断电 Pump power failure |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：005 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: biochemical blower 装置/设备/设施：生化鼓风机 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before fan operation 风机投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Fan operation status inspection 风机运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements 出口压力符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The mechanical seal water is put into use, and the leakage meets the requirements 机封水投用，泄漏量符合要求 |  | Damaged mechanical seal 机封损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Fan shutdown 风机停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | 风机断电 Fan power failure |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：006 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: centrifugal dehydrator 装置/设备/设施：离心脱水机 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before operation of centrifugal dehydrator ### 离心脱水机投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements ### 润滑油质量、油位符合要求 |  | Bearing wear and burnout ### 轴承磨损、烧坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally ### 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage ### 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of centrifugal dehydrator 离心脱水机运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns ### 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens ### 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the torque meets the requirements 转矩是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Shutdown of centrifugal dehydrator 离心脱水机停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Power failure of centrifugal dehydrator 离心脱水机断电 |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：007 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: oil-water separator 装置/设备/设施：油水分离器 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before operation of oil-water separator 油水分离器投用前条件确认 |  | Relevant valves and instrument accessories of oil-water separator are intact and put into normal operation 油水分离器相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Connect relevant processes of oil-water separator 油水分离器相关流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens ### 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | The pressure test of oil-water separator is qualified ### 油水分离器试压合格 |  | Equipment damage and accident 损坏设备、引起事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Normal operation of safety valve 安全阀正常投用 |  | Equipment overpressure, Safety valve take-off 设备超压，安全阀起跳 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Relevant valves and instrument accessories of oil-water separator are intact and put into normal operation 油水分离器相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of oil-water separator 油水分离器运行状态检查 |  | Check whether the oil-water separator and auxiliary pipeline have leakage 油水分离器及附属管道有无泄漏 |  | environmental pollution 环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | All instrument parameters are displayed in the normal range ### 各仪表参数显示在正常范围 |  | Overpressure damages equipment and causes process accidents 超压损坏设备、引起工艺事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation自动控制及联锁投用 |  | Process fluctuation causes process accidents 工艺波动，引起工艺事故 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | The safety valve operates normally 安全阀运行正常 |  | Equipment overpressure, safety valve takes off 设备超压，安全阀起跳 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | Stop the oil-water separator and complete the pressure relief and emptying operations 停运油水分离器，并完成泄压、排空操作 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Effective system isolation 系统有效隔离 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：008 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: cavitation air flotation device 装置/设备/设施：涡凹气浮装置 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before operation of cavitation air flotation device 涡凹气浮装置投用前条件确认 |  | The relevant valves and instrument accessories of the cavitation air flotation device are intact and put into normal operation ### 涡凹气浮装置相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Get through relevant processes of cavitation air flotation device 涡凹气浮装置相关流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Apply grease evenly to the drive chain 传动链条均匀涂抹润滑脂 |  | Chain rust and wear链条锈蚀磨损 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of cavitation air flotation device 涡凹气浮装置运行状态检查 |  | Check whether there is leakage of vortex concave air flotation device and auxiliary pipeline 涡凹气浮装置及附属管道有无泄漏 |  | Environmental pollution 环境污染 | Rarely happens 很少发生 | 1 | 4 | 4 | II |  |  | 1 | 2 | 2 | I |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Occasionally 偶尔 | 2 | 4 | 8 | II |  |  | 1 | 2 | 2 | I |
|  |  |  | Whether the temperature, rotation and operation sound of rotating equipment are normal 转动设备温度、转动、运行声音是否正常 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | The chain breaking alarm operates normally 断链预报警运行正常 |  | Chain break 链条断裂 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | Stop the cavitation air flotation device and complete the emptying operation 停运涡凹气浮装置，并完成排空操作 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | System effective isolation / flushing 系统有效隔离/冲洗 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：009 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: dissolved air flotation unit 装置/设备/设施：溶气气浮装置 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before operation of dissolved air flotation unit 溶气气浮装置投用前条件确认 |  | Relevant valves and instrument accessories of dissolved air flotation device are intact and put into normal operation 溶气气浮装置相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Get through relevant processes of dissolved air flotation device 溶气气浮装置相关流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Apply grease evenly to the drive chain 传动链条均匀涂抹润滑脂 |  | Chain rust and wear链条锈蚀磨损 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of dissolved air flotation device 溶气气浮装置运行状态检查 |  | Check whether there is leakage of dissolved air flotation device and auxiliary pipeline 溶气气浮装置及附属管道有无泄漏 |  | Environmental pollution 环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Automatic control and interlocking operation 自动控制及联锁投用 |  | Process fluctuations, causing process accidents 工艺波动，引起工艺事故 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the temperature, rotation and sound of rotating equipment are normal 转动设备温度、转动、声音是否正常 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | The chain breaking alarm operates normally 断链预报警运行正常 |  | Chain break 链条断裂 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the operation of safety valve meets the requirements 安全阀运行是否符合要求 |  | Tank burst 罐爆裂 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | Stop the dissolved air flotation unit and complete the emptying operation 停运溶气气浮装置，并完成排空操作 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | System effective isolation / flushing 系统有效隔离/冲洗 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：010 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: plant air buffer tank 装置/设备/设施：工厂风缓冲罐 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before putting plant air buffer tank into operation 工厂风缓冲罐投用前条件确认 |  | Relevant valves and instrument accessories of plant air buffer tank are intact and put into normal use 工厂风缓冲罐相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Get through relevant processes of plant air buffer tank 工厂风缓冲罐相关流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of plant air buffer tank 工厂风缓冲罐运行状态检查 |  | All instrument parameters are displayed in the normal range 各仪表参数显示在正常范围 |  | Overpressure damages equipment and causes process accidents 超压损坏设备、引起工艺事故 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | Whether the operation of safety valve meets the requirements 安全阀运行是否符合要求 |  | Tank burst 罐爆裂 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | Shut down the plant air buffer tank and complete the emptying operation 停运工厂风缓冲罐并完成排空操作 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | System effective isolation / flushing 系统有效隔离/冲洗 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：011 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: instrument air buffer tank 装置/设备/设施：仪表风缓冲罐 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before putting instrument air buffer tank into operation 仪表风缓冲罐投用前条件确认 |  | Relevant valves and instrument accessories of instrument air buffer tank are intact and put into normal use 仪表风缓冲罐相关阀门、仪表附件完好并正常投用 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Get through relevant processes of instrument air buffer tank 仪表风缓冲罐相关流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 | Inspection for operation status of instrument air buffer tank 仪表风缓冲罐运行状态检查 |  | All instrument parameters are displayed in the normal range 各仪表参数显示在正常范围 |  | Overpressure damages equipment and causes process accidents 超压损坏设备、引起工艺事故 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | Whether the operation of safety valve meets the requirements 安全阀运行是否符合要求 |  | Tank burst 罐爆裂 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | Stop the instrument air buffer tank and complete the emptying operation 停运仪表风缓冲罐并完成排空操作 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | System effective isolation / replacement 系统有效隔离/置换 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：012 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: exhaust fan 装置/设备/设施：废气风机 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before fan operation 风机投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Inlet and outlet valve status is correct 进出口阀门状态正确 |  | Overload, burning motor 超负荷、烧电机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the belt tightness meets the requirements 皮带松紧是否符合要求 |  | Damaged belt 皮带损坏 | happen once in a while 偶尔发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the pump are intact and put into use normally 机泵安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | 风机运行状态检查 Fan operation status inspection |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The outlet pressure meets the requirements 出口压力符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Fan shutdown 风机停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Fan power failure 风机断电 |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：013 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: mud scraper 装置/设备/设施：刮泥机 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before mud scraper is put into use 乱泥投用前条件确认 |  | The installation is completed and the commissioning is qualified 安装完成，调试合格 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the chain tightness meets the requirement 链条松紧是否符合要求 |  | The chain is damaged, the motor is damaged 链条损坏、电机损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Lubricating oil quality and oil level meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety accessories of the scraper are intact and put into use normally 刮泥机安全附件是否完好并正常投用 |  | Equipment damage, mechanical damage 设备损坏、机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | No jamming or abnormal noise during cranking 盘车无卡涩、无异响 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Operation status inspection of mud scraper 刮泥机运行状态检查 |  | The current does not exceed the rated value 电流不超额定值 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Temperature, vibration and noise meet the requirements 温度、振动、杂音符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Torque meets requirements 扭矩符合要求 |  | Equipment damage 设备损坏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The quality and oil level of lubricating oil meet the requirements 润滑油质量、油位符合要求 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 | Maintenance inspection 维修检查 |  | Shutdown of mud scraper 刮泥机停运 |  | Mechanical damage 机械伤害 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Mud scraper power failure 刮泥机断电 |  | Mechanical damage, electric shock 机械伤害、触电 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause a safety accident 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Qualified maintenance quality 维修质量合格 |  | The equipment is not operating properly 设备运行不正常 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：014 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members SCL分析小组人员： 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit(area)/process flow:sewage treatment plant 单元（区域）/工艺过程名称： 污水处理场 |  |  |  | Unit/equipment/facility: storage tank 装置/设备/设施：储罐 |  |  |  |  |  |  |  | Reviewed by:WangLiang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Confirmation of conditions before storage tank operation 储罐投用前条件确认 |  | Relevant valves and instrument accessories of the storage tank are intact and put into normal use 储罐相关阀门、仪表附件完好并正常投用 |  | Equipment damage Cause process accidents 设备损坏 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Get through relevant process flow of storage tank 储罐相关工艺流程打通 |  | Cause process accidents 引起工艺事故 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Relevant safety accessories of storage tank (breathing valve and emergency relief manhole) are intact and put into use 储罐相关安全附件（呼吸阀、紧急泄放人孔）完好并投用 |  | Overpressure Equipment damage 超压 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Inspection of storage tank operation status 储罐运行状态检查 |  | Whether the tank and auxiliary pipelines are leaking 罐体及附属管道有无泄漏 |  | Leakage and environmental pollution 泄漏环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Relevant automatic control and interlocking of storage tank are put into operation 储罐相关自动控制及联锁投用 |  | Process fluctuation causes process accidents 工艺波动，引起工艺事故 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | The liquid level of the storage tank is normal 储罐液位正常 |  | Pouring tank environmental pollution 冒罐 环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 | Confirmation of operation status of safety accessories 安全附件运行状态确认 |  | The breathing valve operates smoothly in exhalation and inhalation 呼吸阀呼出和吸入运行畅通 |  | The equipment is damaged due to overpressure or negative pressure 设备因超压或负压损坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | The opening and closing of emergency relief manhole shall be free of jamming 紧急泄放人孔打开和关闭无卡涩 |  | It cannot be opened automatically under overpressure, and the storage tank bursts 超压状态下无法自动打开， 储罐爆裂 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 | Maintenance inspection 维修检查 |  | The storage tank is drained and replaced to be qualified 储罐完成排空并置换合格 |  | ncrease the safety risk of inspection and maintenance operation 增加检维修作业安全风险 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Effective system isolation 系统有效隔离 |  | Personnel poisoning / suffocation 人员中毒/窒息 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Maintenance safety precautions and emergency measures are in place 维修安全防范及应急措施到位 |  | Cause safety accidents 造成安全事故 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |