|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-01 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Single unit test run 单机试运 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Power transmission 送电 |  | Wrong equipment tag number of power transmission ticket 送电票设备位号写错 |  |  | electric shock 触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | When power is transmitted betwen cabines, it is sent to other moving equipment 机柜间送电时送到其他动设备上 |  |  | electric shock 触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 2 |  | Pre startup inspection 启动前检查 |  | Pump self start 机泵自启动 |  |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  | Motor leakage 电机漏电 |  |  | electric shock 触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 3 |  | normal operation 正常运行 |  | Exposd rotating parts 转动部位外露 |  |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Shut down 停机 |  | The pump is not in "stop" status 机泵未打“停止”状态 |  |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-02 |  | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: |  |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | samplig 采样 | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Open the sampling valve and wait for the sampling pipeline to flow for 1-2 minutes 打开取样阀，待取样管线流通1-2分钟 |  |  | No drainage ditch 没有排放地沟 |  | environmental pollution 环境污染 | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |  |
| 2 |  | Rinse the sampling bottle 2 to 3 times 取样瓶润洗2至3次 |  |  | Randm pouring of rinsing and washig sampls 润洗样随意乱倒 |  | environmental pollution 环境污染 | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |  |
| 3 |  | Stand upwind and take samples 站在上风向处取样 |  |  | Hydrogen sulfide leakage at the lower air outlet of the samplig station 采样站在下风口硫化氢泄漏 |  | Personnel poisoning 人员中毒 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Label the sample bottle 贴好样瓶标签 |  |  | Wrong label 贴错标签 |  | Data distortion, affecting production 数据失真，影响生产 | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. |  |  |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |  |  |  |
| No. 编号: |  |  | 5152-03 | Department 部门: |  |  |  | Utilities Department 公用工程部 |  |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |  |  |  |  |  |  |
| Task 工作任务: |  |  |  | Sewage accident tank pouring 污水事故罐倒罐 |  |  |  | Members of analysis group 分析小组成员: |  |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |  |  |  |  |
| S/N 序号 |  |  | Operation steps 工作步骤 |  |  |  | Hazard source or potential incident 危险源或潜在事件 |  |  |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  |  |  | Residual risk assessment 剩余风险评价 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | L |  | S |  | R | Level等级 |  |  |  |  |  |  | L |  | S |  | R |  | Level等级 |  |
| 1 |  |  | Inspection and confirmation of accident tank liquid level 事故罐液位检查确认 |  |  |  | Harmful gas Fall from height 有害气体 高处坠落 |  |  |  | Personnel injury 人员伤害 |  |  | Rarely happens 很少发生 |  | 2 |  | 2 |  | 4 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | Inspection of tank pouring process 倒罐流程检查 |  |  |  | Sewage hydrogn sulfide overflw 污水硫化氢溢出 |  |  |  | Personnel poisoning 人员中毒 |  |  | Rarely happens 很少发生 |  | 2 |  | 4 |  | 8 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | Start the emergency water pump and adjust the tank pouring volume 启动事故水泵，调整倒罐量 |  |  |  | The pump empties without looking at it 泵抽空，不打量 |  |  |  | The liquid level in the accident tank continues to rise 事故罐液位持续上涨 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | Liquid level confirmation of regulating tank 调节罐液位确认 |  |  |  | Overflw of regulating tank 调节罐溢流 |  |  |  | environmental pollution 环境污染 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |  |  |  |  |
| No. 编号: | 5152-04 |  |  | Department 部门: |  | Utilities Department 公用工程部 |  |  | Unit 装置: |  |  |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |  |  |  |  |
| Task 工作任务: |  |  |  | Crank/ Turning gear盘车 |  | Members of analysis group 分析小组成员: |  |  |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： |  | Wang Liang 王亮 |  |  |  |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  |  |  |  |  |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  |  |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | L |  | S |  | R | Level等级 |  |  |  |  |  | L | S |  | R |  | Level等级 |  |  |
| 1 |  | Contact the internal operator to cut off the pump interlock 联系内操切除机泵联锁 |  |  | The internal and external operation contact is out of control, and the interlocking removal is wrong 内外操联系失控，联锁切除错误 |  |  |  |  |  |  | Mechanical injury 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | Field pump operation column marking "manual" 现场机泵操作柱打“手动” |  |  | Mark "automatic" on the operatin column 操作柱上打“自动” |  |  |  |  |  |  | \Mechanical injury 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | Turn 2-3 turns in the running direction 按运转方向盘车2-3圈 |  |  | Pump start during turning 盘车过程中泵启动 |  |  |  |  |  |  | Mechanical injury 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | Contact the internal operator to put the pump into operation for interlocking 联系内操投运机泵联锁 |  |  | The internal and external operatin contact is out of control, and the interlocking forgets to put into use 内外操联系失控，联锁忘记投用 |  |  |  |  |  |  | Process accident 工艺事故 |  |  | Rarely happens 很少发生 |  | 2 |  | 3 |  | 6 | II |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5152-05 | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |  |
| Task 工作任务: |  | Oil collectin 收油 | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |  |
| 1 | Liquid level confirmation of waste oil intermediate tank 污油中间罐液位确认 |  |  | Level gauge misalignment 液位计失准 |  | environmental pollution 环境污染 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |  |  |
| 2 | Slowly open the oil collecting valve 缓慢打开收油阀门 |  |  | The opening of oil collecting valve is too large 收油阀门开度太大 |  | environmental pollution 环境污染 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |  |
| 3 | Pay close attention to the liquid level of waste oil intermediate tank 密切关注污油中间罐液位 |  |  | The internal operator forgot to check the liquid level in time 内操忘记及时查看液位 |  | environmental pollution 环境污染 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |  |
|  |  |  |  | P3002 is not put into automatic operatin P3002未投入自动运行 |  | environmental pollution 环境污染 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |  |  |
|  |  |  |  | External operators leave the site 外操离开现场 |  | environmental pollution 环境污染 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |  |
| 4 | Close the oil collecting valve 关闭收油阀门 |  |  | Valve not closed 阀门未关死 |  | environmental pollution 环境污染 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5152-06 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Dewatering of waste oil tank 污油罐脱水 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |  |
| 1 |  | Wear hydrogen sulfide alarm 佩戴硫化氢报警仪 |  | The alarm is not worn or turned on 报警仪未佩戴或未打开 |  |  | Personnel poisoning 人员中毒 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | One person operates and one person monitors 一人操作，一人监护 |  | The guardian is not present 监护人不在现场 |  |  | Personnel poisoning 人员中毒 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | Stand at the upper air outlet for operation 站在上风口操作 |  | No wind direction 未看风向 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Slightly open the dehydration valve and slowly dehydrate 微开脱水阀门，缓慢脱水 |  | The valve opening is too large 阀门开度太大 |  |  | Personnel poisoning 人员中毒 |  | Occasionally 偶尔 | 3 | 3 | 9 | III | 1. Focus on the operation status of the on-site alarm device; 关注现场就地报警仪运行情况； 2.Carry the four-in-one alarm device with you at all times; 随身携带四合一报警仪； 3.Dehydration operations strictly prohibit single-person operation;脱水操作严禁单人操作； |  | 2 | 3 | 6 | II |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5152-07 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |  |
| Task 工作任务: |  |  | Sludge dischare of regulatig tank 调节罐排泥 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |  |
| 1 |  | Liquid level confirmation of sludge concentration tank 油泥浓缩罐液位确认 |  | Indication error of liquid level gauge of sludge concentration tank 油泥浓缩罐液位计指示错误 |  | environmental pollution 环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | Confirmation of sludge discharge process from sewage regulating tank to sludge concentration tank 污水调节罐排泥至油泥浓缩罐流程确认 |  | The pneumatic valve for sludge discharge is not opened or the opening of the hand valve is too small 排泥气动阀门未开或手阀开度太小 |  | Pump damage 机泵损坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 3 |  | Start the pump to discharge sludge 启泵排泥 |  | Sludge concentration tank 油泥浓缩罐冒罐 |  | environmental pollution 环境污染 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Stop pump 停泵 |  | Close the valve before stopping the pump 先关阀门再停泵 |  | Pump damage 机泵损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |  |
| No. 编号: | 5152-08 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Dewatering of sewage tank 污水罐脱水 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |  |
| 1 |  | Wear hydrogen sulfide alarm 佩戴硫化氢报警仪 |  | The alarm is not worn or turned on 报警仪未佩戴或未打开 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | One person operates and one person monitors 一人操作，一人监护 |  | The guardian is not present 监护人不在现场 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | Stand at the upper air outlet for operation 站在上风口操作 |  | No wind direction 未看风向 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Slightly open the dehydration valve and slowly dehydrate 微开脱水阀门，缓慢脱水 |  | The valve opening is too large 阀门开度太大 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-09 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | inducing low pressure steam 引低压蒸汽 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Valve inspection at battery limit of unit 装置界区阀门检查 |  | Fall from height 高处坠落 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Process inspection in the unit 装置内流程检查 |  | Fall from height 高处坠落 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Slowly open the steam inlet valve of the unit 缓慢打开装置蒸汽进装置阀 |  | The valve opening is too large 阀门开度太大 |  |  | Process accident Equipment damage 工艺事故 设备损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Drainage at the end of the process 流程末端排水 |  | The opening of drain valve of steam trap is too large 疏水器导淋阀开度太大 |  |  | Personnel scalding 人员烫伤 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 5 |  | System boost leak detection 系统升压查漏 |  | Pipe leakage 管子泄漏 |  |  | Personnel scalding 人员烫伤 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-10 |  | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: |  |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | High altitude patrol inspection 高处巡检 | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Confirmation of mental state of patrol inspectors and weather conditions 巡检人员精神状态、天气条件确认 |  |  | Fall from height 高处坠落 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  |  | Lightning strike 雷击 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 |  | Hold the handrail when going up the stairs 上楼梯时扶好扶手 |  |  | There is no handrail when going up and down stairs 上下楼梯不扶扶手 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 |  | Check and confirm the facilities on the tank top 对罐顶各部分设施检查确认 |  |  | Fall from height 高处坠落 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | The problems found shall be handled and reported in time 发现的问题及时处理汇报 |  |  | Potential safety hazards are not reported in time 安全隐患未及时汇报 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  |  | Abnormal breathing valve not reported 呼吸阀异常未汇报 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-11 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date:Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Slag scraping of air flotation tank 气浮池刮渣 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Liquid level confirmation of scum intermediate tank 浮渣中间罐液位确认 |  | Level gauge not confirmed 液位计未确认 |  |  | environmental pollution 环境污染 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 2 |  | Wear hydrogen sulfide alarm 佩戴硫化氢报警仪 |  | Hydrogen sulfide in equipment 设备内含硫化氢 |  |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Inspection of slag scraper and auxiliary slag scraper plate and chain 刮渣机及附属刮渣板、链条检查 |  | Chain off and sundries 链条脱链、有杂物 |  |  | Chain damage 链条损坏 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 4 |  | Start the slag scraper 启动刮渣机 |  | Chain falling off and damaged 链条脱落损坏 |  |  | Chain scraper damaged 链条刮板损坏 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 5 |  | Raise the liquid level of reaction tank 抬高反应池液位 |  | The liquid level rises too high 液面抬升太高 |  |  | The liquid level of scum tank rises too fast 浮渣池液位上升太快 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 6 |  | Reduce the liquid level in the reaction tank 降低反应池液位 |  | The opening of water outlet regulating valve is too large 出水调节阀开度太大 |  |  | Valve plate falling off 阀板脱落 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 7 |  | Stop the slag scraper 停运刮渣机 |  | Field operation column marking "stop" 现场操作柱打“停止” |  |  | Slag scraper self starting 刮渣机自启 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-12 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Equipment internal inspection and maintenance 设备内部检维修 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Lower the liquid level and empty the tank 降低液位，放空罐体 |  | Liquid level discharge too fast 液位排放太快 |  |  | Sewage tank overflow 污水池冒池 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |  |
| 2 |  | Handle general maintenance work ticket 办理一般检修作业票 |  | JHA risk identification is not carried out 未开展JHA风险识别 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | Add blind plate isolation 加盲板隔离 |  | The blind plate doesnot meetthe requirements or is not added 盲板不符合要求或未加盲板 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 4 |  | Flushing, purging, replacement and ventilation 冲洗、吹扫、置换、通风 |  | There are toxic and harmful gases inside without replacement 内部含有毒有害气体未经置换 |  |  | poisoning 中毒 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Insufficient internal oxygen without ventilation 内部氧气不足未经过通风 |  |  | asphyxia 窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | It contains combustible gas 内部含有可燃气体 |  |  | Fire and explosion 着火爆炸 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 5 |  | Contact the laboratory for sampling analysis 联系化验采样分析 |  | The sampling location is not typical 采样位置不具代表性 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 6 |  | Handle work ticket in confined space and implement safety measures 办理受限空间作业票，落实安全措施 |  | Inadequate implementation of safety measures 安全措施落实不到位 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 7 |  | Issue work permit and make safety technical disclosure to maintenance personnel 签发作业票并对检修作业人员安全技术交底 |  | Inadequate safety disclosure 安全交底不到位 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 8 |  | Arrange maintenance 安排检修 |  | Inadequate monitoring 监护不到位 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-13 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Pump maintenance 机泵检维修 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | power failure 停电 |  | No power failure 未停电 |  |  | electric shock 触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | No warning sign of "no switching on when someone is working" is hung 未挂“有人作业禁止合闸”警示牌 |  |  | electric shock 触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 |  | Prepare tools and instruments 准备工器具 |  | Tools and instruments are not explosion-proof 工器具不防爆 |  |  | Explosion 爆炸 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | System isolation replacement 系统隔离置换 |  | Corrosive media such as acid and alkali are not completely isolated and replaced 酸碱等腐蚀性介质未彻底隔离、置换 |  |  | Environmental pollution and personnel injury 环境污染人员伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | disassemble 拆卸 |  | Falling of suspended objects 吊物坠落 |  |  | Object strike 物体打击 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Reassembly 回装 |  | Falling of suspended objects 吊物坠落 |  |  | Object strike 物体打击 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  |  |  |  |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-14 |  | Department 部门: |  | Utilities Department 公用工程部 |  |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Reagent configuration 药剂配置 |  |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  |  |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Fill the dissolving tank with clean water 将溶药罐内加注清水 |  | The dissolving tank is full 溶药罐满罐 |  |  |  |  | environmental pollution 环境污染 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 2 |  | Start the mixer 启动搅拌机 |  | Liquid level lower than impeller start 液位低于搅拌机叶片启动 |  |  |  |  | Mixer damaged 搅拌机损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | The reagent is added to the dissolving reagent one by one in batches 将药剂分批逐次加入溶药 |  | Feeding splash 投料喷溅 |  |  |  |  | Personnel burns 人员灼伤 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | Stir and dissolve for more than 30 minutes 搅拌溶解30分钟以上 |  | Insufficient mixing time 搅拌时间不足 |  |  |  |  | The agent is not fully dissolved 药剂未充分溶解 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-15 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Replace lubricating oil of pump 机泵更换润滑油 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Switch the pump to standby pump 机泵切至备用泵 |  | Motor leakage 电机漏电 |  |  | Electric shock to personnel 人员触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Crank 盘车 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Mark "manual" on the operation column 操作柱上打“手动” |  | Mark "automatic" on the operation column 操作柱上打“自动” |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Drain and replace the waste oil 将废油放空并置换 |  | Waste oil is not effectively collected 废油未有效盛接 |  |  | environment pollution 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Replace the lubricating oil according to the five fixed table 按五定表内容更换润滑油 |  | Overfilling of lubricating oil 润滑油加注过满 |  |  | Poor lubrication of pump 机泵润滑不良 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | The pump resumes operation 机泵恢复运行 |  | Motor leakage 电机漏电 |  |  | Electric shock to personnel 人员触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Crank 盘车 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-16 | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  | Biochemical blower cutter 生化鼓风机切机 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 | Inspection before fan startup 风机开机前检查 |  | Motor leakage 电机漏电 |  |  | Electric shock to personnel 人员触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  | Crank 盘车 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 | Fan startup operation 风机开机操作 |  | Exposed rotating parts 转动部位外露 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 | Fan online switching 风机在线切换 |  | High outlet pipeline temperature 出口管线温度高 |  |  | Personnel scalding 人员烫伤 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 | Shutdown of original operating fan 原运行风机停运 |  | Turning operation after stopping 停下就盘车作业 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-17 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Pump switching 机泵切换 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Confirm before starting the pump 机泵开泵前确认 |  | Motor leakage 电机漏电 |  | Electric shock to personnel 人员触电 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Turning gear 盘车 |  | Mechanical injury 机械伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Pump start-up and operation 机泵启动运行 |  | Exposed rotating parts 转动部位外露 |  | Mechanical injury 机械伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Pump on-line switching 机泵在线切换 |  | Flow fluctuation 流量波动 |  | Affect system flow 影响系统流量 | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
| 4 |  | Shutdown of original operating pump 原运行机泵停运 |  | Crank 盘车 |  | Mechanical injury 机械伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-18 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：January 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Centrifuge refueling (grease) 离心机加油（脂） |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Shut down 停机 |  | Hydrogen sulfide leakage 硫化氢泄漏 |  | Personnel poisoning 人员中毒 |  |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 |  | power failure 停电 |  | Wrong equipment tag numbeof power failure ticket 停电票设备位号写错 |  | Electric shock to personnel 人员触电 |  |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Stop on other moving equipet caseof power failurein cabinet room 机柜间停电时停到其他动设备上 |  | Electric shock to personnel 人员触电 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Repair and remove the frame body cover 检维修拆机身罩子 |  | The motor is not powered off 电机未断电 |  | Electric shock to personnel 人员触电 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Add oil (grease) 加油（脂） |  | Lubricating oil (grease) spilled on the ground 润滑油（脂）洒落地面 |  | environment pollution 污染环境 |  |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-19 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Dehydration of oil sludge scum concentration tank 油泥浮渣浓缩罐脱水 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Wear hydrogen sulfide alarm 佩戴硫化氢报警仪 |  | The alarm is not worn or turned on 报警仪未佩戴或未打开 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | One person operates and one person monitors 一人操作，一人监护 |  | The guardian is not present 监护人不在现场 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Stand at the upper air outlet for operation 站在上风口操作 |  | No wind direction 未看风向 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Slightly open the dehydration valve and slowly dehydrate 微开脱水阀门，缓慢脱水 |  | The valve opening is too large 阀门开度太大 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-20 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Sludge thickening tank dewatering 污泥浓缩罐脱水 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Wear hydrogen sulfide alarm 佩戴硫化氢报警仪 |  | The alarm is not worn or turned on 报警仪未佩戴或未打开 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | One person operates and one person monitors 一人操作，一人监护 |  | The guardian is not present 监护人不在现场 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Stand at the upper air outlet for operation 站在上风口操作 |  | No wind direction 未看风向 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Slightly open the dehydration valve and slowly dehydrate 微开脱水阀门，缓慢脱水 |  | The valve opening is too large 阀门开度太大 |  | Personnel poisoning 人员中毒 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-21 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Waste oil delivery 污油外送 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Confirmation of delivery process 外送流程确认 |  | The process is blocked 流程不通 |  | Process accident Pump damage 工艺事故 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 2 |  | Start pumping oil 启泵送油 |  | Waste oil of each unit is channeling with each other 各装置污油互窜 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-22 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Sodium hypochlorite discharge 浓硫酸、次氯酸钠卸料 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Operators shall receive safety training and wear complete PPE 操作人员经过安全培训，穿戴完整PPE |  | Medication splashed onto the skin. 药品溅到皮肤上 |  | Corrode skin 腐蚀皮肤 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 |  | Warning ropes shall be set in the operation area for unloading hazardous chemicals 卸危化品操作区域设置警戒绳 |  | Injury to irrelevant personnel 无关人员受到伤害 |  | Irritating / corrode skin 刺激/腐蚀皮肤 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | The unloading facilities such as unloading pump and hose are confirmed to be in good condition 缷料泵、软管等卸料设施确认完好 |  | Discharge pump leakage 卸料泵泄漏 |  | environment pollution personnel injury 污染环境 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Discharge line joint leakage 卸料管线接头泄漏 |  | environment pollution personnel injury 污染环境 人身伤害 | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 4 |  | The team must be supervised by a specially assigned person 班组必须安排专人监护 |  | Operators operate against rules 作业人员违章作业 |  | Personnel injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Pay close attention to the liquid level during unloading to prevent tank overflow 卸料时密切关注液位，以防止冒罐 |  | Sodium hypochlorite tank 次钠罐冒罐 |  | environment pollution 污染环境 | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 6 |  | After unloading, add clean water into the secondary sodium tank and replace the residual secondary sodium in the unloading pump and hose 卸料结束，向次钠桶内加清水，将卸料泵和软管内残留次钠置换干净 |  | Leakage of sodium hypochlorite to the ground 次氯酸钠泄漏至地面 |  | environment pollution 污染环境 | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
| 7 |  | Platforms and floors with residual media shall be flushed with production water 有残余介质的平台、地面等应用生产给水冲洗干净 |  | Operator skin contact 操作人员皮肤接触到 |  | Corrode skin 腐蚀皮肤 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-23 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Inspection and cleaning of valve well 阀门井检查清理 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Isolation 隔离 |  | Work at heights around the valve well 阀门井周围高处作业 |  |  | Object strike 物体打击 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Ventilation and replacement 通风、置换 |  | There are toxic and harmful gases inside without replacement 内部含有毒有害气体未经置换 |  |  | Poisoning 中毒 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Insufficient internal oxygen without ventilation 内部氧气不足未经过通风 |  |  | asphyxia 窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 |  | Gas analysis 气体化验分析 |  | The sampling location is not typical 采样位置不具代表性 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | Handle work ticket in confined space 办理受限空间作业票 |  | JHA risk identification is not carried out 未开展JHA风险识别 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Implement safety protection measures 落实安全防护措施 |  | Inadequate implementation of safety measures 安全措施落实不到位 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 6 |  | Safety technical disclosure of operators 作业人员安全技术交底 |  | Inadequate safety disclosure 安全交底不到位 |  |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 7 |  | Operator work 作业人员作业 |  | Inadequate monitoring 监护不到位 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5152-24 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: | Sewage treatment plant 污水处理场 |  |  |  |  |  |  | Analysis date：Feb10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Routine inspection 常规巡检 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Wear PPE and prepare required tools and instruments 穿戴PPE，准备所需工器具 |  | PPE not worn as required PPE未按要求穿戴 |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | II |  |  |  |  |  |  |
| 2 |  | Start patrol inspection 开始巡检 |  | Missed inspection 漏检 |  | Abnormal problems were not found in time 异常问题未及时发现 |  | Rarely happens 很少发生 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The patrol inspection is not serious, and the abnormal operation of the equipment is not found in time 巡检不认真，未及时发现设备异常运行 |  | Equipment damage 设备损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Record data 记录数据 |  | Data recording error 数据记录错误 |  | Abnormal problems were not found in time 异常问题未及时发现 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Check data 核对数据 |  | Data not checked 数据未核对 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Handling of problems found during patrol inspection 巡检发现的问题处理 |  | Problems found are not reported 发现问题不汇报 |  | The impact of the problem is expanded, resulting in safety and process accidents 问题影响扩大导致安全、工艺事故 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | If problems are found, they are not handled in time 发现问题不及时处理 |  | The impact of the problem is expanded, resulting in safety and process accidents 问题影响扩大导致安全、工艺事故 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | The inspection found wild animals, such as crocodiles, lizards, venomous snakes, wild bees, wild dogs, etc 巡检时发现野生动物，如鳄鱼、蜥蜴、毒蛇、野土蜂、野狗等 |  | Wild animals accidentally injure people 野生动物意外伤人 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 6 |  | 巡检结束 End of patrol inspection |  | The number of patrol inspection instruments was not uploaded in time 巡检仪数未及时上传 |  | 巡检过程无法有效监控 The patrol inspection process cannot be effectively monitored |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |