|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表模板 |  |  |
|  | Template No. | HYBN-T7-08-2002-2024-2 | Page 1 of 4 |

The following is the body of the template.

以下为模板正文。

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-001 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Jan 10,2025 分析日期：2025年2月 10日 |  |  |  |  |
| Task 工作任务: |  |  | Inclined plate washing of coagulation sedimentation tank 混凝沉淀池斜板冲洗 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 001 |  | Inclined plate flushing of coagulation sedimentation tank 混凝沉淀池斜板冲洗 |  | The liquid level is higher than the inclined plate 液位高于斜板 |  | Unable to reach the flushing effect 达不到冲洗效果 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Standing on the inside of the guardrail or washing at the water inlet 站在护栏内侧或进水端冲洗 |  | Fall into the coagulation pool 跌入混凝池 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Coil not received盘管未收 |  | Personnel tripped and injured 人员绊倒，受伤 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | After flushing, water is not injected above the inclined plate 冲洗完成未注水至斜板以上 |  | The inclined plate is exposed to the sun and damaged 斜板暴晒，损坏 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-002 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Crank 盘车 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 002 |  | 盘车 Crank |  | Internal and external operations are not contacted内外操未联系 |  | Mechanical damage 机械伤害 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | "Manual" or "stop" is not set on site 现场未打到“手动”或“停止” |  | Mechanical damage 机械伤害 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The device is running设备处于运行状态 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | Failure to crank 2-3 turns as required 未按规定盘车2-3圈 |  | Equipment damage 设备损坏 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-003 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Switch pump 切泵 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 003 |  | Switch pump 切泵 |  | The standby pump status is not checked and confirmed未检查确认备用泵状态 |  | Damaged pump 机泵损坏 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Hit the rotating parts when starting the pump启泵时碰到转到部件 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | Pump pressure is hold back during pump switching time 切泵时机泵憋压 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | 停原运行泵流量中断或波动较大 |  | Affect the stable operation of subsequent devices 影响后续装置稳定运行 |  | Occasionally 偶尔 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-004 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Jan 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Manual backwashing quartz sand filter手动反洗石英砂过滤器 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 004 |  | Manual backwashing quartz sand filter手动反洗石英砂过滤器 |  | Quartz sand filter is not switched to "manual" state 石英砂过滤器未切到“手动”状态 |  | Backwashing cannot be carried out 反洗无法进行 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Wrong valve opening 阀门开错 |  | Poor water running or backwashing effect 跑水或反洗效果差 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | Insufficient or too long backwash time 反洗时间不足或过长 |  | High turbidity浊度偏高 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  |  |  | Sewage tank污水池冒池 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-005 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  |  | Analysis date:Feb10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Sampling取样 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 005 |  | Sampling 取样 |  | Sampling bottle is not cleaned取样瓶未清洗 |  |  | Analysis error 分析误差 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | No circulation during sampling for 1-2 minutes取样时未流通1-2分钟 |  |  | Data error 数据误差 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Wrong sample taken during sampling取样时样品取错 |  |  | Data error 数据错误 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Wrong label 标签贴错 |  |  | Data error 数据错误 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-006 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Changing lubricating oil 更换润滑油 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 006 |  | Changing lubricating oil 更换润滑油 |  | Replace the lubricating oil without stopping the pump更换润滑油未停泵 |  |  | Damaged pump 机泵损坏 | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | Waste lubricating oil is dumped at will 废润滑油随意倾倒 |  |  | Polluted environment污染环境 | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The lubricating oil is added incorrectly or the oil level is inappropriate 润滑油加错或油位不合适 |  |  | Damaged pump 机泵损坏 | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The lubricating oil on site is not cleaned现场润滑油未清理 |  |  | Polluted environment污染环境 | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-007 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Pump inspection and maintenance 机泵检维修 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 007 |  | Pump inspection and maintenance 机泵检维修 |  | After the pump stops, the interlock starts automatically |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Wrong equipment tag number for power failure 停电设备位号错误 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The equipment is under pressure or there is medium 设备带压或存在有介质 |  | Personal injury, hot water scald人身伤害 热水烫伤 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The system is not completely isolated 系统未完全隔离 |  | Cause safety incident 引起安全事故 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | The equipment is under pressure or there is medium 设备带压或存在有介质 |  | Personal injury 人身伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Tool slipped 工具滑落 |  | Object strike物体打击 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-008 |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | High altitude inspection高处巡检 |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 008 |  | 高处巡检 High altitude inspection |  | Lightning strike 雷击 |  | Personal injury 人员伤害 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | There is no handrail when going up and down stairs 上下楼梯不扶扶手 |  | Personal injury 人员伤害 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Fall from height 高处坠落 |  | Personnel injury 人员伤害 |  | Occasionally 偶尔 |  | 3 | 4 | 12 | III | 1.Strictly implement operating procedures严格执行操作规程 2.High altitude patrol inspection is not allowed for poor spirit精神不佳不允许高处巡检 |  | 3 | 2 | 6 | II |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5111-3-009 |  | Department 部门: |  | Utilities Department 公用工程部 | Unit 装置: |  | Lake water utilization unit 湖水利用装置 |  |  |  |  |  |  | Analysis date： Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Device inspection 装置巡检 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren 张浩然、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 009 |  | Device inspection装置巡检 |  | Incomplete wearing of patrol inspection tools and instruments 巡检工器具配戴不齐全 |  |  | Personnel injury 人身伤害 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Go up and down the stairs without supporting the handrails上下楼梯不扶扶手 |  |  | Personnel injury 人身伤害 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Fall from height高处坠落 |  |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Safety hazards were not reported in time 安全隐患未及时汇报 |  |  | Safety incident property loss安全事故 财产损失 |  | Occasionally 偶尔 | 3 | 3 | 9 | III | 严格执行巡检制度，建立隐患整改台账 Strictly implement the inspection system and establish a hidden danger rectification account |  | 2 | 2 | 4 | I |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |