|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表模板 |  |  |
|  | Template No. | HYBN-T7-08-2003-2024-2 | Page 1 of 2 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-001 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Device/equipment/facilities: P301, P302A/B Lake water booster pump, water intake pump 装置/设备/设施：P301、P302A/B生活水加压泵、取水泵 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 001 | P301 P302A/B Lake water booster pump and water intake pump operation motor inspection 生活水加压泵、取水泵运行电机检查 |  | Whether the motor bearing grease overflows and fill the record 电机轴承润滑脂是否溢出，加注记录 |  | Motor bearing burnt 电机轴承烧 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the motor temperature and current meet the requirements 电机温度，电流是否符合要求 |  | Overload, motor burns 超负荷，电机烧 | Rarely happens很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the motor vibration value meets the requirements 电机振动值是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the noise of the motor has increased 电机杂声是否增大 |  | Motor shaft holding 电机抱轴 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-003 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Device/equipment/facilities: P303A/B, P304A/B Inlet pump for coagulation sedimentation tank, sand filter inlet pump 装置/设备/设施：P303A/B、P304A/B 混凝沉淀池进水泵、砂滤进水泵 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 002 | Inspection of the running motor of the inlet pump of the coagulation sedimentation tank and the inlet pump of the sand filter P303A/B P304A/B 混凝沉淀池进水泵、砂滤进水泵运行电机检查 |  | Whether the grease of the motor bearing overflows, fill the record 电机轴承润滑脂是否溢出，加注记录 |  | Motor bearing burnt 电机轴承烧 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the motor temperature and current meet the requirements 电机温度，电流是否符合要求 |  | Overload, motor burns超负荷，电机烧 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the motor meets the requirements电机振动值是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the noise of the motor has increased 杂声是否增大 |  | Motor shaft 电机抱轴 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-005 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Installation/equipment/facilities: P307A/B, P308A/B lake aquatic product water pump, backwash water pump 装置/设备/设施：P307A/B、P308A/B湖水产水泵、反洗水泵 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 003 | P307A/B P308A/B Lake aquatic product water pump, backwash pump running motor inspection 湖水产水泵、反洗水泵运行电机检查 |  | Whether the grease of the motor bearing overflows, fill the record 电机轴承润滑脂是否溢出，加注记录 |  | Motor bearing burnt 电机轴承烧 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the motor temperature and current meet the requirements 电机温度，电流是否符合要求 |  | Overload, motor burns超负荷，电机烧 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the vibration value of the motor meets the requirements电机振动值是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens 很少发生 | 2 |  | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the noise of the motor has increased 电机杂声是否增大 |  | Motor shaft 电机抱轴 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-007 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Installation/Equipment/Facilities:B301A/B air washing blower 装置/设备/设施：B301A/B气洗鼓风机 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 004 | B301A/B Air washing blower 气洗鼓风机 |  | Grease addition 润滑脂添加情况 |  | Bearing wear and burnout 轴承磨损、烧坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the current exceeds the rated value 电流有无超额定值 |  | Motor burnt out 电机烧毁 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the temperature, vibration and noise meet the requirements 温度、振动、杂音是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the belt is severely worn 皮带是否磨损严重 |  | Damaged belt 皮带损坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-008 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Installation/Equipment/Facilities:MP301A/B compressor 装置/设备/设施：MP301A/B压缩机 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 005 | MP301A/B Compressor 压缩机 |  | Lubricating condition 润滑油情况 |  | Wear and burnout 磨损、烧坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the current exceeds the rated value 电流有无超额定值 |  | Motor burnt out 电机烧毁 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the temperature, vibration and noise meet the requirements 温度、振动、杂音是否符合要求 |  | Equipment damage 设备损坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether each detection element is normal 各检测元件是否正常 |  | Damage 损坏 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-009 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Installation/Equipment/Facilities:TK301 TK303 Lake water homogenization adjustment tank, water production tank 湖水均质调节罐、产水罐 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 006 | TK301 TK303 Lake water homogenization adjustment tank, water production tank湖水均质调节罐、产水罐 |  | No cracks on the surface of the tank body 罐本体表面无裂纹 |  | The tank body is damaged 罐本体损坏 | Rarely happens很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Safety accessories such as level gauge are intact 液位计等安全附件完好 |  | Venting or evacuation 冒罐或抽空 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Static grounding is intact 静电接地完好 |  | Poor electrostatic grounding 静电接地不良 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | The valve is intact and the connecting flange has no leakage 阀门完好、连接法兰无泄漏 |  | Runoff or leaking 跑冒滴漏 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-010 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Device/equipment/facilities: F301A～D quartz sand filter 装置/设备/设施：F301A～D石英砂过滤器 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 07 | F301A~D Quartz sand filter 石英砂过滤器 |  | Whether the status of each pneumatic valve is normal 确认各气动阀门的状态是否正常 |  | Cause the turbidity of circulating water to rise or "running water" 造成循环水浊度上升或“跑水” | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the differential pressure at the inlet and outlet of quartz sand filter is normal 石英砂过滤器进出口压差是否正常 |  | Quartz sand is hardened and the effluent becomes worse 石英砂板结，出水变差 | Rarely happens很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Whether the on-site PLC control panel displays and operates normally现场PLC控制面板显示、操作是否正常 |  | Unable to manually operate on site or the side filter operation program is chaotic 现场无法手动操作或旁滤器运行程序混乱 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Confirm whether there is "running dripping" 确认有无“跑冒滴漏” |  | Running water 跑水 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号：5111-3-011 |  | Unit:Utilities Department 单位：公用工程部 |  | Members of SCL analysis group: SCL分析小组人员： ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Analysis date: Feb 10,2025 分析日期： 2025 年2月10日 |  |  |  |  |
| Unit (area)/process name: Lake water utilization device 单元（区域）/工艺过程名称：湖水利用装置 |  |  |  | Device/equipment/facilities: dosing room 装置/设备/设施：加药间 |  |  |  |  |  |  |  | Reviewed by:Wang Liang 审核人：王亮 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 08 | Dosing Room加药间 |  | Whether the doors and windows are damaged or cannot be closed门窗有无损坏、关不上情况 |  | Broken doors and windows, rainwater dripping into the dosing room 门窗坏、雨水淋到加药间 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the exhaust fan can work normally 排风扇是否能正常工作 |  | Poor ventilation 通风不畅 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the eyewash water quality and water pressure normal 洗眼器水质、水压否正常 |  | The eyewash cannot be used normally 洗眼器无法正常使用 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the reagent stacking meets the requirements 药剂堆放是否符合要求 |  | Wrong dosage 药剂投加错误 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the liquid level of reagent storage tank and the operation of reagent pump are normal 药剂储罐液位、药剂泵运行是否正常 |  | The pump is damaged and the reagent cannot be added normally 泵损坏，药剂无法正常投加 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the safety signs are complete安全标识是否齐全 |  | Personal injury 人身伤害 | Rarely happens很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |