|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Equipment and Facilities List 设备设施清单模板 |  |  |
|  | Template No. | HYBN-T7-08-2008-2024-2 | Page 1 of 2 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6- -2008- -2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  | Utilities Department公用工程部 |  |  |  | Name of unit (area) 单元（区域）名称 |  | Lake water utilization device 湖水利用装置 |
| Reviewed by 审核人 |  |  | Shishuigen 施水根 |  |  |  | Date 日期 |  | Feb 10 2025 2025年2月10日 |
| S/N 序号 | No. of SCL form SCL表编号 |  |  | Name of equipment/facility 设备设施名称 |  | Inspection item 检查项目 |  |  |  |
|  | 001 |  |  | Lake water booster pump, water intake pump operating pump inspection P301 P302AA/B 生活水加压泵、取水泵运行机泵检查 |  | 1.Whether the motor bearing grease overflows, fill in the record;电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements;电机温度，电流是否符合要求； 3.Whether the vibration value of the motor meets the requirements;电机振动值是否符合要求； 4.Whether the noise of the motor has increased.电机杂声是否增大。 5.Lubricating grease for pump bearing, filling record;机泵轴承润滑脂，加注记录； 6.Whether the temperature of the pump bearing meets the requirements;机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements;机泵振动值是否符合要求； 8.Whether the outlet pressure of the pump is normal;机泵出口压力是否正常； 9.Whether the machine seal is leaking.机封有无泄漏。 |  |  |  |
|  | 002 |  |  | Inspection of the running motor of the inlet pump of the coagulation sedimentation tank and the inlet pump of the sand filter P303A/B P304A/B 混凝沉淀池进水泵、砂滤进水泵运行电机检查 |  | 1.Whether the motor bearing grease overflows, fill in the record;电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements;电机温度，电流是否符合要求； 3.Whether the vibration value of the motor meets the requirements;电机振动值是否符合要求； 4.Whether the noise of the motor has increased.电机杂声是否增大。 5.Lubricating oil for pump bearing, filling record;机泵轴承润滑油，加注记录； 6.Whether the temperature of the pump bearing meets the requirements;机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements;机泵振动值是否符合要求； 8.Whether the outlet pressure of the pump is normal;机泵出口压力是否正常； 9.Whether the machine seal is leaking.机封有无泄漏。 |  |  |  |
|  | 003 |  |  | Lake aquatic product water pump, backwash pump running machine pump inspection P307A/B P308A/B 湖水产水泵、反洗水泵运行机泵检查 |  | 1.Whether the motor bearing grease overflows, fill in the record;电机轴承润滑脂是否溢出，加注记录； 2.Whether the motor temperature and current meet the requirements;电机温度，电流是否符合要求； 3.Whether the vibration value of the motor meets the requirements;电机振动值是否符合要求； 4.Whether the noise of the motor has increased.电机杂声是否增大。 5.Lubricating oil for pump bearing, filling record;机泵轴承润滑油，加注记录； 6.Whether the temperature of the pump bearing meets the requirements;机泵轴承温度是否符合要求； 7.Whether the vibration value of the pump meets the requirements;机泵振动值是否符合要求； 8.Whether the outlet pressure of the pump is normal;机泵出口压力是否正常； 9.Whether the machine seal is leaking.机封有无泄漏。 |  |  |  |
|  | 004 |  |  | B301A/B Air washing blower气洗鼓风机 |  | 1.Lubricating grease (oil) adding situation, filling record;润滑脂（油）添加情况，加注记录； 2.Whether the motor temperature and current meet the requirements;电机温度，电流是否符合要求； 3.Whether the motor bearing temperature, vibration and noise meet the requirements;电机轴承温度、振动、杂音是否符合要求 4.Whether the outlet pressure of the fan is normal;风机出口压力是否正常； 5.Whether the belt is severely worn.带是否磨损严重。 |  |  |  |
|  | 005 |  |  | MP301A/B Compressor 压缩机 |  | 1.Lubricating oil addition situation, filling record;润滑油添加情况，加注记录； 2.Whether the current meets the requirements;电流是否符合要求； 3.Whether the temperature, vibration and noise meet the requirements;温度、振动、杂音是否符合要求； 4.Whether each detection element is normal.各检测元件是否正常。 |  |  |  |
|  | 006 |  |  | TK301 TK302 Lake water homogenization adjustment tank, water production tank 湖水均质调节罐、产水罐 |  | 1.No cracks on the surface of the tank body;罐本体表面无裂纹； 2.Safety accessories such as level gauges are intact;液位计等安全附件完好； 3.The static grounding is intact;静电接地完好； 4.The valve is intact and the connecting flange has no leakage.阀门完好、连接法兰无泄漏。 |  |  |  |
|  | 007 |  |  | F301A~D Quartz sand filter石英砂过滤器 |  | 1.Whether the status of each pneumatic valve is normal;确认各气动阀门的状态是否正常； 2.Whether the differential pressure between the inlet and outlet of the side filter is normal;旁滤器进出口压差是否正常； 3.Whether the on-site PLC control panel display and operation are normal;现场PLC控制面板显示、操作是否正常； 4.Confirm whether there is "running dripping".确认有无“跑冒滴漏”。 |  |  |  |
|  | 008 |  |  | Dosing room 加药间 |  | 1.Whether the doors and windows are damaged or cannot be closed;门窗有无损坏、关不上情况； 2.Whether the exhaust fan can work normally;排风扇是否能正常工作； 3.Whether the water quality and water pressure of the eyewash are normal;洗眼器水质、水压否正常； 4.Whether the reagent stack meets the requirements;药剂堆放是否符合要求； 5.Whether the liquid level of reagent storage tank and the operation of reagent pump are normal;药剂储罐液位、药剂泵运行是否正常； 6.Whether the safety signs are complete.安全标识是否齐全。 |  |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。