|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job List 作业活动清单 |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T6-**-2001-***-2024 |  |  | Page 2 of 2 |
| Department 部门 |  |  | Utilities Department 公用工程部 |  |  | Unit (Area) 单元（区域） | Alkali residue treatment 碱渣处理 |
| Identified by 辨识人 |  |  | Wangliang,ZhangHaoran,Liangbin,Liguan Wangliang,ZhangHaoran,Yuyapeng、Liangbin,Liguangyuan,Lvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 王亮、张浩然、于亚鹏、梁斌、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  | Date 日期 | Feb 10,2025 2025年2月10日 |
| S/N 序号 | Main job 主要作业活动 |  |  |  | Main procedures of job 作业活动主要步骤 |  |  |
| 01 | Unloading sulfuric acid (98% concentration) 硫酸（98%浓度）缷车 |  |  |  | 1.Operators shall receive safety training and wear complete PPE操作人员经过安全培训，穿戴完整PPE |  |  |
|  |  |  |  |  | 2.Warning line shall be set in the operation area for unloading hazardous chemicals卸危化品操作区域设置警戒带 |  |  |
|  |  |  |  |  | 3.The unloading facilities such as unloading pump, quick connector and hose are confirmed to be intact缷料泵、快速接头、软管等卸料设施确认完好 |  |  |
|  |  |  |  |  | 4.The team must be supervised by a specially assigned person班组必须安排专人监护 |  |  |
|  |  |  |  |  | 5.Pay close attention to the liquid level during unloading to prevent tank overflow卸料时密切关注液位，以防止冒罐 |  |  |
|  |  |  |  |  | 6.After unloading, stop the unloading pump first and recover the residual sulfuric acid in the unloading pipeline; Remove the quick coupling and flush the unloading hose卸料结束，先停运泵，将卸料管线内残余硫酸进行回收；拆除快速接头，并对缷车软管进行冲洗 |  |  |
|  |  |  |  |  | 7.Platforms and floors with residual media shall be washed with production water有残余介质的平台、地面等应用生产给水冲洗干净 |  |  |
| 2 | Crank 盘车 |  |  |  | 1.Contact the internal operator to remove the pump interlock联系内操切除机泵联锁 |  |  |
|  |  |  |  |  | 2.Field pump set to “manual” on operation column现场机泵操作柱打“手动” |  |  |
|  |  |  |  |  | 3.Turn 2 ~ 3 turns according to the running direction按运转方向盘车2~3圈 |  |  |
|  |  |  |  |  | 4.Contact the internal operator to put the pump into interlock operation联系内操投运机泵联锁 |  |  |
| 3 | Switchover pump 机泵切换 |  |  |  | 1.Confirm before starting the pump机泵开泵前确认 |  |  |
|  |  |  |  |  | 2.Pump start-up and operation机泵启动运行 |  |  |
|  |  |  |  |  | 3.Pump on-line switching机泵在线切换 |  |  |
|  |  |  |  |  | 4.Shutdown of original operating pump原运行机泵停运 |  |  |
| 4 | Sampling 采样 |  |  |  | 1.Wait for circulation for 1-2 minutes before sampling待取样流通1-2分钟 |  |  |
|  |  |  |  |  | 2.The sampling bottle shall be rinsed for 2 to 3 times取样瓶润洗2至3次 |  |  |
|  |  |  |  |  | 3.Stand upwind and take samples站在上风向处取样 |  |  |
|  |  |  |  |  | 4.Label the sample bottle贴好样瓶标签 |  |  |
| 5 | Changing lubricating oil of pump 机泵更换润滑油 |  |  |  | 1.Switch the pump to standby pump机泵切至备用泵 |  |  |
|  |  |  |  |  | 2.Set to "manual" on the operation column操作柱上打“手动” |  |  |
|  |  |  |  |  | 3.Vent and replace the waste oil将废油放空并置换 |  |  |
|  |  |  |  |  | 4.Replace the lubricating oil according to the five schedules按五定表内容更换润滑油 |  |  |
|  |  |  |  |  | 5.The pump resumes operation机泵恢复运行 |  |  |
| 6 | Pump maintenance 机泵检维修 |  |  |  | 1.Power failure停电 |  |  |
|  |  |  |  |  | 2.Prepare tools and instruments准备工器具 |  |  |
|  |  |  |  |  | 3.System isolation and replacement系统隔离、置换 |  |  |
|  |  |  |  |  | 4.Disassemble拆卸 |  |  |
|  |  |  |  |  | 5.Reassembly回装 |  |  |

Descriptions: 1. List major jobs for each process unit (area) which is considered as the assessment unit.

说明：1以工艺单元（区域）为评价单元，列出单元主要作业活动。

2 Simple jobs need to be more than 4 steps, while complex jobs need to be more than 8 steps.

简单作业活动要求分为4步以上，复杂作业活动要求分为8步以上。

3 This form is used for JHA risk assessment and shall be consistent with the No. in JHA form.

此表用于JHA法风险评价，JHA表编号要对应起来。