|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-01 |  | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: | Alkali residue treatment 碱渣处理 |  |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Sulfuric acid (98% concentration) unloading 硫酸（98%浓度）缷车 |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Operators shall receive safety training and wear complete PPE 操作人员经过安全培训，穿戴完整PPE |  | Illegal operation, sulfuric acid splash 违章操作、硫酸飞溅 |  | Sulfuric acid leakage, Personnel injury 硫酸泄漏、人身伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | A warning line shall be set up in the operation area for unloading hazardous chemicals 卸危化品操作区域设置警戒带 |  | Unrelated personnel were injured 无关人员受到伤害 |  | Personnel injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | The unloading facilities such as unloading pump, quick connector and hose are confirmed to be intact 缷料泵、快速接头、软管等卸料设施确认完好 |  | Leakage of discharge pump 卸料泵泄漏 |  | Pollute environment Personnel injury 污染环境 人身伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  | Leakage in discharge line joint 卸料管线接头泄漏 |  | Pollute environment Personnel injury 污染环境 人身伤害 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | The team must be supervised by a specially assigned person 班组必须安排专人监护 |  | Illegal operations by operators 作业人员违章作业 |  | Personnel injury 人员伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 5 |  | Pay close attention to the liquid level during unloading to prevent tank overflow 卸料时密切关注液位，以防止冒罐 |  | Sulfuric acid tank 硫酸冒罐 |  | Pollute environment 污染环境 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 6 |  | After unloading, stop the unloading pump first and recover the residual sulfuric acid in the unloading pipeline; Remove the quick coupling and flush the unloading hose. 卸料结束，先停运缷车泵，将卸料管线内残余硫酸进行回收；拆除快速接头，并对缷车软管进行冲洗。 |  | Sulfuric acid leakage 硫酸泄漏 |  | Pollute environment 污染环境 |  | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |  |
| 7 |  | Platforms and floors with residual media shall be flushed with production water 有残余介质的平台、地面等应用生产给水冲洗干净 |  | Operator skin contact 操作人员皮肤接触到 |  | Corrode skin 腐蚀皮肤 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-02 |  |  | Department 部门: | Utilities Department 公用工程部 |  |  | Unit 装置: | Alkali residue treatment 碱渣处理 |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |  |
| Task 工作任务: |  |  | Crank/ Turning gear 盘车 |  |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Contact the internal operator to cut off the pump interlock 联系内操切除机泵联锁 |  |  | The internal and external operation contact is out of control, and the interlocking removal is wrong 内外操联系失控，联锁切除错误 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 2 |  | Field pump operation column marking "manual" 现场机泵操作柱打“手动” |  |  | Mark "automatic" on the operatin column 操作柱上打“自动” |  |  | \Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | Turn 2-3 turns in the running direction 按运转方向盘车2-3圈 |  |  | Motor leakage 电机漏电 |  |  | Electric shock to personnel 人员触电 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | Pump start during turning 盘车过程中泵启动 |  |  | Mechanical injury 机械伤害 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 4 |  | Contact the internal operator to put the pump into operation for interlocking 联系内操投运机泵联锁 |  |  | The internal and external operatin contact is out of control, and the interlocking forgets to put into use 内外操联系失控，联锁忘记投用 |  |  | Process accident 工艺事故 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-03 |  | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: | Alkali residue treatment 碱渣处理 |  |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | Switchover pump 机泵切换 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Check and confirm 检查确认 |  | Failure to check the status of the backup pump 未检查确认备用泵状态 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 2 |  | Start the standby pump 启备用泵 |  | When the pump is started, the turning part is encountered 启泵时碰到转到部件 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 3 |  | Switch pump 切泵 |  | When the pump is switch, the pump is holding back pressure 切泵时机泵憋压 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Stop the original operation pump 停原运行泵 |  | The flow of the pump is interrupted or fluctuates greatly when the original operation is stopped 停原运行泵流量中断或波动较大 |  | Affect the stable operation of subsequent devices 影响后续装置稳定运行 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-04 |  | Department 部门: | Utilities Department 公用工程部 | Unit 装置: |  | Alkali residue treatment 碱渣处理 |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |
| Task 工作任务: |  |  | samplig 采样 | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Open the sampling valve and wait for the sampling pipeline to flow for 1-2 minutes 打开取样阀，待取样管线流通1-2分钟 |  | No drainage ditch 没有排放地沟 | environmental pollution 环境污染 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Rinse the sampling bottle 2 to 3 times 取样瓶润洗2至3次 |  | Randm pouring of rinsing and washig sampls 润洗样随意乱倒 | environmental pollution 环境污染 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Stand upwind and take samples 站在上风向处取样 |  | Hydrogen sulfide leakage at the lower air outlet of the samplig station 采样站在下风口硫化氢泄漏 | Personnel poisoning 人员中毒 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | Label the sample bottle 贴好样瓶标签 |  | Wrong label 贴错标签 | Data distortion, affecting production 数据失真，影响生产 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-05 |  |  | Department 部门: | Utilities Department 公用工程部 |  | Unit 装置: | Alkali residue treatment 碱渣处理 |  |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Pump lubricating oil replacement 机泵更换润滑油 |  |  | Members of analysis group 分析小组成员: | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 05 |  | Changing lubricating oil 更换润滑油 |  |  | Replace the lubricating oil without stopping the pump 更换润滑油未停泵 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | Waste lubricating oil is dumped at will 废润滑油随意倾倒 |  | Polluted environment 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | Wrong lubricating oil or improper oil level 润滑油加错或油位不合适 |  | Damaged pump 机泵损坏 |  | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | On-site lubricating oil is not cleaned 现场润滑油未清理 |  | Polluted environment 污染环境 |  | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Record No. | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: | 5154-06 |  |  | Department 部门: |  |  | Utilities Department 公用工程部 |  | Unit 装置: |  | Alkali residue treatment 碱渣处理 |  |  |  |  |  | Analysis date:Feb 10,2025 分析日期：2025年2月 10 日 |  |  |  |  |  |
| Task 工作任务: |  |  | Pump maintenance 机泵检维修 |  |  | Members of analysis group 分析小组成员: |  | ZhangHaoran,Liangbin,Yuyapeng,LiguangyuanLvyingqiang,Yinzujun,PanWentao,Jiangruibin,Dou Jianqiang, Zhaoxueren and team members 张浩然、梁斌、于亚鹏、李光远、吕应强、尹祖君、潘文涛、蒋瑞兵、窦建强、赵学仁及班组人员 |  |  |  |  |  |  |  |  |  | Reviewed by 审核人： | Wang Liang 王亮 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  |  | Hazard source or potential incident 危险源或潜在事件 |  |  |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  |  | L | S | R | Level等级 |
| 1 |  | Power failure 停电 |  |  | No power outage 未停电 |  |  |  | Electric shock 触电 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
|  |  |  |  |  | The warning sign of "No one is allowed to switch on" is not hung up 未挂“有人作业禁止合闸”警示牌 |  |  |  | Electric shock 触电 | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |  |
| 2 |  | Prepare tools 准备工器具 |  |  | Tools and equipment do not meet the requirements 工器具不符合要求 |  |  |  | Personal injury 人身伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 3 |  | System isolation replacement 系统隔离置换 |  |  | Corrosive media such as acid and alkali are not completely isolated and replaced 酸碱等腐蚀性介质未彻底隔离、置换 |  |  |  | Environmental pollution Personnel injury 环境污染人员伤害 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 4 |  | Disassemble 拆卸 |  |  | Falling of suspended objects 吊物坠落 |  |  |  | Object strike 物体打击 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| 5 |  | Reload 回装 |  |  | Falling of suspended objects 吊物坠落 |  |  |  | Object strike 物体打击 | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |