|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Job List 作业活动清单模板 |  |  |
|  | Template No. | HYBN-T7-08-2001-2024-2 | Page 1 of 5 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Job List 作业活动清单 |  |  |  |  |  |
|  |  | Record No. |  |  | HYBN-T6-**-2001-***-2024 |  | Page 2 of 5 |
| Department 部门 |  |  | Utilities Department 公用工程部 |  |  | Unit (Area) 单元（区域） | Air separation and air compression area 空分空压区域 |
| Identified by 辨识人 |  |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  | Date 日期 | MM DD, YYYY 2025年2月15日 |
| S/N 序号 | Main job 主要作业活动 |  |  | Main procedures of job 作业活动主要步骤 |  |  |  |
| 1 | Clean the filter before the pump 清洗泵前过滤器 |  |  | 1.The pump is shut down, the power is cut off and the pump is listed, and the inlet and outlet valves of the pump are closed;泵停运，断电挂牌，关闭泵进出口阀门； |  |  |  |
|  |  |  |  | 2.Enabling the shower to relieve pressure and discharge; 开导淋泄压排放； |  |  |  |
|  |  |  |  | 3. Prepare suitable tools and instruments;准备合适的工器具； |  |  |  |
|  |  |  |  | 4.Loosen the filter bolts to relieve the pressure;松动过滤器螺栓，泄压； |  |  |  |
|  |  |  |  | 5.Remove the filter for cleaning;拆除过滤器清洗； |  |  |  |
|  |  |  |  | 6. Reinstall and open the pump inlet valve to test for leakage 回装，并打开泵进口阀门试漏。 |  |  |  |
| 2 | Cryogenic liquid nitrogen pump cold pump 低温液氮泵冷泵 |  |  | - Replace with dry gas, measure dew point; 干燥气体置换，测露点； |  |  |  |
|  |  |  |  | - Open the reflux valve of the pump body, and it is air-cooled;打开泵体回流阀，气冷； |  |  |  |
|  |  |  |  | - Slightly open the pump inlet valve, liquid cooling; 微开泵进口阀门，液冷； |  |  |  |
|  |  |  |  | - If there is continuous liquid outflow from the discharge at the high point of the pump, the cold pump ends. 泵高点排放处有连续的液体流出，则冷泵结束。 |  |  |  |
| 3 | Hand over maintenance of low temperature moving equipment 低温动设备检修交出 |  |  | - The equipment is out of service, powered off, and listed; 设备停运，断电，挂牌； |  |  |  |
|  |  |  |  | - Close the inlet and outlet valves and return valves; 关闭进出口阀门、回流阀门； |  |  |  |
|  |  |  |  | - Open the pump body to drain; 打开泵体排液； |  |  |  |
|  |  |  |  | - Open the heating gas valve, reheat and hand over. 打开加温气阀门，复温，交出。 |  |  |  |
| 4 | Hand over maintenance of moving equipment at room temperature 常温动设备检修交出 |  |  | - The equipment is out of service, powered off, and listed; 设备停运，断电，挂牌； |  |  |  |
|  |  |  |  | - Close the equipment inlet and outlet valves; 关闭设备进出口阀门； |  |  |  |
|  |  |  |  | - Drain the medium in the equipment and relieve the pressure. 排空设备内介质，泄压。 |  |  |  |
|  |  |  |  | - Replacement and surrender; 置换，交出； |  |  |  |
| 5 | Handover of static equipment overhaul 静设备检修交出 |  |  | - Close the inlet and outlet valves of the equipment; 关闭设备进出口阀门； |  |  |  |
|  |  |  |  | - Close the equipment, pipes, and measuring instrument valves associated with the equipment; 关闭与设备相关联的设备、管道、测量仪表阀门； |  |  |  |
|  |  |  |  | - Drain and relieve pressure and replace; 排空泄压，置换； |  |  |  |
|  |  |  |  | - Open the manhole, check that the conditions for maintenance are met, and hand it over.打开人孔，检测具备检修条件，交出。 |  |  |  |
| 6 | Valve overhaul 阀门检修 |  |  | - Open the valve bypass;将阀门旁路打开； |  |  |  |
|  |  |  |  | - Close the front and rear stop valves of the valve, and cut off the valve;关闭该阀门前后截止阀，切除该阀门； |  |  |  |
|  |  |  |  | - Prepare suitable tools and instruments;准备合适的工器具； |  |  |  |
|  |  |  |  | - Loosen the valve flange bolts to relieve the pressure; 松动阀门法兰螺栓，泄压； |  |  |  |
|  |  |  |  | - Remove the valve for inspection.将阀门拆出检修。 |  |  |  |
| 7 | Air compressor switch 空压机切换 |  |  | - Apply for air compressor switching operation; 申请进行空压机切换操作； |  |  |  |
|  |  |  |  | - Check before starting the standby air compressor; 备用空压机启动前检查； |  |  |  |
|  |  |  |  | - Start the standby air compressor;启动备用空压机； |  |  |  |
|  |  |  |  | - Open the outlet valve of the standby air compressor; 打开备用空压机出口阀； |  |  |  |
|  |  |  |  | - Load the spare air compressor;加载备用空压机； |  |  |  |
|  |  |  |  | - Unload the air compressor that is planned to be out of service;卸载计划停运的空压机； |  |  |  |
|  |  |  |  | - Stop the air compressor that is planned to be out of service; 停止计划停用的空压机； |  |  |  |
|  |  |  |  | - Close the outlet valve of the stopped compressor. 关闭停运压缩机出口阀。 |  |  |  |
| 8 | Pump switching 水泵切换 |  |  | - Check the standby pump before starting; 备用泵启动前检查； |  |  |  |
|  |  |  |  | - The standby pump starts;备用泵启动； |  |  |  |
|  |  |  |  | - Switch operation of main and standby pumps;主、备机泵运行切换； |  |  |  |
|  |  |  |  | - The main pump is out of service.主泵停运。 |  |  |  |
| 9 | Pump lubricant change 机泵更换润滑油 |  |  | - The pump is out of service;机泵停运； |  |  |  |
|  |  |  |  | - Turn the on-site operation column to "COS" mode; 现场操作柱转“COS”模式； |  |  |  |
|  |  |  |  | - Empty and replace waste oil;将废油放空并置换； |  |  |  |
|  |  |  |  | - Replace the lubricating oil according to the contents of the five-set table;按五定表内容更换润滑油； |  |  |  |
|  |  |  |  | - The pump resumes operation.机泵恢复运行。 |  |  |  |
| 10 | Expander switch 膨胀机切换 |  |  | - Apply for expander switching operation;申请进行膨胀机切换操作； |  |  |  |
|  |  |  |  | - Check the standby expander before starting, and measure the dew point;备用膨胀机启动前检查，测露点； |  |  |  |
|  |  |  |  | - Run the expander to reduce the load;运行膨胀机降负荷； |  |  |  |
|  |  |  |  | - Start the standby expander;启动备用膨胀机； |  |  |  |
|  |  |  |  | - Slowly unload and operate the expander, and load the spare expander at the same time; 缓慢卸载运行膨胀机，同时加载备用膨胀机； |  |  |  |
|  |  |  |  | - Stop the expander that is planned to be out of service; 停止计划停用的膨胀机； |  |  |  |
|  |  |  |  | - Adjust the load of the operating expander;调节运行膨胀机负荷； |  |  |  |
|  |  |  |  | - Isolate and shut down the expander;隔离停运膨胀机； |  |  |  |
|  |  |  |  | - Shut down the expander to return the temperature. 停运膨胀机回温。 |  |  |  |
| 11 | Dryer switch 干燥器切换 |  |  | - Check before starting the standby dryer;备用干燥器启动前检查； |  |  |  |
|  |  |  |  | - "Pause" the running dryer;运行的干燥器“暂停”； |  |  |  |
|  |  |  |  | 3 Fully open the outlet valve of the standby dryer; 全开备用干燥器的出口阀门； |  |  |  |
|  |  |  |  | - Slowly fully open the inlet valve of the standby dryer, and at the same time slowly fully close the inlet valve of the operating dryer;缓慢全开备用干燥器进口阀，同时缓慢全关运行干燥器的进口阀； |  |  |  |
|  |  |  |  | - Start the sequence control of the dryer ready to start; 启动准备启动的干燥器顺控； |  |  |  |
|  |  |  |  | - Close the outlet valve of the desiccant dryer. 关闭停运干燥器的出口阀门。 |  |  |  |

Descriptions: 1. List major jobs for each process unit (area) which is considered as the assessment unit.

说明：1以工艺单元（区域）为评价单元，列出单元主要作业活动。

2 Simple jobs need to be more than 4 steps, while complex jobs need to be more than 8 steps.简单作业活动要求分为4步以上，复杂作业活动要求分为8步以上。

3 This form is used for JHA risk assessment and shall be consistent with the No. in JHA form.此表用于JHA法风险评价，JHA表编号要对应起来。