|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Equipment and Facilities List 设备设施清单模板 |  |  |
|  | Template No. | HYBN-T7-08-2008-2024-2 | Page 1 of 7 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment and Facilities List 设备设施清单 |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T6- -2008- -2024 |  |  | Page 2 of 2 |  |
| Department 部门 |  |  | Utilities Department 公用工程部 |  | Name of unit (area) 单元（区域）名称 |  |  | Air separation and compression unit 空分空压装置 |
| Reviewed by 审核人 |  |  | Shishuigen 施水根 |  | Date 日期 | MM DD, YYYY 2025年 2月12日 |  |  |
| S/N 序号 | No. of SCL form SCL表编号 |  | Name of equipment/facility 设备设施名称 |  | Inspection item 检查项目 |  |  |  |
| 1 | 001 |  | Self-cleaning filters 自洁式过滤器 （A/B/C/D/E/F） |  | 1.Whether the filter pressure difference meets the requirements过滤器压差是否符合要求 2.Whether the blowback pressure meets the requirements反吹气压力是否符合要求 3.Whether the quality of the filter cartridge meets the requirements滤筒质量是否符合要求 4.Is the blowback solenoid valve normal反吹电磁阀是否正常 |  |  |  |
| 2 | 002 |  | Air compressors 空压机 （K001A/B/C/D/E/F） |  | 1.Compressor lubricant oil pressure, oil level, oil quality, tank vacuum is in line with the requirements压缩机润滑油油压，油位，油质，油箱真空度是否符合要求 2.Is the compressor vibration in line with the requirements压缩机振动是否符合要求 3.Is the compressor bearing temperature in line with the requirements压缩机轴承温度是否符合要求 4.Compressor operation sound is in line with the requirements压缩机运行声音是否符合要求 5.Compressor interstage drainage is normal 压缩机级间疏水是否正常 6.Compressor water return sight glass and oil return sight glass are normal 压缩机回水视镜及回油视镜是否正常 |  |  |  |
| 3 | 003 |  | Instrument air dryer 仪表风干燥器 （A/B/C） |  | 1.Is the instrument air dryer drain valve normal仪表风干燥器排水阀是否正常 2.Whether the instrument air dryer valve is faulty仪表风干燥器阀门是否故障 3.Whether the instrument air dryer electric heater is faulty仪表风干燥器电加热器是否故障 4.Instrument air dryer heat exchanger water circuit, gas circuit is normal仪表风干燥器换热器水路、气路是否正常 |  |  |  |
| 4 | 004 |  | Buffer tanks (instrument air, plant air, instrument air accident air) 缓冲罐（仪表风、工厂风、仪表空气事故风） |  | 1.Buffer tank remote transmission, field pressure gauge is normal缓冲罐远传、现场压力表是否正常 2.Is the buffer tank safety valve normal缓冲罐安全阀是否正常 3.The bottom of the buffer tank is discharged properly缓冲罐底部排放是否正常 |  |  |  |
| 5 | 005 |  | Instrument air booster compressor 仪表空气增压压缩机 （K002） |  | 1.Instrument air booster compressor bearing temperature is normal仪表风增压压缩机轴承温度是否正常 2.Instrument air booster compressor operation sound is normal仪表风增压压缩机运行声音是否正常 3.Instrument air booster compressor inlet valve is normal仪表风增压压缩机进气阀门是否正常 4.Instrument gas booster compressor oil level, oil pressure, oil temperature is normal 仪表气增压压缩机油位、油压、油温是否正常 5.Instrument gas booster compressor cooling water return is normal仪表气增压压缩机冷却水回水是否正常 |  |  |  |
| 6 | 006 |  | Centrifugal water pumps (ambient, cryogenic, saline sewage, oily sewage, circulating water pumps, chilled water pumps) 离心水泵（常温、低温、含盐污水、含油污水、循环水泵、冷冻水泵） |  | 1.Whether the vibration of the centrifugal pump meets the requirements离心泵振动是否符合要求 2.Whether the bearing temperature of the centrifugal pump meets the requirements离心泵轴承温度是否符合要求 3.whether the centrifugal pump running sound is normal离心泵运行声音是否正常 4.Is the outlet pressure, current and flow rate of the centrifugal pump normal离心泵出口压力、电流及流量是否正常 5.shaft seal flushing cooling water is normal 轴封冲洗冷却水是否正常 |  |  |  |
| 7 | 007 |  | Molecular sieve adsorber 分子筛吸附器 （MS001A/B） |  | 1.Whether the molecular sieve adsorber valve and program control is faulty分子筛吸附器阀门及程控是否故障 2.Whether the molecular sieve adsorber electric heater is faulty分子筛吸附器电加热器是否故障 3.Whether the molecular sieve adsorber inlet hydrophobic normal分子筛吸附器入口疏水是否正常 4.Whether the molecular sieve adsorber penetrates分子筛吸附器是否穿透 |  |  |  |
| 8 | 008 |  | Pressurized turbine expander 增压透平膨胀机 （ET001A/B） |  | 1.Whether the bearing vibration of the pressurized turbine expander is high增压透平膨胀机轴承振动是否高 2.Is the bearing temperature of the pressurized turbine expander normal增压透平膨胀机轴承温度是否正常 3.Whether the oil temperature, oil pressure, oil level and vacuum of the oil tank of the pressurized turbine expander are normal 增压透平膨胀机油温，油压，油位，油箱真空度是否正常 4.Pressurized turbine expander seal gas normal增压透平膨胀机密封气正常 5.Normal temperature of expansion side inlet of pressurized turbine expander增压透平膨胀机膨胀侧进口温度正常 6.Expander oil return is normal膨胀机回油正常 7.Is the expander nozzle heater normal膨胀机喷嘴加热器是否正常 |  |  |  |
| 9 | 009 |  | Steam Injection Evaporator 蒸汽喷射蒸发器 |  | 1.Steam injection evaporator steam line hydrophobic is normal 蒸汽喷射蒸发器蒸汽管线疏水是否正常 2.Steam injection evaporator into the liquid line valve is normal蒸汽喷射蒸发器进液管线阀门是否正常 3.Whether the steam injection evaporator base is normal蒸汽喷射蒸发器基础是否正常 |  |  |  |
| 10 | 010 |  | Liquid nitrogen atmospheric pressure storage tank SV-001 液氮常压贮槽SV-001 |  | 1.Is the pressure of liquid nitrogen atmospheric pressure storage tank normal液氮常压贮槽压力是否正常 2.Whether the interlayer sealing gas pressure of the liquid nitrogen atmospheric storage tank is normal 液氮常压贮槽夹层密封气压力是否正常 3.Liquid nitrogen atmospheric pressure storage tank breathing valve is normal液氮常压贮槽呼吸阀是否正常 4.Liquid nitrogen atmospheric pressure storage tank level is normal液氮常压贮槽液位是否正常 5.Is there any phenomenon such as icing and sand running on the wall of the liquid nitrogen atmospheric storage tank?液氮常压贮槽壁是否有结冰、跑砂等现象 |  |  |  |
| 11 | 011 |  | Liquid nitrogen pump (low pressure NP-001A/B, medium pressure NP-003A/B, charging NP-002) 液氮泵（低压NP-001A/B、中压NP-003A/B、充车NP-002） |  | 1.Is the bearing temperature of liquid nitrogen pump normal液氮泵轴承温度是否正常 2.Is the outlet pressure and flow rate of liquid nitrogen pump normal液氮泵出口压力及流量是否正常 3.Liquid nitrogen pump mixture vent frost (icing) normal液氮泵混合气排放口结霜（结冰）正常 |  |  |  |
| 12 | 012 |  | Water bath vaporizer (low pressure WH-001, medium pressure WH-002) 水浴式汽化器（低压WH-001、中压WH-002） |  | 1.Water bath type vaporizer steam, desalination water pressure, temperature is normal 水浴式汽化器蒸汽、脱盐水压力、温度是否正常 2.The water level and water temperature in the water bath type vaporizer are normal水浴式汽化器内水位、水温均是否正常 3.Water bath type vaporizer steam trapping is normal水浴式汽化器蒸汽疏水是否正常 4.Water bath vaporizer for leakage水浴式汽化器是否泄露 |  |  |  |
| 13 | 013 |  | Nitrogen booster compressor 氮气增压压缩机 （K003A/B） |  | 1.Is the bearing temperature of the nitrogen booster compressor normal氮气增压压缩机轴承温度是否正常 2.Nitrogen booster compressor operation sound is normal氮气增压压缩机运行声音是否正常 3.Nitrogen booster compressor inlet valve is normal氮气增压压缩机进气阀门是否正常 4.Nitrogen booster compressor oil level, oil pressure, oil temperature is normal 氮气增压压缩机油位、油压、油温是否正常 5.Nitrogen booster compressor cooling water return is normal氮气增压压缩机冷却水回水是否正常 |  |  |  |
| 14 | 014 |  | Medium pressure liquid nitrogen storage tank 中压液氮贮槽 SV002A/B |  | 1.Medium pressure liquid nitrogen vacuum storage tank pressure is normal 中压液氮真空储罐压力是否正常 2.Whether the vacuum level of medium pressure liquid nitrogen vacuum storage tank is normal中压液氮真空储罐真空度是否正常 3.Medium pressure liquid nitrogen vacuum storage tank safety valve, rupture disc is normal 中压液氮真空储罐安全阀、爆破片是否正常 4.Medium pressure liquid nitrogen vacuum storage tank level is normal中压液氮真空储罐液位是否正常 5.Medium pressure liquid nitrogen vacuum storage tank wall whether there is icing and other phenomena中压液氮真空储罐壁是否有结冰等现象 |  |  |  |
| 15 | 015 |  | Pre-cooling refrigeration units 预冷冷冻机组 RU001A/B |  | 1.Is the inlet and outlet pressure of the refrigeration unit normal冷冻机组进出口压力是否正常 2.Is the inlet and outlet temperature of the refrigeration unit normal冷冻机组进出口温度是否正常 3.Is the chilled water flowing properly冷冻水流量是否正常 4.Is the sound of compressor operation of refrigerating unit abnormal冷冻机组压缩机运转声音是否异常 |  |  |  |
| 16 | 016 |  | Air-temperature vaporizer (low pressure AH-001, medium pressure AH-002) 空温式汽化器（低压AH-001、中压AH-002 |  | 1.Air-temperature vaporizer safety accessories are in order空温式汽化器安全附件是否正常 2.Whether the outlet pressure and temperature of air-temperature vaporizer are abnormal空温式汽化器出口压力、温度是否异常 3.Severe icing on the surface of air-temperature vaporizer空温式汽化器表面结冰情况是否严重 |  |  |  |

Notes:

说明:

1 List key equipment, major rotary equipment, pressure vessels and pressure pipelines inside each process unit (area) which is considered the assessment unit.

以工艺单元（区域）为评价单元，列出单元内关键设备、主要动设备、压力容器、压力管线等。

2 List major equipment and facilities, such as reactor, tower, heat exchanger and compressor.

列出主要设备设施，如反应器、塔、换热器、压缩机等。

3 This form is used for SCL risk assessment and shall be consistent with the No. in SCL form.

此表用于SCL法风险评价，SCL表编号要对应起来。