|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表模板 |  |  |
|  | Template No. | HYBN-T7-08-2003-2024-2 | Page 1 of 2 |

The following is the body of the template.

以下为模板正文。

|  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety Checklist (SCL) Record Form 安全检查表（SCL）分析记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Record No. |  | HYBN-T7-08-2003-2024-2 |  |  |  |  |  |  |  | Page 2 of 2 |  |  |  |  |
| No.: 编号： |  | Unit: Utilities Department 单位：公用工程部 |  | Members of SCL analysis group:Zhang Haoran, Yu Yapeng, Zhang Yunbo, Deng Wentao, Liangbing、Zhang Feipeng, Chen Liao, Zhang Wenjun, Zhao Yu And team members SCL分析小组人员：张浩然、于亚鹏、张云波、邓文涛、梁斌、张飞鹏、陈聊、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Name of unit (area)/process flow:ASU & ACU 单元（区域）/工艺过程名称：空分空压 |  |  |  | Unit/equipment/facility: compressor, expander, molecular sieve adsorber, cryogenic liquid nitrogen pump, liquid nitrogen storage tank, medium pressure liquid nitrogen storage tank, traveling crane, air-cooling tower, water-cooling tower, instrument air storage tank, accident nitrogen storage tank 装置/设备/设施：压缩机、膨胀机、分子筛吸附器、低温液氮泵、液氮储罐、中压液氮储罐、行车、空冷塔、水冷塔、仪表风储罐、事故氮储罐 |  |  |  |  |  |  |  | Reviewed by:WenJiancheng 审核人：温建成 |  |  |  |  |
| S/N 序号 | Inspection item 检查项目 |  | Inspection standard 检查标准 |  | Major accident consequences 主要事故后果 | Former occurrence frequency 以往发生频率 | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 | Self-cleaning filter operation inspection 自洁式过滤器运行检查 |  | Whether the filter pressure difference meets the requirements 过滤器压差是否符合要求 |  | Reduced air intake to the air compressor 空压机进气量降低 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the back blow air pressure meets the requirements 反吹气压力是否符合要求 |  | Incomplete back-blowing of dust on the filter cartridge and increased differential pressure 滤筒上灰尘反吹不彻底，压差增大 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the quality of the filter cartridge meets the requirements 滤筒质量是否符合要求 |  | Broken filter cartridge 滤筒破损 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Is the blowback solenoid valve normal? 反吹电磁阀是否正常 |  | Some cartridges cannot be back blown 部分滤筒无法反吹 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 | Compressor operation inspection 压缩机运行检查 |  | Whether the compressor lubricating oil pressure, oil level, oil quality, and oil tank vacuum meet the requirements 压缩机润滑油油压，油位，油质，油箱真空度是否符合要求 |  | Bearing burning 轴承烧瓦 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the compressor vibration meets the requirements 压缩机振动是否符合要求 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the compressor bearing temperature meets the requirements 压缩机轴承温度是否符合要求 |  | Deformation of the bearing, damage to the bearing bush, damage to the compressor 轴承变形，轴瓦损伤，损坏压缩机 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the compressor running sound meets the requirements 压缩机运行声音是否符合要求 |  | Compressor impeller, bearing damage 压缩机叶轮、轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the drainage between compressor stages normal? 压缩机级间疏水是否正常 |  | Compressor impeller damage 压缩机叶轮损伤 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the compressor return water sight glass and oil return sight glass are normal 压缩机回水视镜及回油视镜是否正常 |  | Poor heat transfer efficiency; bearing damage 换热效率不佳；轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 | Instrument air dryer operation inspection 仪表风干燥器运行检查 |  | Whether the drain valve of the instrument air dryer is normal 仪表风干燥器排水阀是否正常 |  | Increased water content and higher instrument gas dew point 含水增大，仪表气露点升高 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the valve of the instrument air dryer is faulty 仪表风干燥器阀门是否故障 |  | Short or interrupted supply of instrument air, plant air 仪表气、工厂风短供或断供 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | No failure of the electric heater of the instrument air dryer 仪表风干燥器电加热器无故障 |  | The electric heater is dry; the dryer is not completely regenerated 电加热器干烧；干燥器再生不彻底 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 | Buffer tank operation inspection 缓冲罐运行检查 |  | Whether the remote transmission of the buffer tank and the on-site pressure gauge are normal 缓冲罐远传、现场压力表是否正常 |  | Affects pressure regulation, causing the instrument air booster compressor to trip 影响压力调节，导致仪表风增压压缩机跳车 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the buffer tank safety valve is normal 缓冲罐安全阀是否正常 |  | Buffer tank overpressure cannot be relieved 缓冲罐超压无法泄压 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the bottom discharge of the buffer tank normal? 缓冲罐底部排放是否正常 |  | No way to verify the quality of the product 无法验证产品的质量 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 5 | Instrument air booster compressor operation inspection 仪表风增压压缩机运行检查 |  | Whether the bearing temperature of the instrument air booster compressor is normal 仪表风增压压缩机轴承温度是否正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  | Is the operating sound of the instrument air booster compressor normal? 仪表风增压压缩机运行声音是否正常 |  | Packing damage; motor bearing damage; turbocharger unit damage 填料损伤；电机轴承损伤；增压机机组损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the inlet valve of the instrument air booster compressor normal? 仪表风增压压缩机进气阀门是否正常 |  | Insufficient pumping volume and low outlet pressure 打气量不足，出口压力低 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the oil level, oil pressure and oil temperature of the instrument gas booster compressor are normal 仪表气增压压缩机油位、油压、油温是否正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the return water of the cooling water of the instrument gas booster compressor is normal 仪表气增压压缩机冷却水回水是否正常 |  | Packing damage, cylinder damage 填料损伤，气缸损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 6 | Centrifugal pump operation inspection 离心水泵运行检查 |  | Whether the vibration of the centrifugal pump meets the requirements 离心泵振动是否符合要求 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the bearing temperature of the centrifugal pump meets the requirements 离心泵轴承温度是否符合要求 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the running sound of the centrifugal pump normal? 离心泵运行声音是否正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure, current and flow of the centrifugal pump are normal 离心泵出口压力、电流及流量是否正常 |  | Affects production operations; damages impellers 影响生产运行；损伤叶轮 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the shaft seal flushing cooling water normal? 轴封冲洗冷却水是否正常 |  | Affects production operations; damages impellers 轴封损坏 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 7 | Molecular sieve adsorber operation inspection 分子筛吸附器运行检查 |  | Whether the molecular sieve adsorber valve is faulty 分子筛吸附器阀门是否故障 |  | Molecular sieve cis-controlled stop, adsorption timeout 分子筛顺控停止，吸附超时 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the electric heater of molecular sieve adsorber is faulty 分子筛吸附器电加热器是否故障 |  | Dry burning of electric heater or incomplete regeneration of molecular sieve 电加热器干烧或分子筛再生不彻底 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the inlet hydrophobicity of the molecular sieve adsorber normal? 分子筛吸附器入口疏水是否正常 |  | Watery inlet air to the tower and excessive CO2 进塔空气带水，二氧化碳超标 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the molecular sieve adsorber penetrates 分子筛吸附器是否穿透 |  | Watery inlet air to the tower and excessive CO2 进塔空气带水，二氧化碳超标 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 8 | Booster turbine operation inspection 增压透平膨胀机运行检查 |  | 增压透平膨胀机轴承振动是否正常 Whether the bearing vibration of turbocharger expander is normal |  | Causes bearing damage, expander trip 导致轴承损伤，膨胀机跳车 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the bearing temperature of the turbocharger expander is normal 增压透平膨胀机轴承温度是否正常 |  | Bearing damage, expander trip 轴承损伤，膨胀机跳车 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the oil temperature, oil pressure, oil level, and tank vacuum of the turbocharger are normal 增压透平膨胀机油温，油压，油位，油箱真空度是否正常 |  | Bearing damage; expander trip 轴承损伤；膨胀机跳车 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The sealing gas of the turboexpander is normal 增压透平膨胀机密封气正常 |  | Expander trip; 膨胀机跳车 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | The inlet temperature of the expansion side of the turboexpander is normal 增压透平膨胀机膨胀侧进口温度正常 |  | Impeller damage to the expander 膨胀机叶轮损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Normal oil return of the expander 膨胀机回油正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 9 | Steam injection evaporator operation inspection 蒸汽喷射蒸发器运行检查 |  | Is the steam drain of the steam jet evaporator normal? 蒸汽喷射蒸发器蒸汽管线疏水是否正常 |  | Water buildup in steam pipes, causing water hammer in pipes 蒸汽管道积水，造成管道水锤 | Occasionally 偶尔 | 3 | 2 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the valve of the liquid inlet pipeline of the steam jet evaporator is normal 蒸汽喷射蒸发器进液管线阀门是否正常 |  | Valve leaks/freezes, won't close/drain 阀门泄漏/结冰，无法关闭/排液 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Is the foundation of the steam jet evaporator normal? 蒸汽喷射蒸发器基础是否正常 |  | Frost cracking of the foundation 基础冻裂 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 10 | Liquid nitrogen atmospheric pressure storage tank operation inspection 液氮常压贮槽运行检查 |  | Is the pressure of the liquid nitrogen atmospheric storage tank normal? 液氮常压贮槽压力是否正常 |  | Liquid nitrogen atmospheric pressure storage tank overpressure or aspiration of atmosphere 液氮常压贮槽超压或吸入大气 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the interlayer sealing gas pressure of the liquid nitrogen atmospheric storage tank is normal 液氮常压贮槽夹层密封气压力是否正常 |  | The pearlite sand is damp and icy, and the cold retention becomes poor 珠光砂受潮结冰，保冷变差 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the breathing valve of the liquid nitrogen atmospheric storage tank is normal 液氮常压贮槽呼吸阀是否正常 |  | Liquid nitrogen atmospheric tank overpressure cannot be relieved; air cannot be drawn in at low pressure 液氮常压贮槽超压无法进行泄压；低压时无法吸入空气 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the liquid level of the liquid nitrogen atmospheric storage tank is normal 液氮常压贮槽液位是否正常 |  | Back-up system not operational 后备系统无法正常投入 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether there is icing or sand running on the wall of liquid nitrogen atmospheric storage tank 液氮常压贮槽壁是否有结冰、跑砂 |  | Rupture of storage tank wall 贮罐壁破裂 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 11 | Liquid nitrogen pump operation inspection 液氮泵运行检查 |  | Is the bearing temperature of the liquid nitrogen pump normal? 液氮泵轴承温度是否正常 |  | Bearing damage; motor damage 轴承损伤；电机损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the outlet pressure and flow rate of the liquid nitrogen pump are normal 液氮泵出口压力及流量是否正常 |  | Affects the stability of the backup system; causes damage to the pump impeller 影响备用系统的稳定；造成泵叶轮受损 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Frosting (icing) at the mixed gas discharge port of the liquid nitrogen pump is normal 液氮泵混合气排放口结霜（结冰）正常 |  | Seal damage; low temperature medium hurts people 密封损伤；低温介质伤人 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 12 | Water bath vaporizer operation inspection 水浴式汽化器运行检查 |  | Whether the steam and desalinated water pressure and temperature of the water bath evaporator are normal 水浴式汽化器蒸汽、脱盐水压力、温度是否正常 |  | Nitrogen supply cut off, pipes freeze and crack 氮气断供，管道结冰冻裂 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the water level and water temperature in the water bath vaporizer are normal 水浴式汽化器内水位、水温均是否正常 |  | Nitrogen supply cut off, pipes freeze and crack 氮气断供，管道结冰冻裂 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Is the steam drainage of the water bath vaporizer normal? 水浴式汽化器蒸汽疏水是否正常 |  | Steam line water hammer; dislodged vaporizer nozzle 蒸汽管道水锤；汽化器喷头脱落 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the water bath vaporizer leaks 水浴式汽化器是否泄露 |  | Steam burns; liquid nitrogen frostbite 蒸汽烫伤；液氮冻伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 13 | Nitrogen booster compressor operation inspection 氮气增压压缩机运行检查 |  | Whether the bearing temperature of the nitrogen booster compressor is normal 氮气增压压缩机轴承温度是否正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Is the operating sound of the nitrogen booster compressor normal? 氮气增压压缩机运行声音是否正常 |  | Packing damage; motor bearing damage; booster unit damage 填料损伤；电机轴承损伤；增压机机组损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the inlet valve of the nitrogen booster compressor is normal 氮气增压压缩机进气阀门是否正常 |  | Insufficient pumping volume and low outlet pressure 打气量不足，出口压力低 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  | Whether the oil level, oil pressure and oil temperature of the nitrogen booster compressor are normal 氮气增压压缩机油位、油压、油温是否正常 |  | Bearing damage 轴承损伤 | Rarely happens 很少发生 | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  | Whether the return water of the cooling water of the nitrogen booster compressor is normal 氮气增压压缩机冷却水回水是否正常 |  | Packing damage, cylinder damage 填料损伤，气缸损伤 | Rarely happens 很少发生 | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the control measures that need to be maintained or increased from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要保持或增加的控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |