|  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |
| --- | --- | --- | --- |
|  | Template of Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表模板 |  |  |
|  | Template No. | HYBN-T7-08-2002-2024-2 | Page 1 of 4 |

The following is the body of the template.

以下为模板正文。

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 1 of 2 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Hand over maintenance of moving equipment at room temperature 常温动设备检修交出 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Equipment outage, power failure, listing 设备停运，断电，挂牌 |  | The power supply was cut off, and the pump turned suddenly 断错电源，泵突然转动 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | Ⅱ |  |  |  |  |  |  |
| 2 |  | Close the equipment inlet and outlet valves 关闭设备进出口阀门 |  | Excessive force 用力过猛 |  | Object hit damage 物体打击伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | Ⅱ |  |  |  |  |  |  |
| 3 |  | Evacuate the medium in the equipment and relieve the pressure 排空设备内介质，泄压 |  | The pressure is not released, and the pressurized medium hurts personnel 压力未泄，带压介质伤人 |  | Personnel injury 人员受伤 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | Ⅱ |  |  |  |  |  |  |
| 4 |  | Replace, handover 置换，交出 |  | Not completely replaced 未置换彻底 |  | Asphyxia 窒息 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | Ⅱ |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 1 of 3 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Hand over maintenance of low temperature moving equipment 低温动设备检修交出 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Equipment outage, power failure, listing 设备停运，断电，挂牌 |  | Wrong power supply, the device suddenly rotates 断错电源，设备突然转动 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Close the inlet and outlet valves and return valves 关闭进出口阀门、回流阀门 |  | Excessive force 用力过猛 |  | Object hit damage 物体打击伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Open the pump body to drain 打开泵体排液 |  | Liquid nitrogen discharge 液氮排放 |  | Personnel frostbite, pump foundation frostbite 人员冻伤，泵基础冻裂 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Liquid nitrogen vaporizes into nitrogen 液氮蒸发成氮气 |  | Personnel asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | Open the heating gas valve, reheat, and hand over 打开加温气阀门，复温，交出 |  | Nitrogen venting 氮气排放 |  | Staff asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Excessive heating 加温气过大 |  | Pump shaft damage 泵轴损伤 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 1 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Cryogenic liquid nitrogen pump cold pump 低温液氮泵冷泵 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Dry gas replacement, measuring dew point 干燥气体置换，测露点 |  | Nitrogen emission 氮气排放 |  | Personnel asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 2 |  | Open the reflux valve of the pump body, air-cooled 打开泵体回流阀，气冷 |  | Nitrogen emission 氮气排放 |  | Personnel asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Bearing damage 轴承损伤 |  | Equipment failure 设备故障 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Slightly open the pump inlet valve, liquid cooling 微开泵进口阀门，液冷 |  | Liquid nitrogen discharge 液氮排放 |  | Personnel asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  |  |  | Personnel frostbite 人员冻伤 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | If there is continuous liquid outflow from the discharge at the high point of the pump, the cold pump ends 泵高点排放处有连续的液体流出，则冷泵结束 |  | Liquid nitrogen leak 液氮泄漏 |  | Personnel asphyxiation 人员窒息 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  |  |  | Personnel frostbite 人员冻伤 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Valve overhaul 阀门检修 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Bypass the valve 将阀门旁路打开 |  | Improper use of the "F" wrench “F”扳手使用不当 |  | Personal injury 人身伤害 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Close the stop valve before and after the valve and remove the valve. 关闭该阀门前后截止阀，切除该阀门 |  | Improper use of the "F" wrench “F”扳手使用不当 |  | Personal injury 人身伤害 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Preparation of suitable work tools. 准备合适的工器具 |  | Wrong choice of special tools 特殊工具选用错误 |  | Static electricity sparks and explosions 产生静电火花发生爆炸 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Loosen flange bolts to relieve pressure. 松动法兰螺栓，泄压 |  | Mishandling during loosening of bolts 松动螺栓过程中操作失误 |  | Personal injury 人身伤害 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Incomplete pressure relief 泄压不彻底 |  | Personal injury 人身伤害 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Removal of the valve for servicing 将阀门拆出检修 |  | Valve smashes straight down 阀门直接砸落 |  | Personal injury 人身伤害 |  | Rarely Occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Switch dryer 干燥器切换 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Inspection before starting the backup dryer 备用干燥器启动前检查 |  | Inspection is not in place 检查不到位 |  | The dryer does not start 干燥器未启动 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Running dryer "pause" 运行的干燥器“暂停” |  | Incomplete dryer regeneration 干燥器再生不彻底 |  | Adsorbent powdering 吸附剂粉化 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Fully open the outlet valve of the standby dryer 全开备用干燥器的出口阀门 |  | Irregular use of " F " wrench “F”扳手使用不规范 |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 4 |  | Slowly fully open the inlet valve of the standby dryer, and at the same time slowly fully close the inlet valve of the running dryer 缓慢全开备用干燥器进口阀，同时缓慢全关运行干燥器的进口阀 |  | Fluctuation of instrument air pipe network 仪表气管网波动 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Start the sequence of the dryer ready to start 启动准备启动的干燥器顺控 |  | Turn on the dryer by mistake 误将干燥器启错 |  | Electric heater dryer burnt 电加热器干烧 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 6 |  | Close the outlet valve of the out-of-service dryer 关闭停运干燥器的出口阀门 |  | Irregular use of " F " wrench “F”扳手使用不规范 |  | Personnel injury 人身伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Change the lubricating oil of the pump 机泵更换润滑油 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Pump out of service 机泵停运 |  | After the pump stops, the interlock starts automatically 泵停后联锁自启 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | On-site operation column turns to " COS " mode 现场操作柱转“COS”模式 |  | The oil change starts when the operating column is not in the " COS " state 操作柱没在“COS”状态就开始换油 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Drain and replace waste oil 将废油放空并置换 |  | Dirt enters the lubricating oil tank 杂物进入润滑油箱 |  | Damaged equipment 损坏设备 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Change the lubricating oil according to the contents of the five-set table 按五定表内容更换润滑油 |  | Wrong type of lubricant 润滑油型号错误 |  | Damaged equipment 损坏设备 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Machine pump resumes operation 机泵恢复运行 |  | Motor leakage 电机漏电 |  | Electric shock 人员触电 |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Static equipment overhaul handover 静设备检修交出 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Close the equipment inlet and outlet valves 关闭设备进出口阀门 |  | Wrong valve closed or valve leaking inside 关错阀门或阀门内漏 |  | Suffocation 窒息 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Closure of equipment, piping, measuring instrument valves associated with the equipment 关闭与设备相关联的设备、管道、测量仪表阀门 |  | Not effectively isolated from associated equipment piping 未与相关设备管道 有效隔绝 |  | Suffocation 窒息 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 3 |  | Evacuate to relieve pressure and replace 排空泄压，置换 |  | Failure to relieve pressure as required or gauge error 未按要求泄压或压力表误差 |  | Personal injury 人身伤害 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Displacement failure 置换不合格 |  | Suffocation 窒息 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Open manhole, test for serviceability, hand over 打开人孔，检测具备检修条件，交出 |  | The sampling location is not typical 采样位置不具代表性 |  | Fire, explosion, poisoning and suffocation 着火、爆炸、中毒、窒息 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Switch air compressor 空压机切换 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Application for air compressor switching operation 申请进行空压机切换操作 |  | Grid fluctuations 电网波动 |  | Plant shutdown 装置停工 |  | Occasionally 偶尔 |  | 3 | 3 | 9 | III | Contact the electrical dispatcher and dispatcher in advance before starting the machine 启机前提前联系电调及调度， |  | 2 | 3 | 6 | II |
| 2 |  | Pre-start inspection of the backup air compressor 备用空压机启动前检查 |  | Inadequate inspection 检查不到位 |  | Air compressor not started or damaged 空压机未启动或受损 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Start the backup air compressor 启动备用空压机 |  | 空压机振动、温度异常且压缩机有异响 Air compressor vibration, temperature abnormal and compressor noise |  | Equipment damage 设备损坏 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 4 |  | Open the outlet valve of the standby air compressor 打开备用空压机出口阀 |  | Irregular use of the "F" wrench “F”扳手使用不规范 |  | Personal injury 人身伤害 |  | Rarely occur 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 5 |  | Loading backup air compressor 加载备用空压机 |  | Operating compressor unloading or fluctuating air separation conditions 运行压缩机卸载或空分工况波动 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 6 |  | Unloading of air compressors scheduled to be taken out of service 卸载计划停运的空压机 |  | Fluctuation of air separation working conditions 空分工况波动 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 7 |  | Discontinuation of air compressors scheduled to be taken out of service 停止计划停用的空压机 |  | Mistakenly stopping the compressor by mistake 误操作将压缩机错停 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Air compressor assist not started 空压机辅助未启动 |  | Shaft Vessel Damage 轴瓦损伤 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 8 |  | Close the outlet valve of the out-of-service compressor 关闭停运压缩机出口阀 |  | Irregular use of the "F" wrench “F”扳手使用不规范 |  | Personal injury 人身伤害 |  | Rarely occur 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Switching of expander 膨胀机切换 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Application for expander switching operation 申请进行膨胀机切换操作 |  | Fluctuation of air separation working conditions 空分工况波动 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Pre-start inspection of the standby expander 备用膨胀机启动前检查 |  | Inadequate inspection 检查不到位 |  | Expander not started or equipment damaged 膨胀机未启动或设备受损 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Running the expander to reduce the load 运行膨胀机降负荷 |  | Running expander trip 运行膨胀机跳车 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Start the backup expander 启动备用膨胀机 |  | Running expander trips or standby expander does not start successfully 运行膨胀机跳车或备用膨胀机未启动成功 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 5 |  | Slowly unload the operating expander and load the standby expander 缓慢卸载运行膨胀机，加载备用膨胀机 |  | Expansion machine shutdown 膨胀机停机 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 6 |  | Discontinuation of the planned shutdown of the expansion machine 停止计划停运膨胀机 |  | Mistaken operation stops expander by mistake 误操作将膨胀机错停 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 7 |  | Regulating the operating expander load 调节运行膨胀机负荷 |  | Expansion machine shutdown 膨胀机停机 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 8 |  | Isolation of out-of-service expanders 隔离停运膨胀机 |  | Irregular use of the "F" wrench “F”扳手使用不规范 |  | Personal injury 人身伤害 |  | Rarely occur 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 9 |  | Out of service expander to warm up 停运膨胀机回温 |  | Expander reversal 膨胀机反转 |  | Process accidents 工艺事故 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Clean the filter before the pump 清洗泵前过滤器 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | The pump is out of service, the power is cut off and the pump is listed, and the inlet and outlet valves of the pump are closed 泵停运，断电挂牌，关闭泵进出口阀门 |  | The power supply was cut off, and the pump turned suddenly 断错电源，泵突然转动 |  | Mechanical damage 机械伤害 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Open guide vane to relief pressure discharge 开导淋泄压排放 |  | The pressure is not released, and the pressurized medium hurts personnel 压力未泄，带压介质伤人 |  | Personal injury 人员受伤 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Prepare suitable tools 准备合适的工器具 |  | Using the wrong tool 用错工具 |  | Object hit damage 物体打击伤害 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Loosen the filter bolts 松动过滤器螺栓 |  | Excessive force 用力过猛 |  | Object hit damage 物体打击伤害 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 5 |  | Remove the filter for cleaning 拆除过滤器清洗 |  | Scratched by the filter screen 被过滤器丝网划伤 |  | Finger cut 手指割伤 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Impurities splashed into the eyes 杂质溅入眼睛 |  | Eye injury 眼睛受伤 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 6 |  | Reinstall and open the pump inlet valve to test for leakage 回装，并打开泵进口阀门试漏 |  | Tighten the bolts too hard 紧螺栓过于用力 |  | Object hit damage 物体打击伤害 |  | Rarely occur 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Medium leakage 介质泄漏 |  | Pressurized media hurts personal 带压介质伤人 |  | Occasionally 偶尔 |  | 3 | 2 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Pump switching 水泵切换 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Inspection before starting the standby pump 备用泵启动前检查 |  | Inspection is not in place 检查不到位 |  | Equipment damage Mechanical damage 设备损害 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
| 2 |  | Standby pump start 备用泵启动 |  | Motor or bearing over temperature 电机或轴承超温 |  | Damaged motor or bearing 电机或轴承损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Large vibration of the pump 机泵振动大 |  | Equipment damage 设备损坏 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Exposed rotating parts 转动部位外露 |  | Mechanical damage 机械伤害 |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Main and standby pump operation switching 主、备机泵运行切换 |  | Pressure fluctuation of pipe network caused by switching of outlet valve of pump 机泵出口阀切换引起管网压力波动 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
| 4 |  | The main pump is out of service 主泵停运 |  | Misoperation shuts down the backup pump 误操作将备用泵停运 |  | Process accident 工艺事故 |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | Ⅱ |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Hengyi Industries Sdn Bhd 恒逸实业（文莱）有限公司 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job Hazard Analysis (JHA) Record Form 工作危害分析法 （JHA）记录表 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Record No. |  | HYBN-T7-08-2002-2024-2 |  |  |  |  |  |  |  |  |  |  | Page 2 of 4 |  |  |  |  |
| No. 编号: |  |  | Department 部门: |  | Utilities Department 公用工程部 |  |  | Unit 装置: |  | ASU & ACU 空分空压 |  |  |  |  |  | Analysis date：Feb 10,2025 分析日期：2025年2月10日 |  |  |  |  |
| Task 工作任务: |  |  | Liquid nitrogen filling and unloading operations 液氮装、卸车操作 |  | Members of analysis group 分析小组成员: |  | Zhang Haoran, Yu Yapeng, Deng Wentao, Zhang Yunbo, Chen Liao, Liangbing、Zhang Feipeng, Zhang Wenjun, Zhao Yu and team members 张浩然、于亚鹏、邓文涛、张云波、梁斌、陈聊、张飞鹏、张文俊、赵宇及班组成员 |  |  |  |  |  |  |  |  | Reviewed by 审核人： | WenJiancheng 温建成 |  |  |  |
| S/N 序号 |  | Operation steps 工作步骤 |  | Hazard source or potential incident 危险源或潜在事件 |  | Major accident consequences 主要事故后果 |  |  | Former occurrence frequency 以往发生频率 |  | Initial risk assessment 初始风险评价 |  |  |  | Measures to reduce and control risk 风险削减及控制措施 |  | Residual risk assessment 剩余风险评价 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | L | S | R | Level等级 |  |  | L | S | R | Level等级 |
| 1 |  | Prepare liquid nitrogen filling operation methods, operation tickets, liquid nitrogen tanker filling procedures and tanker inspection lists, and review, approve and publicize them 编制液氮充装操作法、操作票、液氮槽车充装流程及槽车检查清单，并审批、宣贯 |  | No relevant information on liquid nitrogen filling and unloading has been compiled. 未编制液氮装、卸相关资料 |  | Chaos at the liquid nitrogen filling and unloading site 液氮装、卸现场混乱 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 2 |  | Preparation before filling 充装前的准备 |  | No training provided to tank truck drivers and press operators 未对槽车司机、压运员进行培训 |  | Liquid nitrogen filling and unloading operations postponed 液氮装、卸作业延期 |  |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | There is no process training for filling operators and the operators have insufficient experience. 未对充装操作人员进行工艺培训，操作人员经验不足 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | No driving routes within the factory are specified 未规定厂内行车路线 |  | Roads are blocked and even traffic accidents occur 道路堵塞，甚至发生交通事故 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Failure to inspect tankers and vehicles or inadequate inspections (including drivers and transport operators) 未对槽车及车辆进行检查或检查不到位（含司机、压运员） |  | A quality incident or leak occurs 发生质量事件或泄漏事件 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Irregular labor protection attire (including antifreeze gloves, visors, etc.) 劳保着装不规范（含防冻手套、面屏等） |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Unreasonable selection of connecting hoses (including material and pressure level) 连接软管选择不合理（含材质、压力等级） |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 3 |  | Prepare various tools and equipment for filling and unloading 准备装、卸的各种工器具 |  | There is a problem with the tools 工器具存在问题 |  | mechanical damage 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| 4 |  | Liquid nitrogen filling operation 液氮充装作业 |  | The tank truck hit the cryogenic pipeline/equipment during reversing 槽车倒车过程中撞到低温管线/设备 |  | mechanical damage 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | II |  |  |  |  |  |  |
|  |  |  |  | During loading and unloading, the tanker slips and the tanker connection hose is disconnected 装、卸过程中，槽车滑动、造车连接软管断开 |  | mechanical damage 机械伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Tanks not analyzed before filling 充装前未对储罐进行分析 |  | quality event 质量事件 |  |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | Not replaced or purged before filling 充装前未置换、吹扫 |  | quality event 质量事件 |  |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
|  |  |  |  | During the cranking process of the cooling machine pump, the power was not cut off. 冷却机泵盘车过程中，未断电 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | The filling pressure is high and the tank car is leaking due to overpressure 充装压力高，槽车超压泄漏 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Overfill 过量充装 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
|  |  |  |  | Operators leave their jobs 操作人员离岗 |  | Manage vulnerabilities 管理漏洞 |  |  | Rarely happens 很少发生 |  | 2 | 2 | 4 | I |  |  |  |  |  |  |
| 5 |  | Inspection after filling 充装后的检查 |  | The operating valve is not closed 操作阀门未关闭 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | Frost on the outer wall of the tank truck/tank truck leakage 槽车外壁结霜/槽车泄漏 |  | personal injury 人身伤害 |  |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
|  |  |  |  | The connecting hose between the tanker and the loading and unloading platform has not been removed 槽车与装卸台的连接软管未拆除 |  | Equipment damage 设备损坏 |  |  | Rarely happens 很少发生 |  | 2 | 4 | 8 | II |  |  |  |  |  |  |
| 6 |  | Restoration of loading and unloading facilities 恢复装卸设施 |  | Failure to reheat the filling pump and protect the loading and unloading facilities 未对充装泵进行回温、装卸设施进行保护 |  | Equipment damage 设备损坏 |  |  | Rarely happens 很少发生 |  | 2 | 3 | 6 | II |  |  |  |  |  |  |
| Descriptions: 说明： 1. For occurrence frequency, consider how many cases of such hazard have happened on similar equipment of the same industry. 发生频率考虑同行业类似装置设备发生该类危险的案例多少。 2. The initial risk assessment examines situations where there are no controls for the risk, including extreme situations that may occur. 初始风险评价考虑该风险无任何控制措施的情形，包括可能发生的极端情况。 3. Risk reduction and control measures Fill in the executable control measures that need to be added from the aspects of personnel ability and quality, operating procedures, operation plans, safety facilities, inspection and monitoring, hidden danger management, and emergency plans. 风险削减及控制措施填写需要增加的可执行控制措施，从人员能力和素质、操作规程、作业方案、安全设施、检查监测、隐患治理、应急预案等方面考虑。 4. Filling of residual risk assessment adopts the risk assessment results after risk reduction and control measures. 剩余风险评价填写采取风险消减及控制措施后的风险评价结果。 Medium and high risk lists shall be filled based on the results of initial risk assessment. 5. 中高度风险清单的填写以初始风险评价结果为准。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety instructions: 安全告知： I have been informed of the hidden hazard source and harmful factors during the operation process, the corresponding safety measures and emergency evacuation methods. 我已经了解本项作业过程中的危险源、危害因素、相应的安全措施和应急疏散的途径。 Signature for confirmation: 签字确认： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |